| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0012 |
| --- | --- |

| Title: | CE2 Summary Report: Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | Li Zhang Tomohiro Ikai | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of the Core Experiment 2 (CE2) on Residual Prediction. During the 7th JCT-3V meeting, two contributions have been proposed to improve the coding efficiency or reduce the complexity of residual prediction in 3D-HEVC and they were considered for further study in a Core Experiment. The goal of this CE is to evaluate them in the context of the 3D-HTM 10.0r1.

In summary, 9 companies and universities were involved in the CE with 2 CE proposals and 5 CE-related proposals.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Li Zhang Ying Chen Hongbin Liu | lizhang@qti.qualcomm.com cheny@qti.qualcomm.com hongbinl@qti.qualcomm.com | P |
| MediaTek | Jian-Liang Lin Jicheng An Kai Zhang Yu-Lin Chang Yu-Wen Huang | jl.lin@mediatek.com jicheng.an@mediatek.com Kai.zhang@mediatek.com yulin.chang@mediatek.com yuwen.huang@mediatek.com | C |
| Samsung | M. W. Park Jin Young Lee | m.w.park@samsung.com jinyoung79.lee@samsung.com |  |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp | P |
| LG | Sunmi Yoo | sunmi.yoo@lge.com | P |
| NTT | Shiori Sugimoto Shinya Shimizu | sugimoto.shiori@lab.ntt.co.jp shimizu.shinya@lab.ntt.co.jp |  |
| Zhejiang University | Yichen Zhang Peng Lu | felixzyc@gmail.com yiqiu8932@gmail.com |  |
| ETRI | Gun Bang | gbang@etri.re.kr | C |
| HiSilicon | Xu Chen | anderson.chen@huawei.com | P |

# Tools under investigation

## Subset A (coding performance)

### Further improvements on advanced residual prediction [JCT3V-H0130*]

It is proposed to extend the block-level temporal ARP (when sub-PU inter-view merging candidates are used) to inter-view ARP. Meanwhile, for the block-level temporal ARP, the disparity vector refinement process is also applied to further improve the coding efficiency.

Two sets of simulation results are provided, as shown in the following table:

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block-level inter-view ARP | -0.27% | -0.32% | -0.09% | -0.09% | -0.10% | 101% | 95% | 99% |
| Whole proposal | -0.45% | -0.53% | -0.15% | -0.15% | -0.14% | 105% | 105% | 106% |

This proposal was crosschecked by Sharp, and detail information can be referred to JCT3V-H0193.

## Subset B (complexity reduction)

### Results of ARP simplification [JCT3V-H0063*]

It is proposed to disable ARP for 4x4 chroma blocks to reduce ARP’s bandwidth.

The experiment results show that the BD-rate loss is 0.1 %, 0.1 % and 0.1 % in video, total video and synthesis respectively.

It is claimed that the complexity of memory bandwidth of ARP compared to HEVC single view coding is reduced from 122% to 105%.

Detailed results are shown in the following table:

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JCT3V-H0063 | 0.36% | 0.32% | 0.08% | 0.07% | 0.08% | 100% | 101% | 100% |

This proposal was crosschecked by Qualcomm, and detail information can be referred to JCT3V-H0194.

### Improvement disparity vector on temporal ARP and chroma 4x4 off [JCT3V-H0064]

This proposal proposes a combination of the disparity improvement part of JCT3V-H0130 and disabling ARP for chroma 4x4 block of JCT3V-H0063. It is asserted that the combination shows better trade-off between coding efficiency and complexity.

This proposal was crosschecked by Qualcomm, and detail information can be referred to JCT3V-H0195.

### Simplification of Advanced Residual Prediction [JCT3V-H0132]

Two aspects are proposed in this contribution:

- For bi-predictively coded PUs, the number of accessed blocks used in ARP is reduced:

  - When current PU is predicted from one temporal reference picture and one inter-view reference picture, the reference block identified by current disparity vector in temporal ARP is set to the reference block identified with the coded disparity motion vector.

  - When current PU is predicted from two inter-view reference pictures, the temporal reference block in the ARP target reference picture associated with reference picture list 1 is set to the one associated with reference picture list 0.

- It is proposed to disable bi-predictive 4x4 chroma blocks with ARP mode by applying uni-prediction from reference picture list 0.

This proposal was crosschecked by Sharp, and detail information can be referred to JCT3V-H0181.

### Simplification of ARP [JCT3V-H0109]

This contribution proposes to remove the 0.5 weighting factor.

This proposal is crosschecked by MediaTek and detail information can be referred to JCT3V-H0162.

### Removal of ARP for AMVP mode [JCT3V-H0112]

This contribution proposes to remove ARP for AMVP mode.

This proposal is crosschecked by Qualcomm and detail information can be referred to JCT3V-H0198.

### Summary of coding performance and complexity

The following table shows the simulation results and complexity comparisons for all CE and CE related proposals on ARP simplification.

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Memory bandwidth compared to HEVC (worst case) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ARP in 3D-HEVC |  |  |  | 0.0% |  |  |  | 122% |
| JCT3V-H0063* | 0.36% | 0.32% | 0.08% | 0.07% | 0.08% | 100% | 101% | 105% |
| JCT3V-H0064 | 0.06% | 0.01% | -0.02% | -0.04% | 0.00% | 100% | 101% | 105% |
| JCT3V-H0132 #1 | 0.01% | -0.04% | 0.00% | 0.00% | -0.02% | 107% | 101% | 101% |
| JCT3V-H0132 | 0.18% | 0.14% | 0.04% | 0.04% | 0.02% | 104% | 108% | 94% |
| JCT3V-H0132 #1 + JCT3V-H0130 | -0.45% | -0.52% | -0.14% | -0.13% | -0.12% | 103% | 98% | 101% |
| JCT3V-H0132 + JCT3V-H0130 | -0.27% | -0.33% | -0.09% | -0.08% | -0.08% | 104% | 107% | 94% |
| JCT3V-H0109 | 0.04% | -0.06% | 0.03% | 0.03% | -0.02% | 99% | 98% | 122% |
| JCT3V-H0112 | 0.03% | -0.03% | 0.00% | 0.01% | -0.02% | 100% | 98% | 122% |
| JCT3V-H0118 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% | 122% |

## Subset C (entropy coding)

### Separation of syntax elements for ARP mode [JCT3V-H0118]

This contribution proposes to replace the weighting factor index (i.e., iv_res_pred_weight_idx) with two separate one-bit flags. The first flag (ARP_mode_flag) indicates whether ARP is on, and the second one (ARP_weight_flag) indicates if the weighting factor is 1 or 0.5.

Experimental results show that the coding results are identical with current 3D-HTM.

This proposal was not crosschecked ?.

Note: in the above description, a proposal number with * indicates it is a CE proposal. Otherwise, it is a CE related proposal.