| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0013 |
| --- | --- |

| Title: | CE3 Summary report on Entropy Coding Simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang Hongbin Liu | Tel: Email: | +886-3-5670766 ext. 25314 yulin.chang@mediatek.com +86-10-57760696 hongbinl@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document is the summary report of Core Experiment 3 (CE3) on entropy coding simplification. This Core Experiment investigates methods on entropy coding simplifications. Tools under test will be evaluated according to their impact on compression efficiency and implementation complexity. There are 4 CE proposals and 6 CE related proposals.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Kai Zhang Jicheng An Yu-Lin Chang Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com kai.zhang@mediatek.com jicheng.an@mediatek.com yulin.chang@mediatek.com yuwen.huang@mediatek.com | P/C |
| Qualcomm | Hongbin Liu Li Zhang Ying Chen | hongbinl@qti.qualcomm.com lizhang@qti.qualcomm.com cheny@qti.qualcomm.com | P/C |
| HHI | P. Merkle K. Müller | philipp.merkle@hhi.fraunhofer.de karsten.mueller@hhi.fraunhofer.de | P/C |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | C |
| LGE | Jung Hak Nam | Junghak.nam@lge.com | C |

(P = proponent, C = cross checker)

# Tools under Investigation

In this Core Experiment, three categories are listed in the following table: simplification on entropy coding of delta_dc_flag, delta_dc_abs and wedge_full_tab_idx. Total 4 CE contributions and 2 CE-related contributions are included.

- Simplification on entropy coding of delta_dc_flag

  - JCT3V-H0084

  - JCT3V-H0100 (related)

  - JCT3V-H0113 (related)

- Simplification on entropy coding of delta_dc_abs

  - JCT3V-H0084

  - JCT3V-H0131

- Simplification on entropy coding of wedge_full_tab_idx

  - JCT3V-H0078

  - JCT3V-H0092

There are several CE-related contributions, which could be categorized into the following three categories:

- Simplification on entropy coding of dim_not_present_flag

  - JCT3V-H0100 (related)

  - JCT3V-H0119 (related)

  - JCT3V-H0135 (related)

- Simplification on entropy coding of iv_res_pred_weight_idx

  - JCT3V-H0120 (related)

- Improvement of coding performance on entropy coding of wedge_full_tab_idx

  - JCT3V-H0110 (related)

## List of contributions

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| CE contributions |  |  |  |
| HHI | JCT3V-H0078 | CE3: Results on DMM1 signaling using intra direction | Proposal |
| MediaTek | JCT3V-H0161 | Crosschecking for HHI's CE3 (H0078) proposal | Crosscheck |
| MediaTek | JCT3V-H0084 | 3D-CE3: Simplification on CABAC contexts for the syntax related delta DC | Proposal |
| Qualcomm | JCT3V-H0196 | 3D-CE3: Crosscheck of Simplification on CABAC contexts for the syntax related delta DC (JCT3V-H0084) | Crosscheck |
| MediaTek | JCT3V-H0092 | 3D-CE3: Results on simplified DMM mode coding | Proposal |
| HHI | JCT3V-H0124 | CE3: Cross check of simplified DMM mode coding (JCT3V-H0092) | Crosscheck |
| Qualcomm, PKU | JCT3V-H0131 | 3D-CE3: Delta DC coding for SDC and DMM modes | Proposal |
| MediaTek | JCT3V-H0166 | 3D-CE3: Crosscheck on Simplification on Delta DC coding for SDC and DMM modes (JCT3V-H0131) | Crosscheck |
| CE-related contributions (including related other CE-related contributions) |  |  |  |
| Samsung | JCT3V-H0100 | 3D-CE3 related: Simplification on CABAC contexts for a DMM flag | Proposal |
| MediaTek | JCT3V-H0165 | 3D-CE3 related: Crosscheck on Simplification on CABAC contexts for a DMM flag (JCT3V-H0100) | Crosscheck |
| HiSilicon | JCT3V-H0110 | CE3 related: Redundancy removal for DMM1 signalling | Proposal |
| LGE | JCT3V-H0191 | Crosscheck of redundancy removal for DMM1 signalling (JCT3V-H0110) | Crosscheck |
| LGE | JCT3V-H0113 | CE3 related: Alignment of WD and HTM for syntax element depth_dc_flag | Proposal |
| Samsung | JCT3V-H0155 | 3D-CE3 related: Cross check of alignment of WD and HTM for syntax element depth_dc_flag (JCT3V-H0113) | Crosscheck |
| LGE | JCT3V-H0119 | 3D-CE3 related: Simplification on CABAC contexts for the syntax related to depth intra mode | Proposal |
| MediaTek | JCT3V-H0187 | Crosschecking for LGE's JCT3V-H0119 | Crosscheck |
| LGE | JCT3V-H0120 | 3D-CE3 related: Simplification on CABAC contexts for advanced residual prediction mode | Proposal |
| MediaTek | JCT3V-H0192 | 3D-CE3 related: Crosscheck of LGE's proposal on Simplification on CABAC contexts for ARP(JCT3V-H0120) | Crosscheck |
| Qualcomm | JCT3V-H0135 | CE3 related: Simplification of dim_not_present_flag coding | Proposal |
| HHI | JCT3V-H0167 | CE3 related: Cross check of simplification of dim_not_present_flag coding (JCT3V H0135) | Crosscheck |

# Summary of results

## Category (a): [CE] Simplification of CABAC of depth_dc_flag

In the current 3D-HEVC, there is a mismatch in terms of context model between HTM software and WD. One context in WD is used while two contexts (selected according to the number of segments) in HTM are used for coding depth_dc_flag. Three proposals propose to modify the software to be aligned with the WD while the context model for depth_dc_flag is reduced from two to one. Note that the proposed modifications in the three proposals are identical.

TABLE 1: Summary of results on Test (a)

| Proposal | Num. contexts (initType 0 / 1 / 2) | Binarization process | Num. context coded bins | Performance (CTC) | WD |
| --- | --- | --- | --- | --- | --- |
| HTM-10.0r1 | 2 / 2 / 2 | FL | 1 | - | - |
| JCT3V-H0084 Test 1 | 1 / 1/ 1 | FL | 1 | 0.01% | Yes |
| JCT3V-H0100 Test 2 (CE-related) | 1 / 1/ 1 | FL | 1 | 0.01% | Yes |
| JCT3V-H0113 (CE-related) | 1 / 1/ 1 | FL | 1 | 0.01% | Yes |

## Category (b): [CE] Simplification of CABAC of depth_dc_abs

In the current 3D-HEVC, the prefix codeword bins of depth_dc_abs are context coded (up to 13 bins for the worst case), two CE proposals have proposed several methods to reduce the context coded bins from 13 to 1, 3 and 9, respectively.

TABLE 2: Summary of results on Test (b)

| Syntax element | Num. contexts (initType 0 / 1 / 2) | Binarization process | Num. context coded bins | Performance (CTC/ AI) | WD |
| --- | --- | --- | --- | --- | --- |
| HTM-10.0r1 | 1 / 1 /1 | UEG0 | 13 | - | - |
| JCT3V-H0084 Test 2 | 1 / 1/ 1 | UEG0 | 1 | 0.07% / 0.15% | Yes |
| JCT3V-H0131 Test 2 | 1 / 1/ 1 | UEG0 | 3 | -0.06% / -0.02% | Yes |
| JCT3V-H0131 Test 1 | 1 / 1/ 1 | See G0124 | 9 | -0.02% / NA | Yes |

## Category (c): [CE] Simplification of CABAC of wedgelet segmentation

In the current 3D-HEVC, the table index in DMM1 is binarized as a fixed-length code, and all bins share one context model. In the worst cases, 11 context coded bins are coded for DMM1 wedgelet index. Two simplifications of the CABAC of DMM1 wedgelet segmentation are proposed in this category.

TABLE 3-a: Summary of results on Test (c)

| Syntax element | Num. contexts (initType 0 / 1 / 2) | Binarization process | Num. context coded bins (worst case) | Performance (CTC/AI) | WD |
| --- | --- | --- | --- | --- | --- |
| HTM-10.0r1 | 1 / 1 /1 | FL | 11 | - | - |
| JCT3V-H0078 | 1 / 1/ 1 | Table 3-b | 0 | -0.06% / -0.05% | Yes |
| JCT3V-H0092 | 1 / 1/ 1 | FL | 0 | -0.03% / -0.01% | Yes |

TABLE 3-b: Summary of syntax design on Test (c)

| block size | 4×4 |  | 8×8 |  | 16×16 |  | 32×32 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bins | number | type | number | type | number | type | number | type |
| HTM-10.0r1 wedge_full_tab_idx | 7 | CABAC | 10 | CABAC | 11 | CABAC | 11 | CABAC |
| JCT3V-H0078 wedge_dir_idx wedge_refine_idx | 5 0~3 | bypass bypass | 5 3~6 | bypass bypass | 5 4~6 | bypass bypass | 5 4~6 | bypass bypass |
| JCT3V-H0092 wedge_full_tab_idx | 7 | bypass | 10 | bypass | 11 | bypass | 11 | bypass |

## Category (d): [CE-related] Simplification of CABAC of dim_not_present_flag

In current 3D-HEVC, three CABAC context models are used for dim_not_present_flag to indicate whether the current block is encoded as HEVC intra mode or depth intra mode. Three proposed simplifications use only one context as a candidate without referring to any information of neighboring blocks. The concept of the three proposed simplifications is identical, only different initial values are selected. The initial values (CNU, CNU, CNU) for each picture type is used in JCT3V-H0100 and JCT3V-H0119 test1 while the initial values (154, 141, 155) is used in JCT3V-H0135 and JCT3V-H0119 test2.

TABLE 4: Summary of results on Test (d)

| Syntax element | Num. contexts (initType 0 / 1 / 2) | Binarization process | Num. context coded bins | Performance (CTC/AI) | WD |
| --- | --- | --- | --- | --- | --- |
| HTM-10.0r1 | 3 / 3 /3 | FL | 1 | - | - |
| JCT3V-H0100 Test 1 | 1 / 1/ 1 | FL | 1 | -0.02% / 0.00% | Yes |
| JCT3V-H0119 Test 1 | 1 / 1/ 1 | FL | 1 | -0.02% / 0.00% | Yes |
| JCT3V-H0119 Test 2 | 1 / 1/ 1 | FL | 1 | -0.02% / 0.00% | Yes |
| JCT3V-H0135 | 1 / 1/ 1 | FL | 1 | -0.02% / 0.00% | Yes |

## Category (e): [CE-related] Simplification of CABAC of iv_res_pred_weight_idx

In the current 3D-HEVC, two CABAC context models are used to indicate on/off of advanced residual prediction (ARP) by referring to the information of left neighboring block. The proposed method in JCT3V-H0120 simplifies the context models for the iv_res_pred_weight_idx. It uses only one CABAC context model without referring to any information of neighboring blocks.

TABLE 5: Summary of results on Test (e)

| Syntax element | Num. contexts (initType 0 / 1 / 2) | Binarization process | Num. context coded bins | Performance (CTC) | WD |
| --- | --- | --- | --- | --- | --- |
| HTM-10.0r1 | 3 / 3 /3 | FL | 1 | - | - |
| JCT3V-H0120 | 1 / 1/ 1 | FL | 1 | 0.03% | Yes |

## Category (f): [CE] Improvement of coding performance of CABAC of wedgelet segmentation

In the current 3D-HEVC, the table index in DMM1 is binarizied as a fixed-length code. As coding bits for table index can be used for signaling more number than table index, redundancy happens. A method is proposed in JCT3V-H0110 to code the table index by the difference value which can shorten coding list index and fewer bit needed to be coded.

TABLE 6-a: Summary of results on Test (f)

| Syntax element | Num. contexts (initType 0 / 1 / 2) | Binarization process | Num. context coded bins (worst case) | Performance (CTC/AI) | WD |
| --- | --- | --- | --- | --- | --- |
| HTM-10.0r1 | 1 / 1 /1 | FL | 11 | - | - |
| JCT3V-H0110 | 1 / 1/ 1 | Table 6-b | 10 | -0.07% / -0.07% | Yes |

TABLE 6-b: Summary of syntax design on Test (f)

| block size | 4×4 |  | 8×8 |  | 16×16 |  | 32×32 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bins | number | type | number | type | number | type | number | type |
| HTM-10.0r1 wedge_full_tab_idx | 7 | CABAC | 10 | CABAC | 11 | CABAC | 11 | CABAC |
| JCT3V-H0110 wedge_diff_tab_idx wedge_diff_tab_idx_sign | 6 1 | CABAC bypass | 9 1 | CABAC bypass | 10 1 | CABAC bypass | 10 1 | CABAC bypass |