| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0055 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0023 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Comments on general decoding process and selection of CPB operation in the HRD operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi Min Woo Park Jin Young Lee | Email: | {yongjin9.cho, b.d.choi, m.w.park, jinyoung79.lee} @samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes modifications and improvements of texts in the subclause 8.1 General decoding procedure and C.1 General of Annex C with the following items:

- Re-definition of ‘output layer’

- Proposal to substitute ‘TargetOptLayerSetIdx’ with ‘TargetOutputLayerSetIdx’

- Modification of TargetOutputLayerSetIdx setting procedure

- Modification of HighestTid setting procedure

- Modification of NoRaslOutputFlag setting procedure

- Modification of selecting CPB operation for a bitstream conformance test

Changes to the current draft are marked by yellow highlight and strikethorough in red font.

# Proposal

- Re-definition of ‘output layer’

  - Current definitions

output layer: A layer of an output layer set that is output when TargetOptLayerSetIdx is equal to the index of the output layer set.

target output layer: A layer that is to be output and is one of the output layers of the output layer set with index olsIdx such that TargetOptLayerSetIdx is equal to olsIdx.

  - Comment

The difference between ‘output layer’ and ‘target output layer’ is not clear from the definitions.

  - Proposal

output layer: A layer of an output layer set that is to be output when the output layer is selected as a target output layer setwhen TargetOptLayerSetIdx is equal to the index of the output layer set.

- The variables TargetOptLayerSetIdx and TargetOutputLayerSetIdx

‘TargetOptLayerSetIdx’ seems to be an unfinished job of editors. It is proposed to substitute ‘TargetOptLayerSetIdx’ with ‘TargetOutputLayerSetIdx’.

- Setting the value of TargetOutputLayerSetIdx

  - Description in the current draft (8.1 General Decoding Process):

The variable TargetOutputLayerSetIdx, which specifies the index to the list of the output layer set specified by the VPS, of the target output layer set, is specified as follows:

- If some external means, not specified in this Specification, is available to set TargetOutputLayerSetIdx, TargetOutputLayerSetIdx is set by the external means.

- Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in subclause C.1, TargetOutputLayerSetIdx is set as specified in subclause C.1.

- Otherwise, TargetOutputLayerSetIdx is set equal to 0.

  - Comment

In subclause C.1, it is not specified how to set the value of TargetOutputLayerSetIdx.

  - Proposal: Remove the second option providing a reference to subclause C.1.

The variable TargetOutputLayerSetIdx, which specifies the index to the list of the output layer set specified by the VPS, of the target output layer set, is specified as follows:

- If some external means, not specified in this Specification, is available to set TargetOutputLayerSetIdx, TargetOutputLayerSetIdx is set by the external means.

- Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in subclause C.1, TargetOutputLayerSetIdx is set as specified in subclause C.1.

- Otherwise, TargetOutputLayerSetIdx is set equal to 0.

- Setting the value of HighestTid

  - Description in the current draft (8.1 General Decoding Process):

The variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

- If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

- Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in subclause C.1, HighestTid is set as specified in subclause C.1.

- Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

  - Comment

    - In subclause C.1, it is not specified how to set the value of HighestTid.

    - In the last item, it is better that HighestTid is set equal to vps_max_sub_layers_minus1 instead of sps_max_sub_layers_minus1.

  - Proposal: Remove the second option providing a reference to subclause C.1, and switch sps_max_sub_layers_minus1 to vps_max_sub_layers_minus1

The variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

- If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

- Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in subclause C.1, HighestTid is set as specified in subclause C.1.

- Otherwise, HighestTid is set equal to svps_max_sub_layers_minus1.

- Text simplification: the description of setting the value of NoRaslOutputFlag

  - Text in the current draft (8.1 General Decoding Process)

When the current picture is an IRAP picture, the following applies:

- If the current picture with a particular value of nuh_layer_id is an IDR picture, a BLA picture, the first picture with that particular value of nuh_layer_id in the bitstream in decoding order, or the first picture with that particular value of nuh_layer_id that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

- Otherwise, if LayerInitializedFlag[ nuh_layer_id ] is equal to 0 and LayerInitializedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to RefLayerId[ nuh_layer_id ][ j ], where j is in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive, the variable NoRaslOutputFlag is set equal to 1.

- Otherwise, the variable NoRaslOutputFlag is set equal to HandleCraAsBlaFlag

  - Comment: In the first condition, if an IRAP picture is not an IDR or a BLA, then it has to be a CRA picture. That is, it is unnecessary to describe the last picture as ‘first picture ……’.

  - Proposal:

    - Replace ‘the first picture ……’ with ‘or a CRA picture with HandleCraAsBlaFlag equal to 1’.

- If the current picture with a particular value of nuh_layer_id is an IDR picture, a BLA picture, the first picture with that particular value of nuh_layer_id in the bitstream in decoding order, or the first picture with that particular value of nuh_layer_id that follows an end of sequence NAL unit in decoding orderor a CRA picture with HandleCraAsBlaFlag equal to 1, the variable NoRaslOutputFlag is set equal to 1.

    - Simply remove the phrase ‘the first picture …’.

- If the current picture with a particular value of nuh_layer_id is an IDR picture, a BLA picture, the first picture with that particular value of nuh_layer_id in the bitstream in decoding order, or the first picture with that particular value of nuh_layer_id that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

- Selection of CPB operations

  - Current draft (C.1)

When both the vps_vui_bsp_hrd_parameters( ) syntax structure is present in the active VPS and num_bitstream_partitions[ TargetDecLayerSetIdx ] is greater than 1 or both a bitstream partition HRD parameters SEI message is present and the SEI message contains syntax element num_sei_bitstream_partitions_minus1[ TargetDecLayerSetIdx ] greater than 0, either the bitstream-specific CPB operation or the bitstream-partition-specific CPB operation is selected for a conformance test, and both CPB operations shall be tested for checking the conformance of a bitstream.

  - Comment

The text contains conflicting statements:

“either the bitstream-specific CPB operation or the bitstream-partition-specific CPB operation is selected for a conformance test, and both CPB operations shall be tested for checking the conformance of a bitstream.”

  - Proposal: modify the text such that ‘either ……’ part disappears

      - When bitstream-partition specific hrd parameters are present, the modified draft requires that both of the CPB operations be tested.

When both the vps_vui_bsp_hrd_parameters( ) syntax structure is present in the active VPS and num_bitstream_partitions[ TargetDecLayerSetIdx ] is greater than 1 or both a bitstream partition HRD parameters SEI message is present and the SEI message contains syntax element num_sei_bitstream_partitions_minus1[ TargetDecLayerSetIdx ] greater than 0, either the bitstream-specific CPB operation or the bitstream-partition-specific CPB operation is selected for a conformance test, and both CPB operations shall be tested for checking the conformance of a bitstream.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).