| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0060r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0024r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On inter-layer RPS signalling and derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee Jin Soo Choi 218 Gajeong-ro, Yuseong-gu Daejeon, Rep. of Korea | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This contribution proposes two changes for MV-HEVC and SHVC related to inter-layer RPS signalling in the slice segment header and derivation process. The first change targets to improve the inter-layer RPS signalling in the slice segment header by taking into account the number of direct reference layers presented in the same access unit as the current picture, which is indicated by the ‘numRefLayerPics’, instead of the number of direct reference layers indicated by the ‘NumDirectRefLayers’. The second change targets to improve the derivation process of ‘numRefLayerPics’ and ‘refLayerPicIdc’ to support the lightweight option scenario in that syntax element ‘max_tid_il_ref_pics_plus’ is set equal to 0.

In r1 revision, the simplified version is added for Item 2 and comments on JCTVC-Q0079, which includes the conceptually same proposals as this, are added.

# Inter-layer RPS signalling

In the slice segment header of the current MV-HEVC as well as SHVC working draft, there are syntax elements to signal inter-layer reference picture sets used in decoding of the current picture for inter-layer prediction as shown in the table 1.

Since the current inter-layer PRS derivation process considers both the sub-layer inter-layer prediction restriction indicated by the syntax element ‘max_tid_il_ref_pics_plus1’ and the maximum number of sub-layers indicated by the syntax element ‘sub_layers_vps_max_minus1’ that may be present in each layer, there are pictures of some sub-layers not used in decoding of the current picture for inter-layer prediction as shown in the figure 1. In this example configuration, it is assumed that the syntax element ‘all_ref_layers_active_flag’, specified in the VPS extension, is set equal to 0 and the syntax element ‘max_one_active_ref_layer_flag’, also specified in the VPS extension, is set equal to 1. For the picture (a) belonging to Layer ID==3 and TemporalID==3, marked as the blue box, only (b) picture belonging to Layer ID==2 and TemporalID==3, marked as the red box, is available for inter-layer RPS of picture (a) by the inter-layer prediction restriction indicated by the ‘max_tid_il_ref_pics_plus1[][]’ and ‘sub_layers_vps_max_minus1[]’ of direct reference layers, even though the picture (a) has three direct reference layers. According to the current inter-layer RPS signalling design, since NumDirectRefLayers[3] is greater than 1 and the number of active reference layers (NumActiveRefLayerPics) of picture (a), set equal to 1, is not equal to NumDirectRefLayers[3], the syntax element ‘inter_layer_pred_layer_idc[]’ has to be signalled to specify the picture that used by the picture (a) for inter-layer prediction. However, in this case, the syntax element ‘inter_layer_pred_layer_idc[]’ can be inferred, since all of the reference pictures of direct reference layers presented in the same access unit as the current picture are active reference layer pictures.

Table 1. Inter-layer RPS signaling in the slice segment header

| if( nuh_layer_id > 0 && !all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| --- | --- |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |

Figure 1. Example configuration

Since the current inter-layer RPS signalling in the slice segment header considers only the number of direct reference layers (NumDirectRefLayers) without taking into account the number of reference pictures (numRefLayerPics) presented in the same access unit as the current picture, which is derived by applying the syntax elements ‘max_tid_il_ref_pics_plus1[][]’ and ‘sub_layers_vps_max_minus1[]’ in the F.7.4.7.1, the redundant syntax elements are signalled when the number of reference pictures (numRefLayerPics) of direct reference layer presented in the same access unit as the current picture is equal to the number of active reference layer (NumActiveRefLayerPics).

## Improved inter-layer RPS signalling

It is proposed that the redundant signal bits can be removed by considering the number of direct reference layers (numRefLayerPics) presented in the same access unit as the current picture for the inter-layer RPS signalling in the slice segment header, instead of the number of direct reference layers (NumDirectRefLayers[nuh_layer_id]).

Table 2. Proposed inter-layer RPS signaling in the slice segment header

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 && !all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] numRefLayerPics > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] numRefLayerPics > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] numRefLayerPics) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

The variables numRefLayerPics and refLayerPicIdc[ j ] are derived as follows: for( i = 0, j = 0; i < NumDirectRefLayers[ nuh_layer_id ]; i++ ) { refLayerIdx = LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ] if( sub_layers_vps_max_minus1[ refLayerIdx ] >= TemporalId && max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] > TemporalId ) refLayerPicIdc[ j++ ] = i } numRefLayerPics = j The variable NumActiveRefLayerPics is derived as follows: if( nuh_layer_id = = 0 ¦ ¦ numRefLayerPics = = 0 ) NumActiveRefLayerPics = 0 else if( all_ref_layers_active_flag ) NumActiveRefLayerPics = numRefLayerPics else if( !inter_layer_pred_enabled_flag ) NumActiveRefLayerPics = 0 else if( max_one_active_ref_layer_flag ¦ ¦ NumDirectRefLayers[ nuh_layer_id ] numRefLayerPics = = 1 ) NumActiveRefLayerPics = 1 else NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1 All slices of a coded picture shall have the same value of NumActiveRefLayerPics

- Derivation of numRefLayerPics and refLayerIdc

In the current MV-HEVC and SHVC draft text, the variable numRefLayerPics and refLayerPicIdc[j] are derived to specify the direct reference layers presented in the same access unit as the current picture that may be used in decoding of the current picture for inter-layer prediction among the direct reference layers specified in the VPS extension as follows.

for( i = 0, j = 0; i < NumDirectRefLayers[ nuh_layer_id ]; i++ ) { refLayerIdx = LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ] if( sub_layers_vps_max_minus1[ refLayerIdx ] >= TemporalId && max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] > TemporalId ) refLayerPicIdc[ j++ ] = i } numRefLayerPics = j

Also, the syntax element ‘max_tid_il_ref_pics_plus1[i][j]’ has the following semantics

max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. max_tid_il_ref_pics_plus1[ i ][ j ] greater than 0 specifies that within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ][ j ] − 1 are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. When not present, max_tid_il_ref_pics_plus1[ i ][ j ] is inferred to be equal to 7.

The configuration, ‘max_tid_il_ref_pics_plus1[i][j]’ is equal to 0, gives a lightweight option for many encoders as no multiple-loop decoding is necessary. However, when the value of ‘max_tid_il_ref_pics_plus1[i][j]’ and TemporalId are both equal to 0 and the picture with nuh_layer_id equal to layer_id_in_nuh[i] in the current access unit is an IRAP picture, the above condition for ‘refLayerPicIdc[]’ (highlighted in yellow) becomes false so that the IRAP picture with nuh_layer_id equal to layer_id_in_nuh[i] are also not used as reference for inter-layer prediction for picture with nuh_layer_id equal to layer_id_in_nuh[j]. As a result, the current derivation process of numRefLayerPics and refLayerIdc makes the lightweight option disabled in the current MV-HEVC and SHVC.

## Changes to numRefLayerPics and refLayerIdc derivation

The derivation process of numRefLayerPics and refLayerIdc is changed for lightweight option as in the highlighted text below:

The variables numRefLayerPics and refLayerPicIdc[ j ] are derived as follows:

for( i = 0, j = 0;  i < NumDirectRefLayers[ nuh_layer_id ]; i++ ) {
	refLayerIdx = LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ]

if( sub_layers_vps_max_minus1[ refLayerIdx ] >= TemporalId  && 
			max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] > TemporalId )
		refLayerPicIdc[ j++ ] = i
	refLayerPicFlag = ( ( sub_layers_vps_max_minus1[ refLayerIdx ] >= TemporalId ) &&

( max_tid_il_ref_pics_plus1[ refLayerIdx ][LayerIdxInVps[nuh_layer_id]] == 0 ?

( max_tid_il_ref_pics_plus1[ refLayerIdx ][LayerIdxInVps[nuh_layer_id]] == TemporalId )

: ( max_tid_il_ref_pics_plus1[ refLayerIdx ][LayerIdxInVps[nuh_layer_id]] > TemporalId ))

if ( refLayerPicFlag )

refLayerPicIdc[ j++ ] = i 
}

numRefLayerPics = j

The variables numRefLayerPics and refLayerPicIdc[ j ] are derived as follows:

for( i = 0, j = 0;  i < NumDirectRefLayers[ nuh_layer_id ]; i++ ) {
	refLayerIdx = LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ]
	if( sub_layers_vps_max_minus1[ refLayerIdx ] >= TemporalId  && 
		( max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] > TemporalId

| | TemporalId == 0 ) )
		refLayerPicIdc[ j++ ] = i
}

numRefLayerPics = j

# Comments on JCTVC-Q0079 (JCT3V-H0027)

If JCTVC-Q0079 proposal is adopted, the following modifications are necessary for MV-HEVC WD8 and SHVC WD6

- The variables NumDirectRefLayersForTid[lId][tId] and RefLayerIdListForTid[lId][tId][k] are derived as follows according to either vps_max_sub_layers_minus1 or sub_layers_vps_max_minus1[]

Alternative1 :

for( lIdx = 0; lIdx <= MaxLayersMinus1; lIdx++ ) {

lId = layer_id_in_nuh[ lIdx ]

for( tId = 0; tId < 7; tId++ ) {

for( tId = 0; tId <= vps_max_sub_layers_minus1; tId++ ) {

for( rCnt = 0, k = 0; rCnt < NumDirectRefLayers[ lId ]; rCnt++ ) {

refLayerIdx = LayerIdxInVps[ RefLayerId[ lId ][ rCnt ] ]

if( sub_layers_vps_max_minus1[ refLayerIdx ] >= tId  &&

( max_tid_il_ref_pics_plus1[ refLayerIdx ][ lIdx ] > tId  | |  tId  = =  0 ) )

RefLayerIdListForTid[ lId ][ tId ][ k++ ] = RefLayerId[ lId ][ rCnt ]

}

NumDirectRefLayersForTid[ lId ][ tId ] = k

}

}

Alternative2 :

for( lIdx = 0; lIdx <= MaxLayersMinus1; lIdx++ ) {

lId = layer_id_in_nuh[ lIdx ]

for( tId = 0; tId < 7; tId++ ) {

for( tId = 0; tId <= sub_layers_vps_max_minus1[lIdx]; tId++ ) {

for( rCnt = 0, k = 0; rCnt < NumDirectRefLayers[ lId ]; rCnt++ ) {

refLayerIdx = LayerIdxInVps[ RefLayerId[ lId ][ rCnt ] ]

if( sub_layers_vps_max_minus1[ refLayerIdx ] >= tId  &&

( max_tid_il_ref_pics_plus1[ refLayerIdx ][ lIdx ] > tId  | |  tId  = =  0 ) )

RefLayerIdListForTid[ lId ][ tId ][ k++ ] = RefLayerId[ lId ][ rCnt ]

}

NumDirectRefLayersForTid[ lId ][ tId ] = k

}

}

- The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayersForTid[ nuh_layer_id ][ TemporalId ]  = =  0 )

NumActiveRefLayerPics = 0

else if( all_ref_layers_active_flag )

NumActiveRefLayerPics = numRefLayerPics

NumActiveRefLayerPics = NumDirectRefLayersForTid[ nuh_layer_id ][ TemporalId ]

else if( !inter_layer_pred_enabled_flag )

NumActiveRefLayerPics = 0

else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayersForTid[ nuh_layer_id ][ TemporalId ]  = = 1 )

NumActiveRefLayerPics = 1

else

NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

- All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

- When ‘inter_layer_pred_layer_idc[i]’ is not present, the value is inferred to be equal to i

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element inter_layer_pred_layer_idc[ i ] is Ceil( Log2( NumDirectRefLayersForTid[ nuh_layer_id ][ TemporalId ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayersForTid[ nuh_layer_id ][ TemporalId ] − 1, inclusive. When not present, the value of inter_layer_pred_layer_idc[ i ] is inferred to be equal to refLayerPicIdc[ i ]. i

When i is greater than 0, inter_layer_pred_layer_idc[ i ] shall be greater than inter_layer_pred_layer_idc[ i − 1 ].

The variables RefPicLayerId[ i ] for all values of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, are derived as follows:

for( i = 0, j = 0; i < NumActiveRefLayerPics; i++)

RefPicLayerId[ i ] = RefLayerIdListForTid[ nuh_layer_id ][ TemporalId ][ inter_layer_pred_layer_idc[ i ] ]

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).