| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0061 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0025 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On max_tid_il_ref_pics_plus1 in the VSP extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee Jin Soo Choi 218 Gajeong-ro, Yuseong-gu Daejeon, Rep. of Korea | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This contribution proposes the extra syntax elements to indicate the default value for syntax element “max_tid_il_ref_pics_plus1” to remove signalling redundancy.

# Introduction

SHVC WD5 and MV-HEVC WD7 include the syntax element “max_tid_il_ref_pics_plus1[i][j]” in the VPS extension to provide information at VPS-level on whether pictures with nuh_layer_id equal to layer_id_in_nuh[i] are used for inter-layer prediction by the picture with nuh_layer_id equal to layer_id_in_nuh[j] or not. Especially, when the syntax element “max_tid_il_ref_pics_plus1[i][j]” is equal to 0, non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[i] are not used as reference for inter-layer prediction by picture with nuh_layer_id equal to layer_id_nuh[j]. This value makes some encoders/decoders to support a lightweight single-loop decoding by allowing inter-layer prediction at the only reference pictures which are IRAP pictures.

In a typical lightweight option scenario, encoders may set “max_tid_il_ref_pics_plus1” to be equal to 0 for all layers in the CVS. In addition, some encoders may set “max_tid_il_ref_pics_plus1” to be equal to the identical value for all layers in the CVS to control the amount of additional complexity in the scalable bitstream. However, the current layer-specific signalling of “max_tid_il_ref_pics_plus1” results in somewhat redundant signalling in these cases.

In order to save signalling overhead for these cases, an extra syntax element to indicate the default value for syntax element “max_tid_il_ref_pics_plus1” is proposed to be signalled with the syntax element “default_max_tid_il_ref_flag”.

# Proposals

## Alternative1

In the alternative1, the syntax element “max_tid_ref_present_flag” is replaced with the syntax element “default_max_tid_ref_flag”. When “default_max_tid_ref_flag” is equal to 1, the syntax element “default_max_tid_il_ref_pics_plus1” is signalled to indicate the default value, which is in the range of 0 to 7 for all layers. When “default_max_tid_ref_flag” is equal to 0, the syntax element “max_tid_il_ref_pics_plus1[i][j]” is signalled separately for the layers that use the current layer as a direct reference layer.

Table 1. Proposed “default_max_tid_il_ref_pics_plus1” in VPS extension

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| default_max_tid_ref_flag | u(1) |
| if( default_max_tid_ref_flag ) |  |
| default_max_tid_il_ref_pics_plus1 | u(3) |
| else { |  |
| for( i = 0; i < MaxLayersMinus1; i++ ) |  |
| for( j = i + 1; j <= MaxLayersMinus1; j++ ) |  |
| if( direct_dependency_flag[ j ][ i ] ) |  |
| max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |
| } |  |
| … |  |
| } |  |

default_max_tid_ref_flag equal to 1 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is not present. default_max_tid_ref_flag equal to 0 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is present.

default_max_tid_ref_pics_plus1 when present, specifies the inferred value of max_tid_il_ref_pics_plus1[i][j]

max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. max_tid_il_ref_pics_plus1[ i ][ j ] greater than 0 specifies that within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ][ j ] − 1 are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. When not present, max_tid_il_ref_pics_plus1[ i ][ j ] is inferred to be default_max_tid_il_ref_pics_plus1.

## Alternative2

In the alternative2, the syntax element “default_max_tid_ref_flag” is signalled, when the syntax element “max_tid_ref_present_flag” is equal to 1. When “default_max_tid_ref_flag” is equal to 1, the syntax element “default_max_tid_il_ref_pics_plus1” is signalled to indicate the default value, which is in the range of 0 to 7 for all layers. When “default_max_tid_ref_flag” is equal to 0, the syntax element “max_tid_il_ref_pics_plus1[i][j]” is signalled separately for the layers that use the current layer as a direct reference layer. When “max_tid_il_ref_pics_plus1[i][j]” is not present, when “max_tid_ref_present_flag” is equal to 0, “max_tid_il_ref_pics_plus1[ i ][ j ]” is inferred to be equal to 7. Otherwise, “max_tid_il_ref_pics_plus1[ i ][ j ]” is inferred to be “default_max_tid_ref_pics_plus1”.

Table 2. Proposed “default_max_tid_il_ref_pics_plus1” in the VPS extension

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| default_max_tid_ref_flag | u(1) |
| if ( default_max_tid_ref_flag ) |  |
| default_max_tid_il_ref_pics_plus1 | u(3) |
| else { |  |
| for( i = 0; i < MaxLayersMinus1; i++ ) |  |
| for( j = i + 1; j <= MaxLayersMinus1; j++ ) |  |
| if( direct_dependency_flag[ j ][ i ] ) |  |
| max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |
| } |  |
| … |  |

max_tid_ref_present_flag equal to 1 specifies that the syntax element max_tid_il_ref_pics_plus1[i][j] may or may not be present. max_tid_ref_present_flag equal to 0 specifies that the syntax element max_tid_il_ref_pics_plus1[i][j] is not present.

default_max_tid_ref_flag equal to 1 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is not present. default_max_tid_ref_flag equal to 0 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is present.

default_max_tid_ref_pics_plus1 when present, specifies the inferred value of max_tid_il_ref_pics_plus1[i][j]

max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. max_tid_il_ref_pics_plus1[ i ][ j ] greater than 0 specifies that within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ][ j ] − 1 are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. When not present, when max_tid_ref_present_flag is equal to 0, max_tid_il_ref_pics_plus1[ i ][ j ] is inferred to be equal to 7. Otherwise, max_tid_il_ref_pics_plus1[ i ][ j ] is inferred to be default_max_tid_ref_pics_plus1.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).