| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0100-r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0031-r2 |
| --- | --- |

| Title: | Constraints for Reference Picture Set Parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

In this document bitstream constraints are proposed for short-term reference picture set parameters and long-term reference picture set parameters for SHVC and MV-HEVC. These bitstream constraints are proposed for syntax elements num_negative_pics, num_positive_pics, num_long_term_pics, num_long_term_sps considering the DPB parameters signalled in VPS which are used for the DPB operation.

In r1 revision the constraint invocation language is modified based on feedback from HLS BoG.

In r2 revision the additional constraint related to shared sub-DPB is removed based on other decisions at the meeting.

# Introduction

In SHVC and MV-HEVC short-term reference picture set is signalled using st_ref_pic_set( stRpsIdx ) syntax structure which includes num_negative_pics and num_positive_pics syntax elements. num_negative_pics specifies the number of entries in the stRpsIdx-th candidate short-term RPS that have picture order count values less than the picture order count value of the current picture. num_positive_pics specifies the number of entries in the stRpsIdx-th candidate short-term RPS that have picture order count values greater than the picture order count value of the current picture.

In SHVC and MV-HEVC the long-term reference picture set information is signalled  based on parameters in Sequence Parameter Set (SPS) and slice segment header. The syntax element num_long_term_sps specifies the number of entries in the long-term RPS of the current picture that are derived based on the candidate long-term reference pictures specified in the active SPS. The syntax element num_long_term_pics specifies the number of entries in the long-term RPS of the current picture that are directly signalled in the slice segment header.

In JCTVC-P1008 [1] and JCT3V-G1004 [2], the DPB parameters are signalled in dpb_size() syntax structure in Video Parameter Set (VPS). dpb_size() signals various DPB parameters for output layer sets for number of temporal sub-layers for number of sub-DPBs and for number of layers when number of sub-DPBs is different than number of layers. The DPB operation is defined based on parameters signalled in Video Parameter Set (VPS) and Sequence Parameter Set (SPS).

We propose bitstream constraints for short-term reference picture set parameters and long-term reference picture set parameters for SHVC and MV-HEVC.  These bitstream constraints are proposed for syntax elements num_negative_pics, num_positive_pics, num_long_term_pics, num_long_term_sps considering the DPB parameter signalled in VPS which are used for the DPB operation.

# Proposal

In this section bitstream constraints are proposed for syntax elements num_negative_pics, num_positive_pics, num_long_term_pics, num_long_term_sps considering the DPB parameter signalled in VPS which are used for the DPB operation.

Our proposed changes are described below.

The proposed changes are highlighted below compared to JCTVC-L1003 and JCTVC-P1008.

- F.7.4.7  Slice segment header semantics

- F.7.4.7.1 General slice segment header semantics

…

num_long_term_sps specifies the number of entries in the long-term RPS of the current picture that are derived based the candidate long-term reference pictures specified in the active SPS. The value of num_long_term_sps shall be in the range of 0 to num_long_term_ref_pics_sps, inclusive. When not present, the value of num_long_term_sps is inferred to be equal to 0.

num_long_term_pics specifies the number of entries in the long-term RPS of the current picture that are directly signalled in the slice header. When not present, the value of num_long_term_pics is inferred to be equal to 0.

When vps_extension_flag is equal to 1 in the active VPS , the sum of NumNegativePics[ CurrRpsIdx ], NumPositivePics[ CurrRpsIdx ], num_long_term_sps, and num_long_term_pics shall be less than or equal to max_vps_layer_dec_pic_buff_minus1[ olsIdx ][ LayerIdxInVps[ nuh_layer_id ] ][ MaxSubLayersInLayerSetMinus1[ olsIdx ] ] for each value of olsIdx such that LayerSetIdxForOutputLayerSet[ olsIdx ]-th layer set includes the layer with the current nuh_layer_id value.

When nuh_layer_id is equal to 0,  the sum of NumNegativePics[ CurrRpsIdx ], NumPositivePics[ CurrRpsIdx ], num_long_term_sps, and num_long_term_pics shall be less than or equal to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ].

- F.7.4.8 Short-term reference picture set semantics

The specifications in subclause 7.4.8 apply with the following modifications and additions.

num_negative_pics specifies the number of entries in the stRpsIdx-th candidate short-term RPS that have picture order count values less than the picture order count value of the current picture.

When vps_extension_flag is equal to 1 in the active VPS, the value of num_negative_pics shall be in the range of 0 to max_vps_layer_dec_pic_buff_minus1[ olsIdx ][ LayerIdxInVps[ nuh_layer_id ] ][ MaxSubLayersInLayerSetMinus1[ olsIdx ] ] for each value of olsIdx such that LayerSetIdxForOutputLayerSet[ olsIdx ]-th layer set includes the layer with the current nuh_layer_id value..

When nuh_layer_id is equal to 0,  the value of num_negative_pics shall be in the range of 0 to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ], inclusive.

num_positive_pics specifies the number of entries in the stRpsIdx-th candidate short-term RPS that have picture order count values greater than the picture order count value of the current picture. When vps_extension_flag is equal to 1 in the active VPS, the value of num_positive_pics shall be in the range of 0 to max_vps_layer_dec_pic_buff_minus1[ olsIdx ][ LayerIdxInVps[ nuh_layer_id ] ][ MaxSubLayersInLayerSetMinus1[ olsIdx ] ] − num_negative_pics for each value of olsIdx such that LayerSetIdxForOutputLayerSet[ olsIdx ]-th layer set includes the layer with the current nuh_layer_id value..

When nuh_layer_id is equal to 0,  the value of num_positive_pics shall be in the range of 0 to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ] − num_negative_pics, inclusive.

# Conclusion

In this document bitstream constraints are proposed for syntax elements num_negative_pics, num_positive_pics, num_long_term_pics, num_long_term_sps considering the DPB parameter signalled in VPS which are used for the DPB operation.

We propose to adopt the proposals in the draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, Y.-K. Wang, “High Efficiency Video Coding (HEVC) Scalable Extension Draft 5,” JCTVC-P1008, San Jose, January 2014.

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 7,” JCT3V-G1004, San Jose, January 2014.