| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0101 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0032 |
| --- | --- |

| Title: | On Bitstream Partition Buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

In this document various constraints are proposed for bitstream partition buffer related parameters. Additionally signalling modifications are proposed for bitstream partition buffer HRD parameters.

In r1 revision a variant bitstream conformance constraint is added for Proposal 12 and Proposal 13.

# Introduction

In JCTVC-P1008 [1] and JCT3V-G1004 [2], bitstream partitioning is defined. A bitstream partition is a sequence of bits, in the form of a NAL unit stream or a byte stream, that is a subset of a bitstream according to a partitioning. A base bitstream partition is a bitstream partition that is also a conforming bitstream itself. A bitstream specific CPB operation and bitsream partition specific CPB operation is defined in Annex C. For each bitstream conformance test, the operation of the CPB and the BPB is specified in C.2. Bitstream partition specific HRD parameters can be signalled in:

- VPS VUI in vps_vui_bsp_hrd_parameters() syntax structure and/ or

- Bitstream partition HRD parameters SEI message in bsp_hrd( ).

In this document various constraints are proposed for bitstream partition buffer related parameters. Additionally signalling modifications are proposed for bitstream partition buffer HRD parameters.

Following is proposed:

- Proposal 1: A bitstream restriction which makes sure that when HRD parameters are signalled for bitstream partition in VPS VUI, the signalled index of hrd_parameters() structure for a bitstream partition is within a valid range of hrd_parameters() structures signalled in the vps_vui_bsp_hrd_parameters() syntax structure.

- Proposal 2: A bitstream restriction which makes sure that when HRD parameters are signalled for bitstream partition in bitstream partition HRD parameters SEI message, the signalled index of hrd_parameters() structure for a bitstream partition is within a valid range of hrd_parameters() structures signalled in the bsp_hrd() SEI message structure.

- Proposal 3: A bitstream restriction which makes sure that when HRD parameters are signalled for bitstream partition in VPS VUI,  the signalled index of a delivery schedule within hrd_parameters() structure is within a valid range of delivery schedules signalled in the sub_layer_hrd_parameters structure in that hrd_parameters() structure.

- Proposal 4: A bitstream restriction which makes sure that when HRD parameters are signalled for bitstream partition in bitstream partition HRD parameters SEI message,  the signalled index of a delivery schedule within hrd_parameters() structure is within a valid range of delivery schedules signalled in the sub_layer_hrd_parameters structure in that hrd_parameters() structure.

- Proposal 5: A bitstream restriction which makes sure that when only one bitstream partition is signalled for a layer set in vps_vui_bsp_hrd_parameters() the bitstream partition does not exactly match the corresponding layer set.

- Proposal 6: A bitstream restriction which makes sure that when only one bitstream partition is signalled for a layer set in bitstream partition HRD parameters SEI message the bitstream partition does not exactly match the corresponding layer set.

- Proposal 7: Since there is a limited range of values that can be assigned to bsp_comb_hrd_idx[ h ][ i ][ j ]  syntax element in VPS VUI HRD parameters - vps_vui_bsp_hrd_parameters() syntax and that it is typically not likely that some index values are taken more often than others it is proposed to use u(v) coding for this syntax element instead of ue(v) coding currently used.

- Proposal 8: Since there is a limited range of values that can be assigned to sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ]  syntax element in bitstream partition HRD parameters SEI message syntax and that it is typically not likely that some index values are taken more often than others it is proposed to use u(v) coding for this syntax element instead of ue(v) coding currently used.

- Proposal 9: A modification of the syntax and semantics for bitstream partition HRD parameters SEI message is proposed. Also a bitstream constraint related to hybrid scalability is proposed consistent with agreements in AHG15 meetings, JCTVC-Q0015 AHG report and specification draft text output from AHG15.

- Proposal 10: With the current signalling of bitstream partition specific HRD parameters in vps_vui_bsp_hrd_parameters() it is possible to include a layer in a layer set in multiple bitstream partitions. It is asserted that there may not be any benefit of doing partitioning in this manner. Two options are proposed to prevent including a layer in a layer set in multiple bitstream partitions.

- Proposal 11: With the current signalling of bitstream partition specific HRD parameters in bitstream partition HRD parameters SEI message it is possible to include a layer in a layer set in multiple bitstream partitions. It is asserted that there may not be any benefit of doing partitioning in this manner. Two options are proposed to prevent including a layer in a layer set in multiple bitstream partitions.

- Proposal 12: It is proposed that each bitstream partition of a layer set must be distinct. i.e. any two bitstream partitions of a layer set can not be identical. It is proposed that a bitstream partition signalled in VPS VUI bitstream partition HRD parameters for a layer set can not exactly match another bitstream partition for the same layer set.

It is noted that if proposal 10 is adopted then proposal 12 is not necessary.

- Proposal 13: It is proposed that each bitstream partition of a layer set must be distinct. i.e. any two bitstream partitions of a layer set can not be identical. It is proposed that a bitstream partition signalled in bitstream partition HRD parameters SEI message for a layer set can not exactly match another bitstream partition for the same layer set.

It is noted that if proposal 11 is adopted then proposal 13 is not necessary.

# Proposal

In this document various constraints are proposed for bitstream partition buffer related parameters. Additionally signalling modifications are proposed for bitstream partition buffer HRD parameters.

Our proposed changes are described below.

The proposed changes are highlighted below compared to JCTVC-P1008 [1].

- Proposal 1: This bitstream restriction makes sure when HRD parameters are signalled for bitstream partition in VPS VUI, the signalled index of hrd_parameters() structure for a bitstream partition is within a valid range of hrd_parameters() structures signalled in the vps_vui_bsp_hrd_parameters() syntax structure.

It is a requirement of bitstream conformance that bsp_comb_hrd_idx[ h ][ i ][ j ]  shall be in the range of 0 to vps_num_bsp_hrd_parameters_minus1, inclusive.

- Proposal 2: This bitstream restriction makes sure that when HRD parameters are signalled for bitstream partition in SEI message, the signalled index of hrd_parameters() structure for a bitstream partition is within a valid range of hrd_parameters() structures signalled in the bsp_hrd() SEI message structure.

It is a requirement of bitstream conformance that sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ]  shall be in the range of 0 to sei_num_bsp_hrd_parameters_minus1, inclusive.

- Proposal 3: This bitstream restriction makes sure that when HRD parameters are signalled for bitstream partition in VPS VUI,  the signalled index of a delivery schedule within hrd_parameters() structure is within a valid range of delivery schedules signalled in the sub_layer_hrd_parameters structure in that hrd_parameters() structure.

It is a requirement of bitstream conformance that bsp_comb_sched_idx[ h ][ i ][ j ]  shall be in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive where cpb_cnt_minus1[ HighestTid ] is found in the sub_layer_hrd_parameters( HighestTid ) structure from hrd_parameters() structure corresponding to the index bsp_comb_hrd_idx[ h ][ i ][ j ].

- Proposal 4: This bitstream restriction makes sure that when HRD parameters are signalled for bitstream partition in SEI message,  the signalled index of a delivery schedule within hrd_parameters() structure is within a valid range of delivery schedules signalled in the sub_layer_hrd_parameters structure in that hrd_parameters() structure.

It is a requirement of bitstream conformance that sei_bsp_comb_sched_idx[ lsIdx ][ i ][ j ]  shall be in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive where cpb_cnt_minus1[ HighestTid ] is found in the sub_layer_hrd_parameters( HighestTid ) structure from hrd_parameters() structure corresponding to the index sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ].

- Proposal 5: This bitstream restriction makes sure that when only one bitstream partition is signalled for a layer set in vps_vui_bsp_hrd_parameters() the bitstream partition does not exactly match the corresponding layer set.

It is asserted that when num_bitstream_partitions[ h ] is equal to 1 if the signalled bitstream partition exactly matches the layer set then the HRD parameters for this can be already signalled in VPS and such a bitstream partition is not needed to be signalled. Instead in such cases the bitstream specific CPB operation should be used.

It is a requirement of bitstream conformance that, when num_bitstream_partitions[ h ] is equal to 1 for any value of h in the range 1 to vps_num_layer_set_minus1, inclusive, the value of layer_in_bsp_flag[ h ][ 0 ][ j ] should be equal to 0 and the value of layer_id_included_flag[ h ][ j ] should be equal to 1 for at least one value of j in the range 0 to vps_max_layers_minus1, inclusive.

Variant based on AHG 15 JCTVC-Q0015_attachment_d1.docx:

It is a requirement of bitstream conformance that, when num_bitstream_partitions[ h ] is equal to 1 for any value of h in the range 1 to vps_num_layer_set_minus1, inclusive, the value of layer_in_bsp_flag[ h ][ 0 ][ j ] should be equal to 0 for at least one value of j in the range 0 to NumLayersInIdList[ h ] – 1, inclusive.

- Proposal 6: This bitstream restriction makes sure that when only one bitstream partition is signalled for a layer set in bitstream partition HRD parameters SEI message the bitstream partition does not exactly match the corresponding layer set.

It is asserted that when num_sei_bitstream_partitions_minus1[ lsIdx ] is equal to 0 if the signalled bitstream partition exactly matches the layer set then the HRD parameters for this can be already signalled in VPS and such a bitstream partition is not needed to be signalled. Instead in such cases the bitstream specific CPB operation should be used.

It is a requirement of bitstream conformance that, when num_sei_bitstream_partitions_minus1[ lsIdx ] is equal to 0 for any value of h in the range 0 to nesting_num_ops_minus1, inclusive, the value of sei_layer_in_bsp_flag[ lsIdx ][ 0 ][ j ] should be equal to 0 and the value of layer_id_included_flag[ lsIdx ][ j ] should be equal to 1 for at least one value of j in the range 0 to vps_max_layers_minus1, inclusive.

Variant based on AHG 15 JCTVC-Q0015_attachment_d1.docx:

It is a requirement of bitstream conformance that, when num_sei_bitstream_partitions_minus1[ lsIdx ] is equal to 0 for any value of h in the range 0 to nesting_num_ops_minus1, inclusive, the value of sei_layer_in_bsp_flag[ lsIdx ][ 0 ][ j ] should be equal to 0 for at least one value of j in the range 0 to NumLayersInIdList[ lsIdx ] – 1, inclusive.

- Proposal 7: Since there is a limited range of values that can be assigned to bsp_comb_hrd_idx[ h ][ i ][ j ]  syntax element in VPS VUI HRD parameters - vps_vui_bsp_hrd_parameters() syntax and that it is typically not likely that some index values are taken more often than others it is proposed to use u(v) coding for this syntax element instead of ue(v) coding currently used.

- F.7.3.2.1.6 VPS VUI bitstream partition HRD parameters syntax

| vps_vui_bsp_hrd_parameters( ){ | Descriptor |
| --- | --- |
| vps_num_bsp_hrd_parameters_minus1 | ue(v) |
| … | u(1) |
| if( num_bitstream_partitions[ h ] ) { |  |
| num_bsp_sched_combinations[ h ] | ue(v) |
| for( i = 0; i < num_bsp_sched_combinations[ h ]; i++ ) |  |
| for( j = 0; j < num_bitstream_partitions[ h ]; j++ ) { |  |
| bsp_comb_hrd_idx[ h ][ i ][ j ] | ue(v) u(v) |
| bsp_comb_sched_idx[ h ][ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

bsp_comb_hrd_idx[ h ][ i ][ j ] specifies the index of hrd_parameters( ) within the vps_vui_bsp_hrd_parameters( ) syntax structure used in the i-th combination of a delivery schedule and hrd_parameters( ) specified for the bitstream partition with index j and for the layer set with index h. The length of the bsp_comb_hrd_idx[ h ][ i ][ j ] syntax element is Ceil( Log2( vps_num_bsp_hrd_parameters_minus1+1 ) ) bits.

- Proposal 8: Since there is a limited range of values that can be assigned to sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ]  syntax element in bitstream partition HRD parameters SEI message syntax and that it is typically not likely that some index values are taken more often than others it is proposed to use u(v) coding for this syntax element instead of ue(v) coding currently used.

- F.14.1.5  Bitstream partition HRD parameters SEI message syntax

| bsp_hrd( payloadSize ) { | Descriptor |
| --- | --- |
| sei_num_bsp_hrd_parameters_minus1 | ue(v) |
| for( i = 0; i <= sei_num_bsp_hrd_parameters_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| sei_bsp_cprms_present_flag[ i ] | u(1) |
| hrd_parameters( sei_bsp_cprms_present_flag[ i ], nesting_max_temporal_id_plus1[ 0 ] − 1 ) |  |
| } |  |
| for( h=0; h <= nesting_num_ops_minus1; h++ ) { |  |
| lsIdx = nesting_op_idx[ h ] |  |
| num_sei_bitstream_partitions_minus1[ lsIdx ] | ue(v) |
| for( i = 0; i <= num_sei_bitstream_partitions_minus1[ lsIdx ]; i++ ) |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| sei_layer_in_bsp_flag[ lsIdx ][ i ][ j ] | u(1) |
| sei_num_bsp_sched_combinations_minus1[ lsIdx ] | ue(v) |
| for( i = 0; i <= sei_num_bsp_sched_combinations_minus1[ lsIdx ]; i++ ) |  |
| for( j = 0; j <= sei_num_sei_bitstream_partitions_minus1[ lsIdx ]; j++ ) { |  |
| sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ] | ue(v) u(v) |
| sei_bsp_comb_sched_idx[ lsIdx ][ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ] specifies the index of hrd_parameters( ) within this SEI message used in the i-th combination of a delivery schedule and hrd_parameters( ) specified for the bitstream partition with index j and for the layer set with index lsIdx. The length of the sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ] syntax element is Ceil( Log2( sei_num_bsp_hrd_parameters_minus1+1 ) ) bits.

Proposal 9:

A modification of the syntax and semantics for bitstream partition HRD parameters SEI message is proposed. Also a bitstream constraint related to hybrid scalability is proposed consistent with agreements in AHG15 meetings, JCTVC-Q0015 AHG report and specification draft text output from AHG15.

- F.14.1.5  Bitstream partition HRD parameters SEI message syntax

| bsp_hrd( payloadSize ) { | Descriptor |
| --- | --- |
| sei_num_bsp_hrd_parameters_minus1 | ue(v) |
| for( i = 0; i <= sei_num_bsp_hrd_parameters_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| sei_bsp_cprms_present_flag[ i ] | u(1) |
| hrd_parameters( sei_bsp_cprms_present_flag[ i ], nesting_max_temporal_id_plus1[ 0 ] − 1 ) |  |
| } |  |
| for( h=0; h <= nesting_num_ops_minus1; h++ ) { |  |
| lsIdx = nesting_op_idx[ h ] |  |
| num_sei_bitstream_partitions_minus1[ lsIdx ] | ue(v) |
| for( i = 0; i <= num_sei_bitstream_partitions_minus1[ lsIdx ]; i++ ) |  |
| for( j = 0; j <= vps_max_layers_minus1 NumLayerInIdList[ lsIdx ]; j++ ) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ] |  |
| sei_layer_in_bsp_flag[ lsIdx ][ i ][ j ] | u(1) |
| sei_num_bsp_sched_combinations_minus1[ lsIdx ] | ue(v) |
| for( i = 0; i <= sei_num_bsp_sched_combinations_minus1[ lsIdx ]; i++ ) |  |
| for( j = 0; j <= sei_num_sei_bitstream_partitions_minus1[ lsIdx ]; j++ ) { |  |
| sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ] | ue(v) |
| sei_bsp_comb_sched_idx[ lsIdx ][ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

- F.14.2.5 Bitstream partition HRD parameters SEI message semantics

…

sei_layer_in_bsp_flag[ lsIdx ][ i ][ j ] specifies that the layer with index j LayerSetLayerIdList[ lsIdx  ][ j ]    is a part of bitstream partition with index i within the layer set with index lsIdx.

It is a requirement of bitstream conformance that the bitstream partition with index j shall not include direct or indirect reference layers of any layers in the bitstream partition with index i for any values of i and j in the range of 0 to num_sei_bitstream_partitions_minus1[ lsIdx ], inclusive, such that i is less than j.

It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0 and sei_layer_in_bsp_flag[ lsIdx ][ i ][ 0 ] is equal to 1 for any value of h in the range of 0 to nesting_num_ops_minus1, inclusive, and any value of i in the range of 0 to num_sei_bitstream_partitions_minus1[ lsIdx ] , inclusive, the value of sei_layer_in_bsp_flag[ lsIdx ][ i ][ j ] for at least one value of j in the range of 1 to NumLayersInIdList[ lsIdx ] – 1, inclusive, shall be equal to 1.

Proposal 10:

With the current signalling of bitstream partition specific HRD parameters in vps_vui_bsp_hrd_parameters() it is possible to include a layer in a layer set in multiple bitstream partitions. It is asserted that there may not be any benefit of doing partitioning in this manner.

Also if this is allowed duplicate partitions could be signalled.

Two alternative options are proposed for preventing this. It is asserted that the option 1 reduces bits compared to current signalling method and so is preferred.

Option 1: Signalling in vps_vui_bsp_hrd_parameters() is modified to disallow including the same layer in a layer set in more than one bitstream partition of that layer set.

- F.7.3.2.1.6 VPS VUI bitstream partition HRD parameters syntax

| vps_vui_bsp_hrd_parameters( ){ | Descriptor |
| --- | --- |
| vps_num_bsp_hrd_parameters_minus1 | ue(v) |
| for( i = 0; i <= vps_num_bsp_hrd_parameters_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| bsp_cprms_present_flag[ i ] | u(1) |
| hrd_parameters( bsp_cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| for( h=1; h <= vps_num_layer_sets_minus1; h++ ) { |  |
| num_bitstream_partitions[ h ] | ue(v) |
| for( i = 0; i < num_bitstream_partitions[ h ]; i++ ) |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |
| for( k=0, LayerIdPrevIncluded[ h ][ j ]=0; k < i; k++) |  |
| LayerIdPrevIncluded[ h ][ j ]= LayerIdPrevIncluded[ h ][ j ] ¦ layer_in_bsp_flag[ h ][ k ][ j ] |  |
| if( layer_id_included_flag[ h ][ j ] && !LayerIdPrevIncluded[ h ][ j ]) |  |
| layer_in_bsp_flag[ h ][ i ][ j ] | u(1) |
| if( num_bitstream_partitions[ h ] ) { |  |
| num_bsp_sched_combinations[ h ] | ue(v) |
| for( i = 0; i < num_bsp_sched_combinations[ h ]; i++ ) |  |
| for( j = 0; j < num_bitstream_partitions[ h ]; j++ ) { |  |
| bsp_comb_hrd_idx[ h ][ i ][ j ] | ue(v) |
| bsp_comb_sched_idx[ h ][ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Option 1 Variant based on AHG 15 JCTVC-Q0015_attachment_d1.docx:

- F.7.3.2.1.6 VPS VUI bitstream partition HRD parameters syntax

| vps_vui_bsp_hrd_parameters( ){ | Descriptor |
| --- | --- |
| vps_num_bsp_hrd_parameters_minus1 | ue(v) |
| for( i = 0; i <= vps_num_bsp_hrd_parameters_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| bsp_cprms_present_flag[ i ] | u(1) |
| hrd_parameters( bsp_cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| for( h=1; h <= vps_num_layer_sets_minus1; h++ ) { |  |
| num_bitstream_partitions[ h ] | ue(v) |
| for( i = 0; i < num_bitstream_partitions[ h ]; i++ ) |  |
| for( j = 0; j < NumLayersInIdList[ h ]; j++ ) |  |
| for( k=0, LayerIdPrevIncluded[ h ][ j ]=0; k < i; k++) |  |
| LayerIdPrevIncluded[ h ][ j ]= LayerIdPrevIncluded[ h ][ j ] ¦ layer_in_bsp_flag[ h ][ k ][ j ] |  |
| if(!LayerIdPrevIncluded[ h ][ j ]) |  |
| layer_in_bsp_flag[ h ][ i ][ j ] | u(1) |
| if( num_bitstream_partitions[ h ] ) { |  |
| num_bsp_sched_combinations[ h ] | ue(v) |
| for( i = 0; i < num_bsp_sched_combinations[ h ]; i++ ) |  |
| for( j = 0; j < num_bitstream_partitions[ h ]; j++ ) { |  |
| bsp_comb_hrd_idx[ h ][ i ][ j ] | ue(v) |
| bsp_comb_sched_idx[ h ][ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- F.7.4.2.1.6 VPS VUI bitstream partition HRD parameters semantics

…

layer_in_bsp_flag[ h ][ i ][ j ] specifies that the layer with index j LayerSetLayerIdList[ h ][[ j ] is a part of bitstream partition with index i within the layer set with index h. When not present the value of layer_in_bsp_flag[ h ][ i ][ j ] is inferred to be equal to 0.

It is a requirement of bitstream conformance that bitstream partition with index j shall not include direct or indirect reference layers of any layers in bitstream partition i for any values of i and j in the range of 0 to num_bitstream_partitions[ h ] − 1, inclusive, such that i is less than j.

It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0 and layer_in_bsp_flag[ h ][ i ][ 0 ] is equal to 1 for any value of h in the range of 1 to vps_num_layer_sets_minus1, inclusive, and any value of i in the range of 0 to num_bitstream_partitions[ h ] – 1, inclusive, the value of layer_in_bsp_flag[ h ][ i ][ j ] for at least one value of j in the range of 1 to NumLayersInIdList[ h ] – 1, inclusive, shall be equal to 1.

Option 2: A bitstream constraint is proposed to disallow including the same layer in a layer set in more than one bitstream partition of that layer set when signalling bitstream partition information in VPS VUI bitstream partition HRD parameter semantics.

It is a requirement of bitstream conformance that, for any value of h in the range 1 to vps_num_layer_set_minus1, inclusive, the value of layer_in_bsp_flag[ h ][ i ][ j ] shall be equal to 1 for at most one value of i in the range 0 to num_bitstream_partitions[ h ]-1, inclusive.

- Proposal 11: With the current signalling of bitstream partition specific HRD parameters in bitstream partition HRD parameters SEI message it is possible to include a layer in a layer set in multiple bitstream partitions. It is asserted that there may not be any benefit of doing partitioning in this manner.

Also if this is allowed duplicate partitions could be signalled.

Two alternative options are proposed for preventing this. It is asserted that the option 1 reduces bits compared to current signalling method and so is preferred.

Option 1: Signalling in bsp_hrd() is modified to disallow including the same layer in a layer set in more than one bitstream partition of that layer set.

- F.14.1.5  Bitstream partition HRD parameters SEI message syntax

| bsp_hrd( payloadSize ) { | Descriptor |
| --- | --- |
| sei_num_bsp_hrd_parameters_minus1 | ue(v) |
| for( i = 0; i <= sei_num_bsp_hrd_parameters_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| sei_bsp_cprms_present_flag[ i ] | u(1) |
| hrd_parameters( sei_bsp_cprms_present_flag[ i ], nesting_max_temporal_id_plus1[ 0 ] − 1 ) |  |
| } |  |
| for( h=0; h <= nesting_num_ops_minus1; h++ ) { |  |
| lsIdx = nesting_op_idx[ h ] |  |
| num_sei_bitstream_partitions_minus1[ lsIdx ] | ue(v) |
| for( i = 0; i <= num_sei_bitstream_partitions_minus1[ lsIdx ]; i++ ) |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |
| for( k=0, SEILayerIdPrevIncluded[ lsIdx ][ j ]=0; k < i; k++) |  |
| SEILayerIdPrevIncluded[ lsIdx ][ j ]= SEILayerIdPrevIncluded[ lsIdx ][ j ] ¦ sei_layer_in_bsp_flag[ lsIdx ][ k ][ j ] |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ] && !SEILayerIdPrevIncluded[ lsIdx ][ j ] ) |  |
| sei_layer_in_bsp_flag[ lsIdx ][ i ][ j ] | u(1) |
| sei_num_bsp_sched_combinations_minus1[ lsIdx ] | ue(v) |
| for( i = 0; i <= sei_num_bsp_sched_combinations_minus1[ lsIdx ]; i++ ) |  |
| for( j = 0; j <= sei_num_sei_bitstream_partitions_minus1[ lsIdx ]; j++ ) { |  |
| sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ] | ue(v) |
| sei_bsp_comb_sched_idx[ lsIdx ][ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

Option 1 Variant based on Proposal 9 :

- F.14.1.5  Bitstream partition HRD parameters SEI message syntax

| bsp_hrd( payloadSize ) { | Descriptor |
| --- | --- |
| sei_num_bsp_hrd_parameters_minus1 | ue(v) |
| for( i = 0; i <= sei_num_bsp_hrd_parameters_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| sei_bsp_cprms_present_flag[ i ] | u(1) |
| hrd_parameters( sei_bsp_cprms_present_flag[ i ], nesting_max_temporal_id_plus1[ 0 ] − 1 ) |  |
| } |  |
| for( h=0; h <= nesting_num_ops_minus1; h++ ) { |  |
| lsIdx = nesting_op_idx[ h ] |  |
| num_sei_bitstream_partitions_minus1[ lsIdx ] | ue(v) |
| for( i = 0; i <= num_sei_bitstream_partitions_minus1[ lsIdx ]; i++ ) |  |
| for( j = 0; j < = vps_max_layers_minus1 NumLayerInIdList[ lsIdx ]; j++ ) |  |
| for( k=0, SEILayerIdPrevIncluded[ lsIdx ][ j ]=0; k < i; k++) |  |
| SEILayerIdPrevIncluded[ lsIdx ][ j ]= SEILayerIdPrevIncluded[ lsIdx ][ j ] ¦ sei_layer_in_bsp_flag[ lsIdx ][ k ][ j ] |  |
| if(!SEILayerIdPrevIncluded[ lsIdx ][ j ]) |  |
| sei_layer_in_bsp_flag[ lsIdx ][ i ][ j ] | u(1) |
| sei_num_bsp_sched_combinations_minus1[ lsIdx ] | ue(v) |
| for( i = 0; i <= sei_num_bsp_sched_combinations_minus1[ lsIdx ]; i++ ) |  |
| for( j = 0; j <= sei_num_sei_bitstream_partitions_minus1[ lsIdx ]; j++ ) { |  |
| sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ] | ue(v) |
| sei_bsp_comb_sched_idx[ lsIdx ][ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

sei_layer_in_bsp_flag[ lsIdx ][ i ][ j ] specifies that the layer with index j LayerSetLayerIdList[ lsIdx  ][ j ]    is a part of bitstream partition with index i within the layer set with index lsIdx. When not present the value of sei_layer_in_bsp_flag[ lsIdx ][ i ][ j ] is inferred to be equal to 0.

It is a requirement of bitstream conformance that the bitstream partition with index j shall not include direct or indirect reference layers of any layers in the bitstream partition with index i for any values of i and j in the range of 0 to num_sei_bitstream_partitions_minus1[ lsIdx ], inclusive, such that i is less than j.

It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0 and sei_layer_in_bsp_flag[ lsIdx ][ i ][ 0 ] is equal to 1 for any value of h in the range of 0 to nesting_num_ops_minus1, inclusive, and any value of i in the range of 0 to num_sei_bitstream_partitions_minus1[ lsIdx ] , inclusive, the value of sei_layer_in_bsp_flag[ lsIdx ][ i ][ j ] for at least one value of j in the range of 1 to NumLayersInIdList[ lsIdx ] – 1, inclusive, shall be equal to 1.

Option 2: A bitstream constraint is proposed to disallow including the same layer in a layer set in more than one bitstream partition of that layer set when signalling bitstream partition information in bitstream partition HRD parameter SEI message semantics.

It is a requirement of bitstream conformance that, for any value of h in the range 0 to nesting_num_ops_minus1, inclusive, the value of sei_layer_in_bsp_flag[ lsIdx ][ i ][ j ] shall be equal to 1 for at most one value of i in the range 0 to num_sei_bitstream_partitions_minus1[ lsIdx ], inclusive.

- Proposal 12: It is proposed that each bitstream partition of a layer set must be distinct. i.e. Two bitstream partitions of a layer set can not be identical.

It is noted that if proposal 10 is adopted then proposal 12 is not necessary.

It is proposed that a bitstream partition signalled in VPS VUI bitstream partition HRD parameters for a layer set can not exactly match another bitstream partition for the same layer set. It is asserted that it is wasteful and unnecessary to signal same bitstream partitions for a layer set more than once.

It is a requirement of bitstream conformance that, for any value of h in the range 1 to vps_num_layer_set_minus1, inclusive, the value of layer_in_bsp_flag[ h ][ k ][ j ] shall not be equal to the value of layer_in_bsp_flag[ h ][ p ][ j ] for at least one value of j in the range 0 to vps_max_layers_minus1, inclusive, with k not equal to p, and for any value of  k in the range 0 to num_bitstream_partitions[ h ]-1, inclusive and for any value of p in the range of 0 to num_bitstream_partitions[ h ]-1, inclusive.

Variant based on AHG 15 JCTVC-Q0015_attachment_d1.docx:

It is a requirement of bitstream conformance that, for any value of h in the range of 1 to vps_num_layer_set_minus1, inclusive, the value of layer_in_bsp_flag[ h ][ k ][ j ] shall not be equal to the value of layer_in_bsp_flag[ h ][ p ][ j ] for at least one value of j in the range of 0 to NumLayersInIdList[ h  ] - 1, inclusive, with k not equal to p, for any value of k in the range of 0 to num_bitstream_partitions[ h ]-1, inclusive, and for any value of p in the range of 0 to num_bitstream_partitions[ h ]-1, inclusive.

- Proposal 13:

It is proposed that each bitstream partition of a layer set must be distinct. i.e. Two bitstream partitions of a layer set can not be identical.

It is noted that if proposal 11 is adopted then proposal 13 is not necessary.

It is proposed that a bitstream partition signalled in bitstream partition HRD parameters SEI message for a layer set can not exactly match another bitstream partition for the same layer set. It is asserted that it is wasteful and unnecessary to signal same bitstream partitions for a layer set more than once.

It is a requirement of bitstream conformance that, for any value of h in the range 0 to nesting_num_ops_minus1, inclusive, the value of sei_layer_in_bsp_flag[ lsIdx ][ k ][ j ] shall not be equal to the value of sei_layer_in_bsp_flag[ lsIdx ][ p ][ j ] for at least one value of j in the range 0 to vps_max_layers_minus1, inclusive, with k not equal to p and for any value of  k in the range 0 to num_sei_bitstream_partitions_minus1[ lsIdx ], inclusive and for any value of p in the range of 0 to num_sei_bitstream_partitions_minus1[ lsIdx ], inclusive.

Variant based on Proposal 9:

It is a requirement of bitstream conformance that, for any value of h in the range of 0 to nesting_num_ops_minus1, inclusive, the value of sei_layer_in_bsp_flag[ lsIdx ][ k ][ j ] shall not be equal to the value of sei_layer_in_bsp_flag[ lsIdx ][ p ][ j ] for at least one value of j in the range of 0 to NumLayersInIdList[ lsIdx ] - 1, inclusive, with k not equal to p, for any value of k in the range of 0 to num_sei_bitstream_partitions_minus1[ lsIdx ], inclusive, and for any value of p in the range of 0 to num_sei_bitstream_partitions_minus1[ lsIdx ], inclusive.

# Conclusion

In this document various constraints are proposed for bitstream partition buffer related parameters. Additionally signalling modifications are proposed for bitstream partition buffer HRD parameters.

We propose to adopt all the proposals in the draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, Y.-K. Wang, “High Efficiency Video Coding (HEVC) Scalable Extension Draft 5,” JCTVC-P1008, San Jose, January 2014.

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 7,” JCT3V-G1004, San Jose, January 2014.