| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0103 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0034 |
| --- | --- |

| Title: | On DPB Profile Level Limits |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document defines per-layer and shared sub-DPB capacity level limits for SHVC and MV-HEVC profiles. Maximum DPB size level limit constraints for per-layer sub-DPB parameters max_vps_layer_dec_pic_buff_minus1[ ][ ][ ] and for (possibly) shared sub-DPB parameters max_vps_dec_pic_buffering_minus1[ ][ ][ ] for SHVC scalable main and scalable main 10 and MV-HEVC stereo main profiles are proposed.

# Introduction

In JCTVC-P1008 H.11.1.2 defines strawman SHVC Scalable Main and Scalable Main 10 profiles. Also JCT3V-G1004 G.11.1.2 defines Stereo Main profile.

Regarding level limit constraints related to maximum DPB size:

- SHVC Scalable Main and Scalable Main 10 profiles define that each layer in the TargetDecLayerIdList shall obey profile-specific level limit in A.4.1 d.

The constraint in A.4.1 d applies to sps_max_dec_pic_buffering_minus1[ HighestTid ].

- MV-HEVC Stereo Main profile defines a similar constraint on sps_max_dec_pic_buffering_minus1[ HighestTid ].

In JCTVC-P1008 [1] and JCT3V-G1004 [2], the DPB parameters are signalled in dpb_size() syntax structure in Video Parameter Set (VPS). dpb_size() signals various DPB parameters for output layer sets for number of temporal sub-layers for number of sub-DPBs and for number of layers when number of sub-DPBs is different than number of layers. The DPB operation is defined based on parameters signalled in Video Parameter Set (VPS) and Sequence Parameter Set (SPS).

In JCTVC-P1008 and JCT3V-G1004, the representation formats signalled in VPS provide an upper bound on the representation format values used by layers with nuh_layer_id > 0. This is because when update_rep_format_flag is equal to 1, it is a requirement of bitstream conformance that the value of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, or bit_depth_chroma_minus8 shall be less than or equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, or bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

Because of this it is asserted that the per-layer maximum sub-DPB size level limit constraint should use the appropriate representation format parameters signalled in VPS.

For the (possibly) shared sub-DPBs currently no maximum sub-DPB size level limits are defined in SHVC scalable main and scalable main 10 and MV-HEVC stereo main profiles. The qualification “(possibly)” refers to the aspect that depending upon the representation format of each layer the sub-DPB may not actually be shared among layers. For brevity the qualification “(possibly)” is omitted from now on in this document when referring to the shared sub-DPB.

This document defines per-layer and shared DPB capacity level limits for SHVC and MV-HEVC profiles. Maximum DPB size level limit constraints for per-layer sub-DPB parameters max_vps_layer_dec_pic_buff_minus1[ ][ ][ ] and for shared sub-DPB parameters max_vps_dec_pic_buffering_minus1[ ][ ][ ] for SHVC scalable main and scalable main 10 and MV-HEVC stereo main profiles are proposed.

To help define the constraints on shared sub-DPB a change in the “F.7.4.3.1.3 DPB size semantics” to keep track of variables during derivation of the number of sub-DPBs and assignment of a sub-DPB to each layer included in the layer set is also proposed.

# Proposal

In this section per-layer and shared DPB capacity level limits for SHVC and MV-HEVC profiles are proposed. Following is proposed:

- Proposal 1: Maximum DPB size level limit constraints for per-layer sub-DPB parameters max_vps_layer_dec_pic_buff_minus1[ ][ ][ ]

- Proposal 2: Maximum DPB size level limit constraints for shared sub-DPB parameters max_vps_dec_pic_buffering_minus1[ ][ ][ ] for SHVC scalable main and scalable main 10 and MV-HEVC stereo main profiles are proposed.

To help define the constraints on shared sub-DPB a change in the “F.7.4.3.1.3 DPB size semantics” to keep track of variables during derivation of the number of sub-DPBs and assignment of a sub-DPB to each layer included in the layer set is also proposed.

Our proposed changes are described below.

The proposed changes are highlighted below compared to JCTVC-P1008.

Similar modification with addition of per-layer sub-DPB capacity limit and shared sub-DPB capacity limit is proposed to be added to JCT3V-G1004 – MV-HEVC Stereo main profile in section G.11.2.1 “General Tier and Level Limits” section.

Proposal 1: Per-Layer DPB Capacity Limit

Maximum DPB size level limit constraints are proposed for per-layer sub-DPB parameters max_vps_layer_dec_pic_buff_minus1[ ][ ][ ].

H.1 Profiles, tiers, and levels

H.1.1 Profiles

      - H.1.1.1 General

TBD.

      - H.1.1.2 Scalable Main and Scalable Main 10 profiles

Bitstreams containing output layer sets conforming to the Scalable Main or Scalable Main 10 profiles shall obey the following constraints on a derived sub-bitstream for the output layer set, with layerSetIdx being the layer set for the output layer set conforming to Scalable Main or Scalable Main 10 profile, respectively, derived by invoking the sub-bitstream extraction process as specified in subclause Error: Reference source not found with tIdTarget equal to 7 and with TargetDecLayerIdList containing the nuh_layer_id values of the layer set with the index layerSetIdx, and TargetOutputLayerSetIdx containing the index to the list of output layer sets specified by the VPS of the target output layer set.

Bitstreams containing output layer sets conforming to the Scalable Main or Scalable Main 10 profiles shall also obey the following constraints on the base layer bitstream derived by invoking the sub-bitstream extraction process as specified in subclause Error: Reference source not found with tIdTarget equal to 7 and with TargetDecLayerIdList containing only one nuh_layer_id value that is equal to 0 as inputs.

The base layer bitstream derived from bitstreams conforming to the Scalable Main profile shall obey the following constraints:

–	The base layer bitstream shall obey all constraints of the Main profile specified in subclause A.3.2.

–	SPSs of the base layer bitstream shall have general_profile_idc equal to 1 or general_profile_compatibility_flag[ 1 ] equal to 1.

The base layer bitstream derived from bitstreams conforming to the Scalable Main 10 profile shall obey the following constraints:

–	The base layer bitstream shall obey all constraints of the Main 10 profile specified in subclause A.3.3.

–	SPSs of the base layer bitstream shall have general_profile_idc equal to 1 or 2, or general_profile_compatibility_flag[ 1 ] or general_profile_compatibility_flag[ 2 ] equal to 1

The derived sub-bitstream for an output layer set conforming to the Scalable Main or Scalable Main 10 profiles shall obey the following constraints:

–	All active SPSs for the sub-bitstream shall have chroma_format_idc equal to 1 only.

–	CtbLog2SizeY derived from any active SPS for the sub-bitstream shall be in the range of 4 to 6, inclusive.

–	The variables NumResampling, NumSampleResampling, and NumMotionResampling shall be less than or equal to 1 for each decoded picture with nuh_layer_id included in the layer set with the index layerSetIdx.

–	ScalabilityId[ j ][ smIdx ] shall be equal to 0 for any smIdx value not equal to 2 and for any value of j for which layer_id_included_flag[ layerSetIdx ][ j ] is equal to 1.

–	For a layer with layer id iNuhLId equal to any of nuh_layer_id included in the layer set with the index layerSetIdx, the value of NumRefLayers[ iNuhLId ], which specifies the total number of direct and indirect dependent layers and is derived as in F.7.4.3.1, shall be less than or equal to 7.

–	All active SPSs shall have sps_extension_type_flag[ i ] equal to 0 only for i equal to 0, and in the range of 2 to 6, inclusive.

–	All active PPSs shall have pps_extension_type_flag[ i ] equal to 0 only for i in the range of 0 to 6, inclusive.

The derived sub-bitstream for an output layer set conforming to the Scalable Main profile shall obey the following constraints:

–	All active SPSs for the sub-bitstream shall have bit_depth_luma_minus8 equal to 0 only.

–	All active SPSs for the sub-bitstream shall have bit_depth_chroma_minus8 equal to 0 only.

The derived sub-bitstream for an output layer set conforming to the Scalable Main 10 profile shall obey the following constraints:

–	All active SPSs for the sub-bitstream shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

–	All active SPSs for the sub-bitstream shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

H.1.2 Tiers and levels

      - H.1.2.1 Profile specific tier and level limits for the Scalable Main and Scalable Main 10 profiles

Bitstreams containing output layer sets conforming to the Scalable Main or Scalable Main 10 profiles shall obey the following constraints on a derived sub-bitstream for the output layer set, with layerSetIdx being the layer set for the output layer set conforming to the Scalable Main or Scalable Main 10 profile, respectively, derived by invoking the sub-bitstream extraction process as specified in subclause Error: Reference source not found with tIdTarget equal to 7 and with TargetDecLayerIdList containing the nuh_layer_id values of the layer set with the index layerSetIdx, and TargetOutputLayerSetIdx containing the index to the list of output layer sets specified by the VPS of the target output layer set:

- Each layer in the TargetDecLayerIdList shall obey the General tier and level limits in A.4.1, and the Profile-specific level limits a), b), c), d), g), h), i), and j) for the Main and Main 10 profiles specified in A.4.2

- For each k in the range of  0 to NumLayersInIdList[ layerSetIdx] the value of max_vps_layer_dec_pic_buff_minus1[ TargetOutputLayerSetIdx ][ k ][ HighestTid ] + 1 shall be less than or equal to MaxDpbSize[ k ], which is derived as follows:

PicSizeInSamplesY[ k ] is set equal to the value of pic_width_vps_in_luma_samples multiplied by the value of pic_height_vps_in_luma_samples, of the vps_rep_format_idx[ LayerIdxInVps[ LayerSetLayerIdList[ layerSetIdx ][ k ] ] ]-th rep_format( ) syntax structure in the VPS.

if( PicSizeInSamplesY[ k ]  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize[ k ] = Min( 4 * maxDpbPicBuf, 16 )
else if( PicSizeInSamplesY[ k ]  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize[ k ] = Min( 2 * maxDpbPicBuf, 16 )	(A-1)
else if( PicSizeInSamplesY[ k ]  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize[ k ] = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxDpbSize[ k ] = maxDpbPicBuf

where MaxLumaPs is specified in Error: Reference source not found and maxDpbPicBuf is equal to 6.

Proposal 2: Shared DPB Capacity Limit

Maximum DPB size level limit constraints for shared sub-DPB parameters max_vps_dec_pic_buffering_minus1[ ][ ][ ] for SHVC scalable main and scalable main 10 and MV-HEVC stereo main profiles are proposed.

The maximum shared DPB size for each layer sharing a sub-DPB is calculated similarly as the calculation for non-shared DPB. When more than one layer shares a sub-DPB, for each layer after the first layer only a percentage of the maximum shared DPB size is added to arrive at the final shared sub-DPB capacity.

To help define the constraints on shared sub-DPB a change in the “F.7.4.3.1.3 DPB size semantics” to keep track of variables during derivation of the number of sub-DPBs and assignment of a sub-DPB to each layer included in the layer set is also proposed.

- F.7.4.3.1.3 DPB size semantics

…

–	The following applies:

subDpbCtr = 1
SubDpbAssigned[ lsIdx ][ 0 ] = 0
sSubDpbSpatRes[ lsIdx ][ 0 ][ 0 ] = layerSpatRes[ 0 ][ 0 ]
sSubDpbSpatRes[ lsIdx ] [ 0 ][ 1 ] = layerSpatRes[ 0 ][ 1 ]
sSubDpbColourFormat[ lsIdx ][ 0 ] = layerColourFormat[ 0 ] 
sSubDpbBitDepth[ lsIdx ][ 0 ][ 0 ] = layerBitDepth[ 0 ][ 0 ]
sSubDpbBitDepth[ lsIdx ][ 0 ][ 1 ] = layerBitDepth[ 0 ][ 1 ]

NumLayersShareSubDpb[ lsIdx ][ 0 ] = 1; 

for( i = 1; i < NumLayersInIdList[ lsIdx ]; i++ ) {
	newSubDpbFlag = 1
	for( j = 0; j < subDpbCtr  &&  bNewSubDpb( newSubDpbFlag>0 ); j++ )
		if( layerSpatRes[ i ][ 0 ]  = =  sSubDpbSpatRes[ lsIdx ][ j ][ 0 ]  &&
		     layerSpatRes[ i ][ 1 ]  = =  sSubDpbSpatRes[ lsIdx ][ j ][ 1 ]  &&
		     layerColourFormat[ i ]  = =  sSubDpbColourFormat[ lsIdx ][ j ]  &&
		     layerBitDepth[ i ][ 0 ]  = =  sSubDpbBitDepth[ lsIdx ][ j ][ 0 ]  &&
		     layerBitDepth[ i ][ 1 ]  = =  sSubDpbBitDepth[ lsIdx ][ j ][ 1 ] ) {
			SubDpbAssigned[ lsIdx ][ i ] = j
			newSubDpbFlag = 0
			NumLayersShareSubDpb[ lsIdx ][ j ] ++
			j = subDpbCtr
		}
	if( newSubDpbFlag ) {
		sSubDpbSpatRes[ lsIdx ][ subDpbCtr ][ 0 ] = layerSpatRes[ i ][ 0 ]
		sSubDpbSpatRes[ lsIdx ][ subDpbCtr ][ 1 ] = layerSpatRes[ i ][ 1 ]
		sSubDpbColourFormat[ lsIdx ][ subDpbCtr ] = layerColourFormat[ i ] 
		sSubDpbBitDepth[ lsIdx ][ subDpbCtr ][ 0 ] = layerBitDepth[ i ][ 0 ]
		sSubDpbBitDepth[ lsIdx ][ subDpbCtr ][ 1 ] = layerBitDepth[ i ][ 1 ]
		sSubDpbAssigned[ lsIdx ][ i ] = subDpbCtr++
	}
}
NumSubDpbs[ lsIdx ] = subDpbCtr

For the lsIdx-th layer set, the number of sub-DPBs is NumSubDpbs[ lsIdx ], and for each layer with a particular value of nuh_layer_id in the layer set, the sub-DPB with index SubDpbAssigned[ lsIdx ][ layerIdx ] is assigned, where LayerSetLayerIdList[ lsIdx ][ layerIdx ] is equal to nuh_layer_id.

…

H.1 Profiles, tiers, and levels

…

H.1.2 Tiers and levels

- H.1.2.1 Profile specific tier and level limits for the Scalable Main and Scalable Main 10 profiles

Bitstreams containing output layer sets conforming to the Scalable Main or Scalable Main 10 profiles shall obey the following constraints on a derived sub-bitstream for the output layer set, with layerSetIdx being the layer set for the output layer set conforming to the Scalable Main or Scalable Main 10 profile, respectively, derived by invoking the sub-bitstream extraction process as specified in subclause Error: Reference source not found with tIdTarget equal to 7 and with TargetDecLayerIdList containing the nuh_layer_id values of the layer set with the index layerSetIdx, and TargetOutputLayerSetIdx containing the index to the list of output layer sets specified by the VPS of the target output layer set: 
a) Each layer in the TargetDecLayerIdList shall obey the General tier and level limits in A.4.1, and the Profile-specific level limits a), b), c), d), g), h), i), and j) for the Main and Main 10 profiles specified in A.4.2

b) For each k in the range of  0 to NumSubDpbs[ layerSetIdx] following applies: 
NPicSizeInSamplesY[ k ] =  SubDpbSpatRes[ layerSetIdx ][ k ][ 0 ] * SubDpbSpatRes[ layerSetIdx ][ k ][ 1 ] 
NPicSizeInSamplesY[ k ] shall be less than or equal to MaxLumaPs, where  MaxLumaPs is specified in Table A-1.

For each k in the range of  0 to NumSubDpbs[ layerSetIdx] the value of max_vps_dec_pic_buffering_minus1[ TargetOutputLayerSetIdx ][ k ][ HighestTid ] + 1 shall be less than or equal to MaxSharedDpbSize[ k ], which is derived as follows: 
if( NPicSizeInSamplesY[ k ]  <=  ( MaxLumaPs  >>  2 ) )
	MaxSharedDpbSize[ k ] = Min( 4 * maxDpbPicBuf, 16 )
else if( NPicSizeInSamplesY[ k ]  <=  ( MaxLumaPs  >>  1 ) )
	MaxSharedDpbSize[ k ]  = Min( 2 * maxDpbPicBuf, 16 )	(A-3)
else if( NPicSizeInSamplesY[ k ]  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxSharedDpbSize[ k ]  = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxSharedDpbSize[ k ]  = maxDpbPicBuf
MaxSharedDpbSize[ k ]  = Ceil ( MaxSharedDpbSize[ k ] * ( 1 + ( x * ( NumLayersShareSubDpb[ layerSetIdx ][ k ] - 1 ) ) ) ) 
where MaxLumaPs is specified in Error: Reference source not found and maxDpbPicBuf is equal to 6 and 0 <= x <= 1.

# Results for shared DPB capacity limits

Results for shared sub-DPB MaxSharedDpbSize from our proposal 2 are shown below for 1920 x 1080 for level 4.1.

| Number of Layers sharing sub-DPB | x=1 | x=0.5 |
| --- | --- | --- |
| 2 | 12 | 9 |
| 3 | 18 | 12 |
| 4 | 24 | 15 |
| 8 | 48 | 27 |

Table 1- MaxSharedDpbSize

Based on these results x=0.5 is recommended.

# Conclusion

In this document following is proposed:

- Proposal 1: Maximum DPB size level limit constraints for per-layer sub-DPB parameters max_vps_layer_dec_pic_buff_minus1[ ][ ][ ]

- Proposal 2: Maximum DPB size level limit constraints for shared sub-DPB parameters max_vps_dec_pic_buffering_minus1[ ][ ][ ] for SHVC scalable main and scalable main 10 and MV-HEVC stereo main profiles are proposed.

To help define the constraints on shared sub-DPB a change in the “F.7.4.3.1.3 DPB size semantics” to keep track of variables during derivation of the number of sub-DPBs and assignment of a sub-DPB to each layer included in the layer set is also proposed.

We propose to adopt the proposals in the draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, Y.-K. Wang, “High Efficiency Video Coding (HEVC) Scalable Extension Draft 5,” JCTVC-P1008, San Jose, January 2014.

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 7,” JCT3V-G1004, San Jose, January 2014.