| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0105r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0035r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On temporal enhancement layers and diagonal inter-layer prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution asserts that diagonal inter-layer prediction would be useful in the following use cases:

- An enhancement layer provides a temporal enhancement, possibly along with spatial or quality enhancement, relative to the base layer, where the picture rate ratio is non-dyadic, e.g. a 24-Hz base layer, a 30-Hz first enhancement layer, and a 120-Hz second enhancement layer.

- An SHVC-coded temporal enhancement layer is provided for an AVC or HEVC base layer.

- Interlaced-to-progressive scalability, when the base layer contains coded frames.

It is proposed to enable the use of diagonal inter-layer reference pictures as summarized in the following:

- A gating flag in SPS multilayer extension specifies if diagonal inter-layer prediction is enabled in the slice header level.

- A short_term_ref_pic_set( ) syntax structure can be included in the slice header for a direct reference layer and is referred to as the reference-layer short-term RPS. The reference-layer short-term RPS syntax structure specifies the pictures from the direct reference layer that are included in the initial reference picture list(s) of the current picture, but causes no change on the marking of the pictures.

- The decoding process for reference picture lists construction is modified to include reference pictures from the reference-layer short-term RPS syntax structure for the current picture.

The following changes were made in revision 1 of the contribution:

- The short_term_ref_pic_set( ) syntax structure is used as requested in the first review in the JCTVC#Q meeting (instead of a specific reference-layer short-term RPS syntax structure).

- The use of diagonal inter-layer prediction with hybrid codec scalability is enabled.

# Use cases

The contribution asserts that the support for the following use cases is desirable and therefore proposes changes to support the use cases.

- Non-dyadic temporal enhancement

Many displays support high picture rate displaying. Likewise, many cameras support high frame rate capturing and high-frame-rate content may also originate from screen capturing or other computer-generated sources. It is good to maintain support for legacy picture rates for cinema, PAL, and NTSC. Some interesting capture rates are:

- 75 fps, which enables temporal subsets of 15 fps, 25 fps, and 75 fps

- 120 fps, which enables temporal subsets of 15 fps, 24 fps, 30 fps, 60 fps, and 120 fps

- 150 fps, which enables temporal subsets of 15 fps, 25 fps, 30 fps, 50 fps, 75 fps, and 150 fps

The figures below illustrate possibilities that the encoder could take when the capture rate has been 120 fps and diagonal prediction is enabled. In these examples, the encoder generates a bitstream that can be decoded to produce 24, 30 or 120 fps output. In the first figure, it is assumed that the decoder producing 30 fps output can decode a bitstream at higher frame rate and hence the 24-fps base layer pictures are used for diagonal inter-layer prediction. In the second figure, “skipped” enhancement layer pictures are not coded to avoid unnecessary DPB usage and the sub-bitstream consisting of BL and EL1 at maximum TemporalId equal to 1 requires decoding capability at 30 fps.

Figure 1. Layers providing temporal scalability at 24, 30, and 120 Hz. Decoding of EL1 requires full decoding of BL.

Figure 2. Layers providing temporal scalability at 24, 30, and 120 Hz. Sub-bitstream consisting of BL and EL1 at maximum TemporalId equal to 1 requires decoding and provides output at 30 Hz picture rate.

- HEVC temporal enhancement layer for HEVC base layer

Figure 3. HEVC temporal enhancement layer for HEVC base layer.

An alternative could be to encode the temporal enhancement as a sub-layer within the base layer. However, that alternative might not be suitable for all situations, because:

- The base layer may have been coded earlier than the enhancement layer. By coding the temporal enhancement as its own layer, its bitstream partition specific HRD parameters can be derived independently of the base layer.

- The systems layer might disallow sub-layers to be included in separate logical channels (e.g. MPEG-2 PIDs) while they are likely to have the functionality to carry different layers in separate logical channels.

- The base layer bitstream may conform to a certain profile and level. If temporal enhancement were included in the same bitstream the base layer bitstream would have a higher level (and might no longer conform to the profile either). There can be legacy players, which are not able to extract and decode bitstreams containing only certain sub-layers.

In this use case the spatial resolution of the layers is the same.

- HEVC temporal enhancement layer for AVC base layer

Figure 4. HEVC temporal enhancement for AVC base layer.

In this use case it is desirable to let the enhancement-layer pictures to predict from temporally non-aligned base-layer pictures in order to avoid redundant coding of "skipped" enhancement layer pictures (that would co-exist in the same access units as the base-layer pictures) and the DPB memory usage for such "skipped" enhancement layer pictures.

In this use case the spatial resolution of the layers is the same.

In revision 1 of the contribution, the specification text was updated to support this use case. A decoded reference layer picture is provided by external means for each picture that is included in the reference-layer short-term reference picture set.

- Interlaced-to-progressive scalability with coded frames in the base layer

Figure 5. Interlaced-to-progressive scalability with coded frames in the base layer

In this use case, interlaced source content is coded as frames in the base layer, whereas the enhancement layer represents a progressive scan enhancement of the base layer. The base-layer frames are used twice as reference for inter-layer prediction, the first time using conventional inter-layer prediction, and the second time using diagonal inter-layer prediction. One of the predicted EL pictures corresponds to one field parity of the BL frame and the other one of these EL pictures corresponds to the opposite field parity of the BL frame.

- Interlaced source coding for non-interlace-aware players

HEVC version 1 includes support for interlaced source material e.g. through field_seq_flag of VUI and pic_struct of the picture timing SEI message. However, it is up to the display process to have the capability to display interlaced source material correctly. It is asserted that players may e.g. ignore the pic_struct syntax element of picture timing SEI messages and display fields as if they were frames – which would cause an unsatisfactory playback behavior. By separating fields of different parity to different layers, base-layer decoders would display fields of only one parity, which provides a stable and satisfactory displaying behavior.

Figure 6. Fields of different parity in different layers.

# Proposal

It is proposed to enable the use of diagonal inter-layer reference pictures as summarized in the following:

- A gating flag in SPS multilayer extension specifies if diagonal inter-layer prediction is enabled in the slice header level.

- A short_term_ref_pic_set( ) syntax structure can be included in the slice header for a direct reference layer and is referred to as the reference-layer short-term RPS. The reference-layer short-term RPS syntax structure specifies the pictures from the direct reference layer that are included in the initial reference picture list(s) of the current picture, but causes no change on the marking of the pictures.

- The decoding process for reference picture lists construction is modified to include reference pictures from the reference-layer short-term RPS syntax structure for the current picture.

Note that only one reference picture form the direct reference layer that causes resampling to be invoked can be included in the additional short-term RPS, as already constrained by the Scalable Main profile. If quality scalability is in use between the direct reference layer and the current layer, the number of pictures in the additional short-term RPS is not separately limited, but the constraint on NumPicTotalCurr (all pictures that may be used for reference) jointly limits the number of reference pictures from the current layer and the reference pictures included in the additional short-term RPS.

The following open issues have been identified on which JCT-VC/JCT-3V should decide if the diagonal inter-layer prediction mechanism is included in MV-HEVC/SHVC:

- It should be decided whether the number of diagonal prediction reference layers is limited to 1 or whether diagonal prediction is allowed from any number of direct reference layers. The specification text presents the former option, and also includes editor's notes commenting how to allow a number of diagonal prediction reference layers greater than 1.

- It should be determined whether the motion vectors are required to be equal to 0 for resampled diagonal inter-layer reference pictures. The specification text currently includes such constraints.

- It should be determined whether the diagonal inter-layer reference pictures are temporarily marked as "used for long-term reference" for the purpose of decoding the current picture, while their previous marking is restored after the decoding of the current picture. The reason for marking aligned inter-layer reference pictures as "used for long-term reference" is reportedly that otherwise POC differences used for motion vector scaling could be equal to 0. However, in diagonal inter-layer prediction a POC difference of zero cannot happen.

The detailed proposal is presented by change-marked text in the accompanying specification document. The change marks with user identity "Miska Hannuksela" indicate the changes relative to "JCTVC-P0141_JCT3V-G0141 spectext v2.docx".

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).