| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0109 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0037 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On TemporalId constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes that pictures with different TemporalId values are allowed in the same access unit. It is asserted that the following use cases or benefits are achieved with the proposal:

- It provides encoders flexibility in determining prediction hierarchies differently across layers, allowing some layers to be coded with a greater number of TemporalId values and frequent sub-layer up-switch points and some layers with a prediction hierarchy aiming at a better rate-distortion performance.

- It provides flexibility to encode layer trees of the same bitstream independently from each other.

# Introduction

JCTVC-Q0108/JCT3V-H0036 proposed to remove the constraints on aligning TSA and STSA pictures within an access unit due to the following reasons:

- The alignment constraints do not guarantee that sub-layer up-switching would be possible across all layers of the target output layer set, when a base-layer TSA or STSA picture at the desired switch-to sub-layer appears. In other words, layer-wise consideration of sub-layer up-switching is needed in any case.

- Without the alignment constraints, it would be possible to adapt the highest transmitted TemporalId of the highest layer separately from other layers and hence achieve finer-grain bitrate adjustment.

- Without the alignment constraints, reference pictures for inter prediction can be selected differently across layers, allowing some layers to be coded with more frequent sub-layer up-switch points and some layers with a larger number of reference pictures for inter prediction (aiming at better rate-distortion performance).

This contribution proposes that pictures with different TemporalId values are allowed in the same access unit. It is asserted that the following use cases or benefits are achieved with the proposal:

- It provides encoders flexibility in determining prediction hierarchies differently across layers, allowing some layers to be coded with a greater number of TemporalId values and frequent sub-layer up-switch points and some layers with a prediction hierarchy aiming at a better rate-distortion performance.

- It provides flexibility to encode layer trees of the same bitstream independently from each other. For example, the base layer and an auxiliary picture layer could be encoded by different encoders, and/or encoding of different layer trees could take place at different times. By allowing encoders to operate independently from each other, the encoders have flexibility in determining a prediction hierarchy and the number of TemporalId values used according to the input signal.

Figure 1 presents an example for the first motivation above. In this example, the base layer (BL) is coded with the prediction hierarchy of the random access common test conditions (which codes all pictures with TemporalId equal to 0). It is assumed that the prediction hierarchy of the random access common test conditions has been used to obtain a good rate-distortion performance for the base layer. The enhancement layer (EL) has four sub-layers and frequent TSA pictures, which provide the capability of dynamically selecting how many sub-layers are transmitted for the EL.

It is remarked that some prediction arrows from EL TID0 pictures are not included in the illustration (but can be concluded from the DPB dump). Likewise, the BL prediction arrows are excluded and can be concluded from the DPB dump. The DPB dump considers pictures marked as "used for reference" but does not consider pictures marked as "needed for output" (which might have already been marked "unused for reference").

Figure 1. BL using the prediction hierarchy of random access common conditions, EL coded with several TemporalId values.

In Figure 2, it is assumed that the sender adjusts the bitrate of the transmitted bitstream and chooses to transmit only three sub-layers of the EL. It is noted that if the use of alternative output layers is enabled, pictures would be output constantly at "full" picture rate in this example.

Figure 2. EL TIDs from 0 to 2, inclusive, transmitted.

# Background: analysis of specification text clauses assuming TemporalId being equal in all pictures of an access unit

Subclause 7.4.2.2 of HEVC version 1 states the following:

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of an access unit is the value of the TemporalId of the VCL NAL units of the access unit.

However, this constraint of TemporalId does not necessarily concern enhancement-layer pictures, because in HEVC version 1 bitstreams nuh_layer_id is required to be equal to 0.

TemporalId value associated with access units is used for the following purposes:

- Specifying requirements for the value of temporal_id_plus1 for certain non-VCL NAL units as follows:

- The following is specified in subclause 7.4.2.2:

…

– Otherwise, if nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId shall be equal to the TemporalId of the access unit containing the NAL unit.

– Otherwise, TemporalId shall be greater than or equal to the TemporalId of the access unit containing the NAL unit.

- The following is specified in subclause D.3.1:

When a non-nested SEI message has payloadType equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, or 134 (i.e., one of the SEI messages that have payloadType not equal to 0, 1, or 130, and that are allowed to be nested SEI messages), the SEI NAL unit containing the non-nested SEI message shall have TemporalId equal to the TemporalId of the access unit containing the SEI NAL unit.

- Specifying apperance requirements for PPS, SPS, and VPS NAL units in subclause 7.4.2.4.2:

…

A PPS RBSP, with that particular value of pps_pic_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId less than or equal to the TemporalId of the PPS NAL unit or provided through external means.

…

An SPS RBSP, with that particular value of sps_seq_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means.

…

A VPS RBSP, with that particular value of vps_video_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means.

# Overview of the proposal

This contribution proposes that pictures with different TemporalId values are allowed in the same access unit. In addition to liberalizing this constraint, the following changes are proposed.

The value of TemporalId of an access unit is specified to be the smallest TemporalId value of the coded pictures of the access unit. With that definition, the locations where TemporalId value associated with access units was referred to (see above) are proposed to be resolved as follows.

- Specifying requirements for the value of temporal_id_plus1 for certain non-VCL NAL units:

- TemporalId for access unit delimiter NAL unit is required to be equal to TemporalId of the first VCL NAL unit following, in decoding order, the access unit delimiter NAL unit. Consequently, if the first picture of the access unit is subject to sub-bitstream extraction, the access unit delimiter is also removed, which guarantees that sub-bitstream extractions does not result into dangling access unit delimiters remaining in the bitstream.

TemporalId for filler data NAL unit with nuh_layer_id equal to layerId is required to be the same as the TemporalId of the picture with nuh_layer_id equal to layerId in the same access unit.

- Depending on whether suffix or prefix SEI NAL unit is used and whether the next or previous VCL NAL unit is within the same access unit, a VCL NAL unit is associated with the SEI NAL unit. The TemporalId of the SEI NAL unit is required to be the same as the associated VCL NAL unit.

- No changes in requirements for PPS, SPS, and VPS NAL units are proposed.

Additionally, the following changes are proposed:

- It is proposed to add a VPS extension flag, tid_aligned_flag, to specify whether TemporalId values of pictures of the same access unit are the same or may differ.

- When tid_aligned_flag is equal to 0, the determination of possible reference layers no longer depends on the max_tid_il_ref_pics_plus1 value.

- The early marking of sub-layer non-reference pictures not needed for inter-layer prediction is modified to take the TemporalId of a reference layer picture into account, i.e. the early marking of a reference layer picture can take place only if the TemporalId of the reference layer has TemporalId equal to HighestTid.

The detailed proposal is presented by change-marked text in the accompanying specification document.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).