| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0115 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0038 |
| --- | --- |

| Title: | REXT/MV-HEVC/SHVC/3D-HEVC HLS: On indication of decoding process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The proposal can be summarized as follows:

- An indication of the applied decoding process is included in the VPS for each auxiliary picture layer and for each layer that is not included in any output layer set (excluding the additional independent output layer sets as proposed in JCTVC-Q0078/JCT3V-H0026).

  - independent_layer_dec_process_idc is provided for independent layers and can specify a decoding process of HEVC version 1 or range extensions.

  - predicted_layer_dec_process_idc is provided for predicted layers and can specify a decoding process of MV-HEVC or SHVC.

  - Both independent_layer_dec_process_idc and predicted_layer_dec_process_idc are ue(v)-coded and decoders shall ignore layers that have a value of independent_layer_dec_process_idc or predicted_layer_dec_process_idc that falls outside the currently allowed range.

  - independent_layer_dec_process_idc and predicted_layer_dec_process_idc also specify constraints on sps_extension_type_flag[ i ] values for the associated layer.

- The decoding process to be used for the current picture is selected as follows: If the current picture is within an auxiliary picture layer, the decoding process determined by the independent_layer_dec_process_idc or predicted_layer_dec_process_idc for the auxiliary picture layer is used. Otherwise, the decoding process determined by the profile of the target output layer set is used.

# Problem statement

JCTVC-Q0078/JCT3V-H0026 proposes to indicate additional layer sets, which may not include the base layer. A combination of profile, tier and level is associated with an output layer set of such an additional layer set. However, this approach still has the following shortcomings:

- A profile-tier-level triplet may allow multiple decoding processes used for different layers. For example, it is expected that the 3D-HEVC profile would allow non-base views coded either with MV-HEVC tools or 3D-HEVC tools. Hence, the profile-tier-level is not sufficient to conclude which decoding process is used for a particular layer when the profile allows more than one decoding process to be selected.

- Some layers may not conform to any profiles. For example, monochrome chroma sampling has not (at least not yet) been included in any profile. It would anyhow be desirable to indicate for example that a depth auxiliary layer conforms to the decoding process of HEVC version 1.

It is asserted that in many cases it is desirable to include auxiliary picture layers in layer sets (and corresponding output layer sets) conforming to a range extension, multiview or scalable profile, because HRD parameter signalled for the layer set then cover also the auxiliary picture layers and hence indicate the total buffering requirements for the case when decoding of both primary and auxiliary layers is carried out. However, the indicate profile for an output layer set that includes auxiliary picture layers should not constrain the profile which is used for coding the auxiliary picture layers. For example, it should be allowed to have a REXT-coded auxiliary picture layer for a version-1 coded base layer.

Furthermore, it is asserted that some layers may fall outside of any specified output layer sets. For semantic constraints of those layers as well as for decoding those layers, it should be able to determine which decoding process is used for those layers. Examples of cases where some layers may fall outside of any specified output layer sets include the following:

- There is no profile the constraints of which would be fulfilled when the layers are included in any output layer set. For example, a bitstream may include a greater number of views than allowed in any profile.

- It is allowed to exclude an auxiliary picture layer from all output layer sets.

It is asserted that the decoding process for each layer should be known in the session negotiation. This information lets the receiver to choose e.g. whether to receive a particular auxiliary picture layer or which one of the auxiliary picture layers offered as alternatives to choose. For example, a decoder may be able to process auxiliary picture layers decodable with version 1 decoding process only and hence desires not to receive e.g. REXT-coded auxiliary picture layers.

# Overview of the proposal

The proposal can be summarized as follows:

- An indication of the applied decoding process is included in the VPS for each auxiliary picture layer and for each layer that is not included in any output layer set (excluding the additional independent output layer sets as proposed in JCTVC-Q0078/JCT3V-H0026).

  - independent_layer_dec_process_idc is provided for independent layers and can specify a decoding process of HEVC version 1 or range extensions.

  - predicted_layer_dec_process_idc is provided for predicted layers and can specify a decoding process of MV-HEVC or SHVC.

  - Both independent_layer_dec_process_idc and predicted_layer_dec_process_idc are ue(v)-coded and decoders shall ignore layers that have a value of independent_layer_dec_process_idc or predicted_layer_dec_process_idc that falls outside the currently allowed range.

  - independent_layer_dec_process_idc and predicted_layer_dec_process_idc also specify constraints on sps_extension_type_flag[ i ] values for the associated layer.

- The decoding process to be used for the current picture is selected as follows: If the current picture is within an auxiliary picture layer, the decoding process determined by the independent_layer_dec_process_idc or predicted_layer_dec_process_idc for the auxiliary picture layer is used. Otherwise, the decoding process determined by the profile of the target output layer set is used.

The proposed specification text has been authored on top of "JCTVC-Q0078_JCT3V-H0026 spectext.docx" with user identity "(JCTVC-Q0115)". However, the proposed text is applicable also without the changes proposed in JCTVC-Q0078/JCT3V-H0026.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).