| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0040 |
| --- | --- |

| Title: | 3D-HEVC HLS: Inter-layer reference picture list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In order to prohibit the inclusion of decoded depth map pictures into the inter-layer reference picture sets of texture pictures and decoded texture pictures into the inter-layer reference picture sets of depth map pictures, a simple modification of decoding process for inter-layer reference picture set is proposed. Only when the value of DepthFlag of a picture PicX in the DPB belonging to the same access unit is identical to the value of DepthFlag of the current picture, the PicX can be included in RefPicSetInterLayer0 or RefPicSetInterLayer1 of the current picture. If the value of DepthFlag is not identical, the decoded picture cannot be included in the reference picture set.

# Proposal

Layers with depth map pictures having DepthFlag equal to 1 can be used as dependent layers of a layer with texture pictures for depth-oriented texture coding tools, such as depth oriented neighboring block disparity vector (DoNBDV), view synthesis prediction (VSP) and depth-based block partitioning (DBBP), because those depth-oriented texture coding tools refer to the decoded depth map data for texture coding.  The dependent depth map layers for texture layers will be indicated by direct_dependency_flag in VPS extension and the inter-layer RPS in slice headers. However, even if decoded depth map pictures are used for depth oriented coding tools, those depth map pictures shall not be included inter-layer reference picture set for inter-layer disparity compensation. If a decoded picture with DepthFlag equal to 1 is included in RefPicSetInterLayer0 or RefPicSetInterLayer1 for coding a picture with DepthFlag equal to 0, it will degrade the coding performance of inter-view prediction, because the decoded depth data will be almost not used as the predicted data of the texture data.

Reversely, the insertion of texture pictures into RefPicSetInterLayer0 or RefPicSetInterLayer1 of depth map pictures shall be prohibited. Texture-oriented depth coding tools, which are motion parameter inheritance (MPI) and Quad-tree limitation & prediction coding (QTLPC), refer to decoded texture pictures in DPB. Hence, the corresponding layers with the texture pictures will be indicated as the dependent layers of the layer with depth map pictures in VPS extension. However, those texture pictures shall be excluded from the reference picture set for disparity compensation. Note that the cross-referencing between texture pictures and depth map pictures for inter-view disparity compensation has not been considered and agreed for 3D-HEVC.

Simply, by adding a condition check in the decoding process for inter-layer reference picture set, the argued problem can be resolved. Only when the value of DepthFlag of a decoded picture in the DPB that is in the same access unit is identical to the value of DepthFlag of the current picture, the decoded picture can be inserted into RefPicSetInterLayer0 or RefPicSetInterLayer1 of the current picture. If the value of DepthFlag is not identical, the decoded picture cannot be included in the reference picture set.

# Proposed specification text

- I.8.1.2  Decoding process for inter-layer reference picture set

Outputs of this process are updated lists of inter-layer pictures RefPicSetInterLayer0 and RefPicSetInterLayer1 and the variables NumActiveRefLayerPics0 and NumActiveRefLayerPics1.

The lists RefPicSetInterLayer0 and RefPicSetInterLayer1 are first emptied, NumActiveRefLayerPics0 and NumActiveRefLayerPics1 are set equal to 0 and the following applies:

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	refPicSet0Flag = ( ViewId[ nuh_layer_id ]  <=  ViewId[ 0 ]   &&
								ViewId[ nuh_layer_id ]  <=  ViewId[ RefPicLayerId[ i ] ] )  | |
					( ViewId[ nuh_layer_id ]  >=  ViewId[ 0 ]  &&
								ViewId[ nuh_layer_id ]  >=  ViewId[ RefPicLayerId[ i ] ] ) ) 
	if(VpsDepthFlag[nuh_layer_id ] == VpsDepthFlag [ RefPicLayerId[ i ] ] ) {
   	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
										nuh_layer_id equal to RefPicLayerId[ i ] ) {
			if( refPicSet0Flag ) {
				RefPicSetInterLayer0[ NumActiveRefLayerPics0 ] = picX
				RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] is marked as "used for long-term reference"
			} else {
				RefPicSetInterLayer1[ NumActiveRefLayerPics1 ] = picX
				RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] is marked as "used for long-term reference"
			}
		} else {
			if( refPicSet0Flag )
				RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] = "no reference picture"
			else
				RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] = "no reference picture"
		}
   }
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer0 or RefPicSetInterLayer1.

There shall be no picture that has discardable_flag equal to 1 in RefPicSetInterLayer0 or RefPicSetInterLayer1.

If the current picture is a RADL picture, there shall be no entry in RefPicSetInterLayer0 or RefPicSetInterLayer1 that is a RASL picture.

NOTE – An access unit may contain both RASL and RADL pictures.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).