| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0142 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0044 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On extraction of independent non-base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Adarsh K. Ramasubramonian Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution asserts the presence of two problems related to the semantics of poc_lsb_not_present_flag[ i ] and the use of long-term reference pictures. The asserted problems can be summarized as follow:

- When poc_lsb_not_present_flag[ i ] is equal to 1 and the IDR picture in layer i is in the same access unit with a base layer picture that has slice_pic_order_cnt_lsb not equal to 0, the bitstream may become non-conformant as the derived picture order count (POC) for that IDR may be different from other pictures in the same access unit.

- When an independent non-base layer is extracted for input for HEVC decoder, if pictures in layer uses long-term reference picture (LTRP) and the original bitstream has non-cross layer aligned IDR, a problem may occur such that HEVC decoder cannot locate the correct LTRP.

The contribution also asserts that the proposed constraints to the semantics of poc_lsb_not_present_flag[ i ] and base_layer_parameter_set_compatibility_flag[ i ] would solve the problems above.

# Problem statements

## On poc_lsb_not_present_flag[ i ]

The syntax element poc_lsb_not_present_flag[ i ] allows not signalling of POC LSB information in slice segment headers of slices that belong to IDR picture of an independent non-base layer. This feature was asserted to ease the extraction of independent enhancement layers from a multi-layer bitstream such that an extracted layer can be decoded by an HEVC decoder without modifying the slice segment headers of the pictures in the layer and by only changing the nuh_layer_id in the NAL unit headers of these slice segments.

Currently, based on the syntax and semantics of poc_lsb_not_present_flag[ i ], the only explicit restriction on poc_lsb_not_present_flag[ i ] is that the value of poc_lsb_not_present_flag[ i ] can be set to 1 for a non-base layer only when the layer is an independent layer. However, such requirement alone does not demonstrate all the constraints required for the flag. For example, when IDR pictures are not cross-layer aligned and the poc_lsb_not_present_flag[ i ] for a non-base layer picture is set to 1, the bitstream may be non-conformant as the derived POC values of pictures may not be cross-layer aligned within an access unit; such a bitstream is illustrated in Figure 1. Since POC LSB of the IDR picture in the Layer 1 is not signalled, it is inferred to be equal to 0. Consequently, the POC of this Layer 1 IDR picture is derived to be 0 since the POC MSB for that picture is reset as indicated by poc_reset_idc equal to 1; the POC of the Layer 0 picture in the same AU as the Layer 1 IDR picture (in Figure 1) is derived to be 4. This would result in the POC of the pictures in the access unit to be unequal, which violates the constraint of cross-layer alignment of POC.

Figure 1: Problem of poc_lsb_not_present_flag[ i ] in non-cross layer aligned IDR scenario

## On long-term reference pictures

For extraction of independent non-base layers and enabling it to be decoded by HEVC version 1 decoders without changing slice segment headers, one should not only pay attention to whether or not POC LSB is signalled in the IDR picture in that layer (as explained in the earlier section) but also whether long-term reference pictures are used for prediction of pictures and whether pictures that reset both POC LSB and POC MSB are present in the layer. These additional two parameters are also needed to determine whether it is possible to extract the independent non-base layer without having to change slice segment headers of pictures in the layer. Note that when a particular independent layer is extracted, it is assumed that the corresponding non-VCL NAL units that are needed for decoding the independent layer are also included in the extracted bitstream.

Consider the scenario illustrated in Figure 2. Layer 1 of a two-layer bitstream is independent of the base layer Layer 0, and the IDR pictures in the bitstream are not cross-layer aligned and long-term reference pictures are used for coding of pictures of Layer 1 as illustrated. When a long-term reference picture is indicated in the RPS of a picture in Layer 1 that follows a POC resetting picture with poc_reset_idc equal to 2 (i.e. reset both POC LSB and POC MSB), and this Layer 1 is extracted, an HEVC version 1 decoder could interpret the bitstream as non-conformant if the slice segment header contents are not changed. The long-term reference picture is indicated in the RPS, say for example by using its POC LSB. An HEVC version 1 decoder would not perform POC resetting, and hence would not decrement the POC of any picture. Therefore, when Layer 1 is extracted and fed in to this decoder without changing slice segment header of pictures in the layer, the HEVC version 1 decoder may fail to find the indicated long-term reference picture since the indicated POC LSB in RPS may not match with any existing long-term reference pictures.

Figure 2: Problem of extraction of independent EL

# Proposals

To solve the above describes problems, the following modifications are proposed.

## Proposal 1

It is proposed that the value of poc_lsb_not_present_flag[ i ] be constrained such that it is not set equal to 1 when the bitstream contains access units that have a base layer picture that is a non-IDR picture with a non-zero value of POC LSB and the independent non-base layer picture (corresponding to the i-th layer in the VPS) is an IDR picture. Such restriction can be written as the following constraint that is added at the end of semantics of poc_lsb_not_present_flag[ i ].

It is a requirement of bitstream conformance that, within all CVSs that refer to the VPS, when there is at least one access unit that contains a non-IDR picture with nuh_layer_id equal to 0 and slice_pic_order_cnt_lsb greater than 0, and that contains an IDR picture with nuh_layer_id equal to layer_id_in_nuh[ i ], poc_lsb_not_present_flag[ i ] shall not be equal to 1.

## Proposal 2

The following changes are made to the semantics of base_layer_parameter_set_compatibility_flag[ i ] in VPS VUI. If these proposed changes are agreed, we further suggest to rename the syntax element from base_layer_parameter_set_compatibility_flag[ i ] to base_layer_compatibility_flag[ i ].

base_layer_parameter_set_compatibility_flag[ i ] equal to 1 specifies that the following constraints apply to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. base_layer_parameter_set_compatibility_flag[ i ] equal to 0 specifies that the following constraints may or may not apply to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. When not present, the value of base_layer_compatibility_flag[ i ] is inferred to be equal to 0.

–	Each coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS shall refer to a PPS with nuh_layer_id value equal to 0.

–	Each coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS shall refer to a SPS with nuh_layer_id value equal to 0.

–	The values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, of the active SPS for the layer with nuh_layer_id equal to layer_id_in_nuh[ i ] shall be the same as the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, of the vps_rep_format_idx[ i ]-th rep_format( ) syntax structure in the active VPS.

–	The value of poc_lsb_not_present_flag[ i ] shall be equal to 1.

–	When an access unit that contains a picture picA with nuh_layer_id equal to layer_id_in_nuh[ i ] referring to the VPS also contains a picture with nuh_layer_id equal to 0 and slice_pic_order_cnt_lsb greater than 0, the picture picA shall be a non-IDR picture.

–	When coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS that has non-zero value of either num_long_term_sps or num_long_term_pics, or both, and slice_pic_order_cnt_lsb greater than 0, poc_reset_idc shall not be equal to 2.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).