| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0154 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0045 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On picture flushing and DPB parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ye-Kui Wang Hendry 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes a change to the picture flushing operation and the inference of DPB-related parameters. The behaviour of NoOutputOfPriorPicsFlag for controlling output of pictures is proposed to be made applicable across all layers, and removal of pictures is decoupled from their output such that the removal is not controlled by NoOutputOfPriorPicsFlag. It is asserted that this design can avoid problems of exceeding the buffer size limit in certain bitstreams. The second change proposes to infer the value of DPB-related parameters in the VPS from the corresponding parameters in SPS for the 0-th output layer set.

# Flushing of pictures

Currently, the flag NoOutputOfPriorPicsFlag governs the flushing of pictures when decoding IRAP pictures with NoRaslOutputFlag equal to 1 and non-IRAP pictures contained in an access unit that contains a picture with nuh_layer_id equal to 0 and NoClrasOutputFlag equal to 1. In output-order conformant decoders, if NoOutputOfPriorPicsFlag is equal to 1, all decoded pictures in the current layer are removed from the DPB without output; otherwise (NoOutputOfPriorPicsFlag is equal to 0), all decoded pictures in the current layer that are marked as "needed for output" are output (by invoking the bumping process) and subsequently removed from the DPB. In output-timing conformant decoders, if NoOutputOfPriorPicsFlag is equal to 1, all decoded pictures in the current layer are removed from the DPB without output. The specified behaviours of NoOutputOfPriorPicsFlag for output-order and output-timing conformant decoders are given in the following subsections for convenience.

## Output timing conformance

- C.3.2	Removal of pictures from the DPB

When the current picture is not picture 0 in the current layer, the removal of pictures in the current layer, with nuh_layer_id equal to currLayerId, from the DPB before decoding of the current picture, i.e. picture n, but after parsing the slice header of the first slice of the current picture, happens instantaneously at the CPB removal time of the first decoding unit of the current picture and proceeds as follows:

–	The decoding process for RPS as specified in subclause Error: Reference source not found is invoked.

–	When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, or the base layer picture in the current access unit is an IRAP picture with NoRaslOutputFlag equal to 1 and NoClrasOutputFlag is equal to 1, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture with NoRaslOutputFlag equal to 1, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS for the current layer is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS that was active for the current layer when decoding the preceding picture in the current layer, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, if the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, the base layer picture in the current access unit is an IRAP picture with NoRaslOutputFlag equal to 1, and NoClrasOutputFlag is equal to 1), NoOutputOfPriorPicsFlag is set equal to 1.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD, such that when the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers that contain pictures with nuh_layer_id equal to currLayerId in the sub-DPB are emptied without output of the pictures they contain, and the DPB fullness is decremented by the number of picture storage buffers that were emptied.

–	When both of the following conditions are true for any pictures k in the DPB, all such pictures k in the DPB are removed from the DPB:

–	picture k is marked as "unused for reference"

–	picture k has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e. DpbOutputTime[ k ] is less than or equal to CpbRemovalTime( m )

–	For each picture that is removed from the DPB, the DPB fullness is decremented by one.

## Output order conformance

- C.5.2.1	Output and removal of pictures from the DPB

When the current picture is not picture 0 in the current layer, the output and removal of pictures in the current layer, with nuh_layer_id equal to currLayerId, from the DPB before the decoding of the current picture , i.e. picture n, but after parsing the slice header of the first slice of the current picture and before the invocation of the decoding process for picture order count, happens instantaneously when the first decoding unit of the current picture is removed from the CPB and proceeds as follows:

–	When the current picture is a POC resetting picture, all pictures in the DPB that do not belong to the current access unit and that are marked as "needed for output" are output, starting with pictures with the smallest value of PicOrderCntVal of all pictures excluding those in the current access unit in the DPB, in ascending order of the PicOrderCntVal values, and pictures with the same value of PicOrderCntVal are output in ascending order of the nuh_layer_id values. When a picture is output, it is cropped using the conformance cropping window specified in the active SPS for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

–	The decoding processes for picture order count and RPS are invoked. When decoding a CVS conforming to one or more of the profiles specified in Annex A using the decoding process specified in clauses 2 through 10, the decoding processes for picture order count and RPS that are invoked are as specified in subclauses Error: Reference source not foundand Error: Reference source not found, respectively. When decoding a CVS conforming to one or more of the profiles specified in Annex G or H using the decoding process specified in Annex F, and Annex G or H, the decoding processes for picture order count and RPS that are invoked are as specified in subclauses Error: Reference source not found and Error: Reference source not found, respectively.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, or the base layer picture in the current access unit is an IRAP picture with NoRaslOutputFlag equal to 1 and NoClrasOutputFlag is equal to 1, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture with NoRaslOutputFlag equal to 1, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS for the current layer is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS that was active for the current layer when decoding the preceding picture in the current layer, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, if the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, the base layer picture in the current access unit is an IRAP picture with NoRaslOutputFlag equal to 1, and NoClrasOutputFlag is equal to 1), NoOutputOfPriorPicsFlag is set equal to 1.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers that contain pictures with nuh_layer_id equal to currLayerId in the sub-DPB are emptied without output of the pictures they contain, and the sub-DPB fullness is decremented by the number of picture storage buffers that were emptied.

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers that contain pictures with nuh_layer_id equal to currLayerId in the sub-DPB are output by repeatedly invoking the "bumping" process specified in subclause Error: Reference source not found until all these pictures are marked as "not needed for output", all pictures that have nuh_layer_id equal to currLayerId in the sub-DPB are emptied, and the sub-DPB fullness is decremented by the number of picture storage buffers emptied.

–	Otherwise, all picture storage buffers that contain a picture in the current layer and that are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the sub-DPB fullness is decremented by one. When one or more of the following conditions are true, the "bumping" process specified in subclause Error: Reference source not found is invoked repeatedly until none of the following conditions are true:

- The number of access units that contain at least one decoded picture in the DPB marked as "needed for output" is greater than MaxNumReorderPics.

- MaxLatencyIncreasePlus1 is not equal to 0 and there is at least one access unit that contains at least one decoded picture in the DPB marked as "needed for output" for which the associated variable PicLatencyCount is greater than or equal to MaxLatencyPictures.

- The number of pictures in the sub-DPB is greater than or equal to MaxDecPicBufferingMinus1 + 1.

- The number of pictures in the current layer in the sub-DPB is greater than or equal to MaxLayerDecPicBuffMinus1 + 1.

## Problem Statement

This section provides examples that describe the problems associated with the current design of NoOutputOfPriorPicsFlag.

### Problem in case of spliced bitstream

Consider a bitstream as shown in Figure 1. Access unit auA is a splicing point, and only the base layer picture is present in auA; this base layer picture has NoClrasOutputFlag derived to be equal to 1. When decoding auA, all the pictures preceding auA are marked as "unused for reference", while the output and removal of decoded pictures are determined by NoOutputOfPriorPicsFlag.

Figure 1: Example of a bitstream where the current design of NoOutputOfPriorPicsFlag would not flush pictures of the EL of the DPB until much later. The red dotted lines indicate a splicing point.

If pictures in all the layers of the bitstream were present in auA, then the value of NoOutputOfPriorPicsFlag is derived as explained in subclauses C.3.2 and C.5.2.2, and all the pictures preceding auA may or may not be output (depending on the value of NoOutputOfPriorPicsFlag derived for that layer). In this particular case, if non-IRAP pictures were present in EL1 and EL2 in auA, all pictures in EL1 and EL2 preceding, in decoding order, auA would not be output and would be removed from the DPB when decoding the auA, as NoOutputOfPriorPicsFlag would be derived to be equal to 1 for the EL1 and EL2 pictures in auA. However, when the pictures are not present in the layers EL1 and EL2 in auA (as in the figure), all pictures preceding auA in EL1 and EL2 (represented in the boxes in the figure) are not removed from the DPB until decoding the next picture in that layer, i.e. when decoding the IDR pictures in auB and auC (shown in the figure), respectively.

One problem here is that these pictures are still marked as "needed for output", and may mess up picture output (for simplicity, assume that the output order is the same as the decoding order) when outputting BL pictures in AUs from auA to auB, exclusive, and even when outputting more pictures later unless the IDR pictures in auB and auC both have NoOutputOfPriorPicsFlag equal to 1.

Another problem is that this can cause DPB overflow, irrespective of whether a shared sub-DPB model is used or a layer-specific sub-DPB model is used, when the IDR picture in auB or auC has NoOutputOfPriorPicsFlag equal to 0. For the shared sub-DPB model, the DPB overflow problem is the same as described in JCTVC-Q0157/JCT3V-H0046, even when the IDR pictures in auB and auC both have NoOutputOfPriorPicsFlag equal to 1. For the layer-specific sub-DPB model, the problem can only when either of the IDR pictures in auB and auC has NoOutputOfPriorPicsFlag equal to 0. For example, when the IDR picture in auB has NoOutputOfPriorPicsFlag equal to 0, pictures in EL1 before auA can stay in the DPB waiting for output when and after decoding the IDR picture in auB, and the sub-DPB may become full before decoding a picture in EL1 and consequently cause DPB overflow as there is no space to store the being decoded picture.

### Access units with a new VPS activated or that are bitstream splicing points

When a new VPS is activated, it is possible that sub-DPB allocation of the DPB could change due to change in one or more of the following: the number of layers, the number of sub-DPBs, and the representation format of one or more layers. Currently, the activation of a new VPS, other than the VPS activated for the first picture in the bitstream, can occur in the following access units:

- All access units that include a picture for each nuh_layer_id value in TargetDecLayerIdList and for which each picture in the access unit is an IDR picture.

- All access units that include an IRAP picture with nuh_layer_id equal to 0 for which NoClrasOutputFlag is equal to 1.

In the remainder of this subsection, it is assumed that NoClrasOutputFlag is not set equal to 1. When the sub-DPB allocation of the DPB changes or the representation format of the layers change due to activation of a new VPS, it would be desirable to clear the contents of the DPB prior to decoding any picture that refers to the new VPS. If the contents are not cleared, it may lead to buffer overflow and in some cases, simply confusing DPB design; a non-exhaustive list of such situations is given below:

- Two layers sharing the same sub-DPB may now share different sub-DPBs according to the new VPS, and pictures with same nuh_layer_id may then be present in different sub-DPBs.

- Two layers in different sub-DPBs now share the same sub-DPB.

- The resolution of one or more layers changes in the new VPS.

In theory it may be sufficient to clear the contents of the DPB only when the sub-DPB allocation changes in the new VPS, instead of always doing it whenever a new VPS is activated. However, it was found to be very difficult and complicated to specify this, and the use case does not happen so often thus we concluded that it is better to have a simpler design.

When an AU is a bitstream splicing point, i.e. the BL picture is an IRAP picture with NoClrasOutputFlag equal to 1, regardless of whether a new VPS is activated, it makes sense to just clear the sub-DPB for all layers.

### Prior examples favouring layer-specific behaviour of NoOutputOfPriorPicsFlag

In JCTVC-O0266/JCT3V-F0090, it was argued that a layer-specific behaviour of NoOutputOfPriorPicsFlag was needed. The example used to justify these were the following (copied from the JCTVC-O0266/JCT3V-F0090):

- In a bitstream with two layers, when a different resolution is activated at the EL at a Layer Initialisation Picture (LIP) that is an IRAP picture and that does not belong to an IRAP AU (and the resolution of the base layer cannot change at this AU as the base layer picture in an non-IRAP AU cannot be an IRAP picture), layer specific flushing of pictures may be needed. Here only pictures from the EL but not from the BL need to be flushed. This is currently not possible for output order conformance.

- In a bitstream with two layers, let there be an access unit where the BL picture is an IDR picture and the EL picture is a non-IRAP picture, and the resolution of the BL picture is updated, while the resolution of the EL picture is not updated. In this case, flushing should be performed only for the pictures from the BL, while the EL pictures should not be flushed. Again this is currently not possible for output order conformance.

The first example above refers to the case where the picture resolution of the EL is updated without any change in the resolution of the base layer. This is done by specifying a different resolution in the active SPS of the enhancement layer. The reason to flush pictures when the resolution changes was to avoid buffer overflow in these cases, which could occur if the old resolution was smaller than the new resolution. There is, however, a restriction that the updated resolution has to be less than the initial resolution for that EL. Therefore, in this example, we need not flush the EL pictures, and this example actually does not justify layer-specific behaviour of NoOutputOfPriorPicsFlag.

In the second example, when the BL picture is IDR and EL picture is not IDR, it was argued that flushing of pictures was needed only for BL pictures. This example still is true, and it would not be wise to remove the pictures of enhancement layers; but there would be no problem if pictures from both the layers are output but the EL pictures are not removed, by decoupling removal of the EL pictures from their. Thus this example would not justify layer-specific behaviour  of NoOutputOfPriorPicsFlag, either.

## Proposal

The following are a few related observations:

- If pictures in certain AUs are available for output in all layers but only outputting for some of the layers while not for the other layers does not seem to generate a good user experience.

- The original intent for the introduction of NoOutputOfPriorPicsFlag since AVC was to enable avoiding DPB issues for bitstream splicing and representation format change.

- It seemed that output and removal of pictures based on NoOutputOfPriorPicsFlag are coupled in AVC and HEVC version 1, but actually they are not really coupled, as those pictures are actually marked as "unused for reference" by IRAP pictures with NoRaslOutputFlag, and once they are determined not to be output by NoOutputOfPriorPicsFlag equal to 1, they happen to become not needed to be stored in the DPB anymore.

Based on these observations and the discussions in Section 1, we believe that conceptually and architecturally it is cleaner to specify an AU-level behavior of NoOutputOfPriorPicsFlag for controlling of output, and decouple the removal of pictures from their output such that the removal is not controlled by NoOutputOfPriorPicsFlag.

The proposal described below is such an approach. In this approach, NoOutputOfPriorPicsFlag is used to control picture output in AU level, and a variable, named crossLayerBufferEmptyFlag, is used to control whether pictures in all the layers are to be removed from the DPB or only the base layer pictures are to be removed.

The proposed text changes are provided below, where additions are highlighted and removals are strikethrough.

- C.3.2	Removal of pictures from the DPB

When the current picture is not picture 0 in the current layer, the removal of pictures in the current layer, with nuh_layer_id equal to currLayerId, from the DPB before decoding of the current picture, i.e. picture n, but after parsing the slice header of the first slice of the current picture, happens instantaneously at the CPB removal time of the first decoding unit of the current picture and proceeds as follows:

–	The decoding process for RPS as specified in subclause Error: Reference source not found is invoked.

–	The variable crossLayerBufferEmptyFlag is derived as follows:

–	If a new VPS is activated by the current access unit or the current picture is an IRAP picture with nuh_layer_id equal to 0, NoRaslOutputFlag equal to 1 and NoClrasOutputFlag equal to 1, crossLayerBufferEmptyFlag is set to 1.

–	Otherwise, crossLayerBufferEmptyFlag is set to 0.

–	When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, or the base layer picture in the current access unit is an IRAP picture with NoRaslOutputFlag equal to 1 and NoClrasOutputFlag is equal to 1 and nuh_layer_id equal to 0, the following ordered steps are applied:

- The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture with NoRaslOutputFlag equal to 1, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS for the current layer is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS that was active for the current layer when decoding the preceding picture in the current layer, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, if the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, the base layer picture in the current access unit is an IRAP picture with NoRaslOutputFlag equal to 1, and NoClrasOutputFlag is equal to 1), NoOutputOfPriorPicsFlag is set equal to 1.

- The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:, such that

–	wWhen the value of NoOutputOfPriorPicsFlag is equal to 1 and the value of crossLayerBufferEmptyFlag is equal to 1, all non-empty picture storage buffers that contain pictures with nuh_layer_id equal to currLayerId in all the sub-DPBs are emptied without output of the pictures they contain, and the sub-DPB fullness of each sub-DPBs is set to 0decremented by the number of picture storage buffers that were emptied.

–	When the value of NoOutputOfPriorPicsFlag is equal to 1 and the value of crossLayerBufferEmptyFlag is equal to 0, all non-empty picture storage buffers that contain pictures with nuh_layer_id equal to 0 are emptied without output of the pictures they contain, and the sub-DPB fullness of the sub-DPB associated with nuh_layer_id equal to 0 is decremented by the number of picture storage buffers that were emptied.

–	When both of the following conditions are true for any pictures k in the DPB, all such pictures k in the DPB are removed from the DPB:

–	picture k is marked as "unused for reference"

–	picture k has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e. DpbOutputTime[ k ] is less than or equal to CpbRemovalTime( m )

–	For each picture that is removed from the DPB, the DPB fullness is decremented by one.

- C.5.2.1	Output and removal of pictures from the DPB

When the current picture is not picture 0 in the current layer, the output and removal of pictures in the current layer, with nuh_layer_id equal to currLayerId, from the DPB before the decoding of the current picture , i.e. picture n, but after parsing the slice header of the first slice of the current picture and before the invocation of the decoding process for picture order count, happens instantaneously when the first decoding unit of the current picture is removed from the CPB and proceeds as follows:

–	When the current picture is a POC resetting picture, all pictures in the DPB that do not belong to the current access unit and that are marked as "needed for output" are output, starting with pictures with the smallest value of PicOrderCntVal of all pictures excluding those in the current access unit in the DPB, in ascending order of the PicOrderCntVal values, and pictures with the same value of PicOrderCntVal are output in ascending order of the nuh_layer_id values. When a picture is output, it is cropped using the conformance cropping window specified in the active SPS for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

–	The decoding processes for picture order count and RPS are invoked. When decoding a CVS conforming to one or more of the profiles specified in Annex A using the decoding process specified in clauses 2 through 10, the decoding processes for picture order count and RPS that are invoked are as specified in subclauses Error: Reference source not foundand Error: Reference source not found, respectively. When decoding a CVS conforming to one or more of the profiles specified in Annex G or H using the decoding process specified in Annex F, and Annex G or H, the decoding processes for picture order count and RPS that are invoked are as specified in subclauses Error: Reference source not found and Error: Reference source not found, respectively.

–	The variable crossLayerBufferEmptyFlag is derived as follows:

–	If a new VPS is activated by the current access unit or the current picture is IRAP picture with nuh_layer_id equal to 0, NoRaslOutputFlag equal to 1 and NoClrasOutputFlag equal to 1, crossLayerBufferEmptyFlag is set to 1.

–	Otherwise, crossLayerBufferEmptyFlag is set to 0.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, or the base layer picture in the current access unit is an IRAP picture with NoRaslOutputFlag equal to 1 and NoClrasOutputFlag is equal to 1and nuh_layer_id equal to 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture with NoRaslOutputFlag equal to 1, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS for the current layer is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS that was active for the current layer when decoding the preceding picture in the current layer, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, if the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, the base layer picture in the current access unit is an IRAP picture with NoRaslOutputFlag equal to 1, and NoClrasOutputFlag is equal to 1), NoOutputOfPriorPicsFlag is set equal to 1.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 0, all non-empty picture storage buffers in all the sub-DPBs are output by repeatedly invoking the "bumping" process specified in subclause C.5.2.4 until all these pictures are marked as "not needed for output".

–	If crossLayerBufferEmptyFlagNoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers that contain pictures with nuh_layer_id equal to currLayerId in all the sub-DPBs are emptied without output of the pictures they contain, and the sub-DPB fullness of all the sub-DPBs is set to 0decremented by the number of picture storage buffers that were emptied.

–	Otherwise (crossLayerBufferEmptyFlagNoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers that contain pictures with nuh_layer_id equal to currLayerId in the sub-DPB are output by repeatedly invoking the "bumping" process specified in subclause Error: Reference source not found until all these pictures are marked as "not needed for output", all pictures that have nuh_layer_id equal to 0currLayerId in the sub-DPB containing layer nuh_layer_id equal to 0 are emptied, and the sub-DPB fullness of each sub-DPB is decremented by the number of picture storage buffers emptied in that sub-DPB.

–	Otherwise, all picture storage buffers that contain a picture in the current layer and that are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the sub-DPB fullness is decremented by one. When one or more of the following conditions are true, the "bumping" process specified in subclause Error: Reference source not found is invoked repeatedly until none of the following conditions are true:

- The number of access units that contain at least one decoded picture in the DPB marked as "needed for output" is greater than MaxNumReorderPics.

- MaxLatencyIncreasePlus1 is not equal to 0 and there is at least one access unit that contains at least one decoded picture in the DPB marked as "needed for output" for which the associated variable PicLatencyCount is greater than or equal to MaxLatencyPictures.

- The number of pictures in the sub-DPB is greater than or equal to MaxDecPicBufferingMinus1 + 1.

- The number of pictures in the current layer in the sub-DPB is greater than or equal to MaxLayerDecPicBuffMinus1 + 1.

# Inference of DPB-related parameters in VPS from SPS

## Problem

A multi-layer bitstream (such as an SHVC or an MV-HEVC bitstream) would be conforming to one or more profiles in Annex A as well as one or more profiles in Annex G or H. Some DPB parameters in the SHVC/MV-HEVC specifications are derived or inferred based on which Annex contains the profile that is used to decode this bitstream. Consider the following paragraphs in Annex C of the Specification.

The following paragraph is from subclause C.1.

For each bitstream conformance test, the CPB size (number of bits) for the bitstream-specific CPB operation and the BPB size for the bitstream-partition-specific CPB operation is CpbSize[ SchedSelIdx ] as specified in subclause E.3.3, where SchedSelIdx and the HRD parameters are specified above in this subclause. When a CVS conforming to one or more of the profiles specified in Annex A is decoded by applying the decoding process specified in clauses 2−10, the sub-DPB size (number of picture storage buffers) of the sub-DPB for the base layer is sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1, where sps_max_dec_pic_buffering_minus1[ HighestTid ] is from the active SPS for the base layer. When a CVS conforming to one or more of the profiles specified in Annex G or H is decoded by applying the decoding process specified in clauses 2−10, Annex F, and Annex G or H, the sub-DPB size of the sub-DPB for a layer with nuh_layer_id equal to currLayerId is max_vps_dec_pic_buffering_minus1[ TargetOutputLayerSetIdx ][ subDpbIdx ][ HighestTid ] + 1, where the variable subDpbIdx is equal to SubDpbAssigned[ LayerSetIdxForOutputLayerSet[ TargetOptLayerSetIdx ] ][ layerIdx ] and LayerSetLayerIdList[ lsIdx ][ layerIdx ] is equal to currLayerId.

The following paragraph is from subclause C.5.2.1.

When these processes are applied for a layer with nuh_layer_id equal to currLayerId, the variables MaxNumReorderPics, MaxLatencyIncreasePlus1, MaxLatencyPictures, and MaxDecPicBufferingMinus1 are derived as follows:

–	If a CVS conforming to one or more of the profiles specified in Annex G or H is decoded by applying the decoding process specified in clauses 2−10, Annex F, and Annex G or H, the following applies:

- MaxNumReorderPics is set equal to max_vps_num_reorder_pics[ TargetOutputLayerSetIdx ][ HighestTid ] of the active VPS.

- MaxLatencyIncreasePlus1 is set equal to the value of the syntax element max_vps_latency_increase_plus1[ TargetOutputLayerSetIdx ][ HighestTid ] of the active VPS.

- MaxLatencyPictures is set equal to VpsMaxLatencyPictures[ TargetOutputLayerSetIdx ][ HighestTid ] of the active VPS.

- MaxDecPicBufferingMinus1 is set equal to the value of the syntax element max_vps_dec_pic_buffering_minus1[ TargetOutputLayerSetIdx ][ subDpbIdx ][ HighestTid ] of the active VPS, where the value of the variable subDpbIdx is equal to SubDpbAssigned[ LayerSetIdxForOutputLayerSet[ TargetOptLayerSetIdx ] ][ layerIdx ] and LayerSetLayerIdList[ lsIdx ][ layerIdx ] is equal to currLayerId.

- MaxLayerDecPicBuffMinus1 is set equal to the value of the syntax element max_vps_layer_dec_pic_buff_minus1[ TargetOptLayerSetIdx ][ LayerIdxInVps[ currLayerId ] ][ HighestTid ] of the active VPS.

–	Otherwise (a CVS conforming to one or more of the profiles specified in Annex A is decoded by applying the decoding process specified in clauses 2−10), the following applies:

- MaxNumReorderPics is set equal to sps_max_num_reorder_pics[ HighestTid ] of the active SPS for the base layer.

- MaxLatencyIncreasePlus1 is set equal to sps_max_latency_increase_plus1[ HighestTid ] of the active SPS for the base layer.

- MaxLatencyPictures is set equal to SpsMaxLatencyPictures[ HighestTid ] of the active SPS for the base layer.

- MaxDecPicBufferingMinus1 and MaxLayerDecPicBuffMinus1 are both set equal to sps_max_dec_pic_buffering_minus1[ HighestTid ] of the active SPS for the base layer.

Any HEVC bitstream or any SHVC/MV-HEVC bitstream would be conformant to one or more profiles in Annex A and one or more profiles in Annex G or H. When the above subclauses are applied, the value of the DPB parameters is ambiguous or unavailable. Moreover, the DPB parameters signalled in the VPS extension are neither signalled nor inferred for the 0-th output layer set, where the layer set only comprises the base layer and only the base layer pictures are output.

## Proposal

It would be sufficient to infer the values of MaxLayerDecPicBuffMinus1, MaxNumReorderPics, MaxLatencyIncreasePlus1, MaxLatencyPictures, and MaxDecPicBufferingMinus1 for the 0-th output layer set as equal to the values signalled for the active SPS of the base layer. By only referring to these inferred values in Annex C, the ambiguity can be removed.

The semantics of syntax elements in dpb_size( ) are modified as follows.

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] plus 1 specifies the maximum required size of the k-th sub-DPB for the CVS in the i-th output layer set in units of picture storage buffers when HighestTid is equal to j. When j is greater than 0, max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] shall be greater than or equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j - 1 ]. When max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive, it is inferred to be equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j - 1]. The value of max_vps_dec_pic_buffering_minus1[ 0 ][ 0 ][ j ] is inferred to be equal to sps_max_dec_pic_buffering_minus1[ j ] of the active SPS of the base layer.

- max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] plus 1 specifies the maximum number of decoded pictures, of the k-th layer for the CVS in the i-th output layer set, that need to be stored in the DPB when HighestTid is equal to j. When j is greater than 0, max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] shall be greater than or equal to max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j - 1 ]. When max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] is not present for j in the range of 1 to vps_max_sub_layers_minus1 − 1, inclusive, it is inferred to be equal to max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j - 1]. The value of max_vps_layer_dec_pic_buffering_minus1[ 0 ][ 0 ][ j ] is inferred to be equal to sps_max_dec_pic_buffering_minus1[ j ] of the active SPS of the base layer.

- max_vps_num_reorder_pics[ i ][ j ] specifies, when HighestTid is equal to j, the maximum allowed number of access units containing a picture with PicOutputFlag equal to 1 that can precede any access unit auA that contains a picture with PicOutputFlag equal to 1 in the i-th output layer set in the CVS in decoding order and follow the access unit auA that contains a picture with PicOutputFlag equal to 1 in output order. When max_vps_num_reorder_pics[ i ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive, due to sub_layer_dpb_info_present_flag[ i ][ j ] being equal to 0, it is inferred to be equal to max_vps_num_reorder_pics[ i ][ j - 1]. The value of max_vps_num_reorder_pics[ 0 ][ j ] is inferred to be equal to sps_max_num_reorder_pics[ j ] of the active SPS of the base layer.

- max_vps_latency_increase_plus1[ i ][ j ] not equal to 0 is used to compute the value of VpsMaxLatencyPictures[ i ][ j ], which, when HighestTid is equal to j, specifies the maximum number of access units containing a picture with PicOutputFlag equal to 1 in the i-th output layer set that can precede any access unit auA that contains a picture with PicOutputFlag equal to 1 in the CVS in output order and follow the access unit auA that contains a picture with PicOutputFlag equal to 1 in decoding order. When max_vps_latency_increase_plus1[ i ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive, due to sub_layer_dpb_info_present_flag[ i ][ j ] being equal to 0, it is inferred to be equal to max_vps_latency_increase_plus1[ i ][ j - 1 ]. The value of max_vps_latency_increase_plus1[ 0 ][ j ] is inferred to be equal to sps_max_latency_increase_plus1[ j ] of the active SPS of the base layer.

- When max_vps_latency_increase_plus1[ i ][ j ] is not equal to 0, the value of VpsMaxLatencyPictures[ i ][ j ] is specified as follows:

VpsMaxLatencyPictures[ i ][ j ] = max_vps_num_reorder_pics[ i ][ j ] +
	max_vps_latency_increase_plus1[ i ][ j ] - 1	(F-1)

- When max_vps_latency_increase_plus1[ i ][ j ] is equal to 0, no corresponding limit is expressed. The value of max_vps_latency_increase_plus1[ i ][ j ] shall be in the range of 0 to 232 − 2, inclusive.

The following paragraph in subclause C.1 is modified as follows.

For each bitstream conformance test, the CPB size (number of bits) for the bitstream-specific CPB operation and the BPB size for the bitstream-partition-specific CPB operation is CpbSize[ SchedSelIdx ] as specified in subclause E.3.3, where SchedSelIdx and the HRD parameters are specified above in this subclause. When a CVS conforming to one or more of the profiles specified in Annex A is decoded by applying the decoding process specified in clauses 2−10, the sub-DPB size (number of picture storage buffers) of the sub-DPB for the base layer is sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1, where sps_max_dec_pic_buffering_minus1[ HighestTid ] is from the active SPS for the base layer. When a CVS conforming to one or more of the profiles specified in Annex G or H is decoded by applying the decoding process specified in clauses 2−10, Annex F, and Annex G or H, tThe sub-DPB size of the sub-DPB for a layer with nuh_layer_id equal to currLayerId is max_vps_dec_pic_buffering_minus1[ TargetOutputLayerSetIdx ][ subDpbIdx ][ HighestTid ] + 1, where the variable subDpbIdx is equal to SubDpbAssigned[ LayerSetIdxForOutputLayerSet[ TargetOptLayerSetIdx ] ][ layerIdx ] and LayerSetLayerIdList[ lsIdx ][ layerIdx ] is equal to currLayerId.

The following paragraph in subclause C.4 is modified as follows.

- For each current picture, after invocation of the process for removal of pictures from the sub-DPB as specified in subclause , the following two conditions shall be satisfied:

- tThe number of decoded pictures in the sub-DPB for the current layer, including all pictures n in the current layer that are marked as "used for reference", or that have PicOutputFlag equal to 1 and AuCpbRemovalTime[ n ] less than or equal to AuCpbRemovalTime[ currPic ], where currPic is the current picture, shall be less than or equal to sps_max_dec_pic_buffering_minus1[ HighestTid ] when currPicLayerId is equal to 0 or max_vps_dec_pic_buffering_minus1[ TargetOutputLayerSetIdx ][ subDpbIdx ][ HighestTid ] when currPicLayerId is greater than 0, where the variable subDpbIdx is equal to SubDpbAssigned[ LayerSetIdxForOutputLayerSet[ TargetOptLayerSetIdx ] ][ layerIdx ] and LayerSetLayerIdList[ lsIdx ][ layerIdx ] is equal to currPicLayerId.

- The number of decoded pictures in the sub-DPB for the current layer, including all pictures n in the current layer that are marked as "used for reference", or that have PicOutputFlag equal to 1 and AuCpbRemovalTime[ n ] less than AuCpbRemovalTime[ currPic ], where currPic is the current picture, shall be less than or equal to max_vps_layer_dec_pic_buff_minus1[ TargetOutputLayerSetIdx ][ layerIdx ][ HighestTid ], where the variable layerIdx is such that LayerSetLayerIdList[ LayerSetIdxForOutputLayerSet[ TargetOptLayerSetIdx ] ][ layerIdx ] is equal to currPicLayerId.

The following paragraphs in subclause C.5.2.1 are modified as follows.

When these processes are applied for a layer with nuh_layer_id equal to currLayerId, the variables MaxNumReorderPics, MaxLatencyIncreasePlus1, MaxLatencyPictures, and MaxDecPicBufferingMinus1 are derived as follows:

–	If a CVS conforming to one or more of the profiles specified in Annex G or H is decoded by applying the decoding process specified in clauses 2−10, Annex F, and Annex G or H, the following applies:

- MaxNumReorderPics is set equal to max_vps_num_reorder_pics[ TargetOutputLayerSetIdx ][ HighestTid ] of the active VPS.

- MaxLatencyIncreasePlus1 is set equal to the value of the syntax element max_vps_latency_increase_plus1[ TargetOutputLayerSetIdx ][ HighestTid ] of the active VPS.

- MaxLatencyPictures is set equal to VpsMaxLatencyPictures[ TargetOutputLayerSetIdx ][ HighestTid ] of the active VPS.

- MaxDecPicBufferingMinus1 is set equal to the value of the syntax element max_vps_dec_pic_buffering_minus1[ TargetOutputLayerSetIdx ][ subDpbIdx ][ HighestTid ] of the active VPS, where the value of the variable subDpbIdx is equal to SubDpbAssigned[ LayerSetIdxForOutputLayerSet[ TargetOptLayerSetIdx ] ][ layerIdx ] and LayerSetLayerIdList[ lsIdx ][ layerIdx ] is equal to currLayerId.

- MaxLayerDecPicBuffMinus1 is set equal to the value of the syntax element max_vps_layer_dec_pic_buff_minus1[ TargetOptLayerSetIdx ][ LayerIdxInVps[ currLayerId ] ][ HighestTid ] of the active VPS.

–	Otherwise (a CVS conforming to one or more of the profiles specified in Annex A is decoded by applying the decoding process specified in clauses 2−10), the following applies:

- MaxNumReorderPics is set equal to sps_max_num_reorder_pics[ HighestTid ] of the active SPS for the base layer.

- MaxLatencyIncreasePlus1 is set equal to sps_max_latency_increase_plus1[ HighestTid ] of the active SPS for the base layer.

- MaxLatencyPictures is set equal to SpsMaxLatencyPictures[ HighestTid ] of the active SPS for the base layer.

MaxDecPicBufferingMinus1 and MaxLayerDecPicBuffMinus1 are both set equal to sps_max_dec_pic_buffering_minus1[ HighestTid ] of the active SPS for the base layer.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).