| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0160r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0048r1 |
| --- | --- |

| Title: | SHVC/MV-HEVC HLS: On alternative output layer flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto Takeshi Tsukuba Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Author(s) or Contact(s): | Tomoyuki Yamamoto Takeshi Tsukuba Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to introduce a gating flag for alt_output_layer_flag. The proposed change is asserted to reduce parsing complexity as well as to assure the extensibility of the usage of the alternative output layer. In addition, it is proposed to restrict usage of the alternative output layer in Stereo Main profile.

In revision 1, proposed text is provided for restricting stereo main profile from using the alt output layer.

# Proposed text for stereo main profile restriction

An output layer set with index olsIdx conforming to the Stereo Main profile shall obey the following constraints:

Let numTextureViews be equal to the number of those layers that are not auxiliary picture layers and for which output_layer_flag[ olsIdx ][ j ] is equal to 1 for all values of j in the range of 0 to NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ olsIdx ] ] – 1, inclusive. numTextureViews shall be equal to 2.

NOTE – This restriction implies that alt_output_layer_flag[ olsIdx ] is equal to 0.

# Problem statement

The current design of alt_output_layer_flag has three problems.

- Complex conditional parsing of alt_output_layer_flag

- Limited extensibility of alt_output_layer_flag usage

- Bring unnecessary implementation complexity to Stereo Main profile where the functionality is not required

On the parsing complexity, alt_output_layer_flag is conditionally signalled. As shown in Table 1, variables OlsHighestOutputLayerId and NumOutputLayersInOutputLayerSet are used for the condition. However, those variables complicate the parsing because both variables need to be derived for each output layer set and they are derived just for the parsing purpose.

On the extensibility, alt_output_layer_flag is currently signalled only when the number of output layers in an output layer set is one, and at least one direct reference layer exists for the highest output layer of the output layer set. This condition could be too restrictive in terms of future usage. For example, alt_output_layer_flag might be used when two output layer exists in the output layer set that includes two independent layer dependency trees. Since syntax modification in later stage is difficult, it is better to relax this condition.

MV-HEVC, especially Stereo Main profile is unlikely to use alternative output layer since all layers are set to be output. Thus supporting this functionality in Stereo Main should bring about implementation and testing burden without valid benefit.

Table 1. VPS extension syntax (JCTVC-P1008_v4 / JCT3VG1004_v6)

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... | u(1) |
| num_add_output_layer_sets | ue(v) |
| NumOutputLayerSets = num_add_output_layer_sets + vps_num_layer_sets_minus1 + 1 |  |
| if( NumOutputLayerSets > 1 ) |  |
| default_target_output_layer_idc | u(2) |
| for( i = 1; i < NumOutputLayerSets; i++ ) { |  |
| if( i > vps_num_layer_sets_minus1 ) |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| if( i > vps_num_layer_sets_minus1 ¦ ¦ default_target_output_layer_idc = = 2 ) |  |
| for( j = 0; j < NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ]; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| profile_level_tier_idx[ i ] | u(v) |
| if( NumOutputLayersInOutputLayerSet[ i ] = = 1 && NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] > 0 ) |  |
| alt_output_layer_flag[ i ] | u(1) |
| } |  |
| ... |  |
| } |  |

# Proposal

It is proposed to simplify parsing by introducing a gating flag called alt_output_layer_present_flag. Additionally, it is proposed to restrict alt_output_layer_present_flag to 0 for MV-HEVC Stereo Main profile, where using alternative output layer do not make much sense.

Please also refer to JCTVC-P0132 [1], which also denies such use case.

Table 2. VPS extension syntax (Proposed)

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... | u(1) |
| num_add_output_layer_sets | ue(v) |
| NumOutputLayerSets = num_add_output_layer_sets + vps_num_layer_sets_minus1 + 1 |  |
| if( NumOutputLayerSets > 1 ) |  |
| default_target_output_layer_idc | u(2) |
| alt_output_layer_present_flag | u(1) |
| for( i = 1; i < NumOutputLayerSets; i++ ) { |  |
| if( i > vps_num_layer_sets_minus1 ) |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| if( i > vps_num_layer_sets_minus1 ¦ ¦ default_target_output_layer_idc = = 2 ) |  |
| for( j = 0; j < NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ]; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| profile_level_tier_idx[ i ] | u(v) |
| if(alt_output_layer_present_flag && NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ] > 1) |  |
| alt_output_layer_flag[ i ] | u(1) |
| } |  |
| ... |  |
| } |  |

alt_output_layer_present_flag equal to 1 specifies that the syntax elements alt_output_layer_flag may be present. alt_output_layer_present_flag equal to 0 specifies that the syntax elements alt_output_layer_flag are not present.

alt_output_layer_flag[ i ] equal to 0 specifies that an alternative output layer is not used for any output layer in the i-th output layer set. alt_output_layer_flag[ i ] equal to 1 specifies that an alternative output layer may be used for the output layer in the i-th output layer set.

–	If NumOutputLayersInOutputLayerSet[ i ] is equal to 1 and NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] is greater than 0, the variable AltOptLayerFlag[ i ] is set equal to alt_output_layer_flag[ i ].

–	Otherwise, the variable AltOptLayerFlag[ i ] is set equal to 0.

Note: The condition "NumOutputLayersInOutputLayerSet[ i ]  = =  1 &&  NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] > 0" has been already included in semantics (derivation process of AltOptLayerFlag as shown above). The proponent considers the restriction in semantics brings better future extensibility compared to the restiction in syntax.

MV-HEVC Profile restriction

Bitstreams conforming to the Stereo Main profile shall obey the following constraints:

- VPSs shall have alt_output_layer_present_flag equal to 0 only

…

There is related discussion in JCTVC-P0132 [1].

Quote from JCTVC-P0132:

For multi-view case, it would not be good to replace a picture in a target output layer with a picture from another layer, since a different layer belongs to different views. For example, in a three view case wherein decoded pictures may be used for view synthesis of the views in the intermediate viewing positions. Outputting a different view than the target output view might need to completely messed-up view synthesis operation. In this case, it is much better just not to output any alternative layer.

In case of two views are needed for output, while a third view is used to replace a second view of these two when alt_output_layer_flag is equal to 1, the stereoscopic viewing experience will not be good, since the third view will in a different viewing position, thus corresponds to completely different 3D perception. The unexpected 3D perception might either be compensated by the render or viewers adjusted manually by the viewer with a specific UI. In any case, the user experience is unacceptable.

# Conclusion

A gating flag is proposed to gate alt_output_layer_flag in order to reduce parsing complexity as well as to assure the extensibility of the usage of the alternative output layer. It is recommended to adopt the gating flag in SHVC and MV-HEVC. It is also recommended to restrict alternative output layer in MV-HEVC Stereo Main Profile.

# References

[1] A. K. Ramasubramonian, Y.-K. Wang, Hendry, Y. Chen, “MV-HEVC/SHVC HLS: On alt_output_layer_flag”, JCTVC-P0132, JCT-VC 16th Meeting: San José, US, 9–17 Jan. 2014

# Patent rights declaration

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).