| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0165r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0053r2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Clean up for output layer set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba * Tomoyuki Yamamoto * Tomohiro Ikai * Sachin Deshpande ** *1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN **5750 Pacific Rim Blvd, Camas, WA 98607, USA | Tel: Email: | +81-43-299-8526 tsukuba.takeshi@sharp.co.jp |
| Source: | SHARP Corporation and Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution presents several proposals regarding output layer set as follows:

- Prohibits 1) the case no output layer exists in an output layer set, and 2) duplicated output layer sets

- Clean up signaling condition related to num_add_output_layer_sets and default_target_output_layer_set.

- Add a flag, ptl_dpb_info_present_flag, indicating whether or not to explicitly signal profile_tier_level_idx and DPB parameters for additional output layer sets

In revision 1, the proposed text for proposal 1 is updated. In addition, bit-estimation analysis on proposal 3 is added. According to the analysis, it is asserted that proposal 3 can reduce bits with negligible overhead.

In revision 2, the proposed texts for proposal 3 is updated.

# Introduction

Clean up for output layer set definition

Output layer can be explicitly defined by output_layer_flag [1][2][3]. Thus the problematic cases exist as illustrated in Fig.1:

- Output layer set without any output layer.

- Duplicated output layer definition on the same layer set.

Figure 1: Example of problematic case

There may exist use-cases for no output layer and duplicated output layer as follows:

- No output layer express that external means should decide output layer.

- Duplication can provide a capability that shows two or more profile level definition can be indicated to one output layer set or indicates DPB differences depends on AltOptLayerFlag.

In reality, the first usage arise conformance problem. Specifically if we do allow external means should decide, the DPB or such parameters should support the case all output_layer_flag is equal to 1. However, with no output case, the signaled DPB only confirms no output case.

On the second point, we should care about AltOptLayerFlag in terms of duplication of output layer flag. But other than that, it is beneficial to avoid duplicate output layers since duplicate output layer sets could increase MANE operation which repetitively calculate required resource and the actual resource and choose the optimal target output layer set.

For example, if we allow the duplication, at maximum 1024 output layer sets with same output layers, which may have same AltOptLayerFlag and same profile/level/tier, could be defined as additional output layer sets in VPS.

Clean up for output layer derivation

The second problem is related to syntax redundancy. Currently num_add_output_layer_sets is always signaled and default_target_output_layer_idc is signaled on the condition of NumOutputLayerSets > 1. However when vps_num_layer_sets_minus1 is equal to 0 (base layer only), output layer must be base layer itself unless we allow no output layer case. In this case, additional output layer set syntax doesn’t make sense. So num_add_output_layer_set, should be signalled on the condition of vps_num_layer_sets_minus1 is greater than 0.

In addition, if vps_num_layer_sets_minus1 is greater than 0, the current condition of NumOutputLayerSets > 1 is always true. Consequently, the overall signaling can be simplified.

Syntax redundancy on Profile/Tier/Level and DPB parameters for Additional Output layer sets

In the current specification [2][3], the decoder is set to decode all layers of the target layer set regardless of output_layer_flag. If all target layers are necessary to be decoded, all output layer sets referring to the same layer set should have the same DPB size and probably the same profile/level. Therefore, signalling DPB parameters and profile_level_tier_idx of output layer sets referring to the same layer set is redundant.

# Proposal

Note: Text changes is based on JCTVC-P1008_v3.

## Proposal 1

Proposed restriction on output layer sets without no output layer are highlighted in yellow.

Proposed restriction on duplicated output layer sets are highlighted in cyan.

- F.7.4.3.1.1 Video parameter set extension semantics

output_layer_flag[ i ][ j ] equal to 1 specifies that the j-th layer in the i-th output layer set is an output layer. output_layer_flag[ i ][ j ] equal to 0 specifies that the j-th layer in the i-th output layer set is not an output layer.

…

It is requirement of bitstream comformance that the i-th output layer set shall have one or more output layers.

Or

The sum of OutputLayerFlag[ i ][ j ] for j in the range of 0 to NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ] − 1, inclusive shall be greater than 0.

The variable NumOutputLayersInOutputLayerSet[ i ] is derived as follows:

NumOutputLayersInOutputLayerSet[ i ] = 0
for( j = 0 ; j < NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ]; j++) { 
	NumOutputLayersInOutputLayerSet[ i ] += OutputLayerFlag[ i ][ j ]
	if( OutputLayerFlag[ i ][ j ] )
		OlsHighestOutputLayerId[ i ] = LayerSetLayerIdList[ LayerSetIdxForOutputLayerSet[ i ] ][ j ]

When LayerSetIdxForOutputLayerSet[ iA ] is equal to LayerSetIdxForOutputLayerSet[ iB ] for any iA and iB in the range of 0 to NumOutputLayerSets-1, inclusive, with iA not equal to iB, at least one of the following conditions shall be satisfied:

- the value of OutputLayerFlag[ iA ][ n ] shall not be equal to OutputLayerFlag[ iB ][ n ] for at least one value of n in the range of 0 to NumLayerInIdList[ LayerSetIdxForOutputLayerSet[ iA ] ] – 1, inclusive.

- the value of AltOuputLayerFlag[ iA ] shall not be equal to AltOutputLayerFlag[ iB ].

- the value of profile_level_tier_idx[ iA ] shall not be equal to profile_level_tier_idx[ iB ].

Note: HEVC version 1 spec already has similar restriction for layer set definition:

When numLayersInIdList[ iA ] is equal to numLayersInIdList[ iB ] for any iA and iB in the range of 0 to vps_num_layer_sets_minus1, inclusive, with iA not equal to iB, the value of layerSetLayerIdList[ iA ][ n ] shall not be equal to layerSetLayerIdList[ iB ][ n ] for at least one value of n in the range of 0 to numLayersInIdList[ iA ], inclusive.

## Proposal 2

The proposed change is to signal num_add_output_layer_sets and default_target_output_layer_idc when vps_num_layer_sets_minus1 is greater than 0. Proposed changes are highlighted in green.

F.7.3.2.1.1 Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| if (vps_num_layer_sets_minus1 > 0) |  |
| { |  |
| num_add_output_layer_sets | ue(v) |
| if( NumOutputLayerSets > 1 ) |  |
| default_target_output_layer_idc | u(2) |
| } |  |
| NumOutputLayerSets = num_add_output_layer_sets + vps_num_layer_sets_minus1 + 1 |  |

num_add_output_layer_sets specifies the number of output layer sets in addition to the first vps_num_layer_sets_minus1 + 1 output layer sets specified by the VPS. The value of num_add_output_layer_sets shall be in the range of 0 to 1023, inclusive. When not present, it is inferred to 0.

## Proposal 3

The proposed change is to add a flag ptl_dpb_info_present_flag[i] indicating whether or not to explicitly signal profile_tier_level_idx and DPB parameters for all additional i-th output layer sets (i>vps_num_layer_sets_minus1), which is highlighted in cyan.

We also suggest to add the additional condition on (AltOptLayerFlag [i] != AltOptLayerFlag [output_layer_set_idx_minus1[ i ]]) for not signaling ptl_dpb_info_present_flag to guarantee the explicit signaling (or to give up bit-saving) when additional output layer sets the same AltOptLayerFlag values and the same output layers (if proposal 1’s restriction is adopt).

- F.7.3.2.1.1 Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 1; i < NumOutputLayerSets; i++ ) { |  |
| if( i > vps_num_layer_sets_minus1 ) |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| if( i > vps_num_layer_sets_minus1 ¦ ¦ default_target_output_layer_idc = = 2 ) |  |
| for( j = 0; j < NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ]; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| if( NumOutputLayersInOutputLayerSet[ i ] = = 1 && NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] > 0 ) |  |
| alt_output_layer_flag[ i ] | u(1) |
| if (i >vps_num_layer_sets_minus1 ) |  |
| ptl_dpb_info_present_flag[ i ] | u(1) |
| if ( ptl_dpb_info_present_flag[ i ] ) |  |
| profile_level_tier_idx[ i ] | u(v) |
| if( NumOutputLayersInOutputLayerSet[ i ] = = 1 && NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] > 0 ) |  |
| alt_output_layer_flag[ i ] | u(1) |
| } |  |
| ... |  |
| dpb_size() |  |
| ... |  |
| } |  |

ptl_dpb_info_present_flag[ i ] equal to 1 specifies that the syntax elements profile_level_tier_level_idx[ i ], sub_layer_flag_info_present_flag[ i ], max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ], max_vps_num_reorder_pics[ i ][ j ] max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are present for i-th output layer set.

ptl_dpb_info_present_flag[ i ] equal to 0 specifies that the syntax elements profile_level_tier_level_idx[ i ], sub_layer_flag_info_present_flag[ i ], max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ], max_vps_num_reorder_pics[ i ][ j ] max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are not present for i-th output layer set.

When not present, the value of ptl_dpb_info_present_flag[ i ] is inferred to be equal to 1.

profile_level_tier_idx[ i ] specifies the index, into the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to i-th output layer set. The length of the profile_level_tier_idx[ i ] syntax element is Ceil( Log2( vps_num_profile_tier_level_minus1 + 1 ) ) bits. The value of profile_level_tier_idx[ 0 ] is inferred to be equal to 0. The value of profile_level_tier_idx[ i ] shall be in the range of 0 to vps_num_profile_tier_level_minus1, inclusive. For the i-th output layer set, when ptl_dpb_info_present_flag[ i ] is equal to 0, profile_level_tier_idx[ i ] is inferred to be equal to profile_level_tier_idx[ output_layer_set_idx_minus1[ i ] + 1 ].

| dpb_size( ) { |  |
| --- | --- |
| for( i = 1; i < NumOutputLayerSets; i++ ) { |  |
| if( ptl_dpb_info_present_flag[ i ]){ |  |
| sub_layer_flag_info_present_flag[ i ] | u(1) |
| for( j = 0; j <= MaxSubLayersInLayerSetMinus1[ i ]; j++ ) { |  |
| if( j > 0 && sub_layer_flag_info_present_flag[ i ] ) |  |
| sub_layer_dpb_info_present_flag[ i ][ j ] | u(1) |
| if( sub_layer_dpb_info_present_flag[ i ][ j ] ) { |  |
| for( k = 0; k < NumSubDpbs[ LayerSetIdxForOutputLayerSet[ i ] ]; k++ ) |  |
| max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] | ue(v) |
| max_vps_num_reorder_pics[ i ][ j ] | ue(v) |
| if( NumSubDpbs[ LayerSetIdxForOutputLayerSet[ i ] ] != NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ] ) |  |
| for( k = 0; k < NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ]; k++ ) |  |
| max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] | ue(v) |
| max_vps_latency_increase_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

- F.7.4.3.1.3 DPB size semantics

The variable OlsIdx[i] is set to be equal to (output_layer_set_idx_minus1[ i ]+1) for i in the range of 0 to NumOutputLayerSets – 1, inclusive.

- sub_layer_flag_info_present_flag[ i ] equal to 1 specifies that sub_layer_dpb_info_present_flag[ i ][ j ] is present for i in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive. sub_layer_flag_info_present_flag[ i ] equal to 0 specifies that, for each value of j greater than 0, sub_layer_dpb_info_present_flag[ i ][ j ] is not present and the value is inferred to be equal to 0.

- sub_layer_dpb_info_present_flag[ i ][ j ] equal to 1 specifies that  max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] is present for k in the range of 0 to NumSubDpbs[ LayerSetIdxForOutputLayerSet[ i ] ] − 1, inclusive, for the j-th sub-layer, and max_vps_num_reorder_pics[ i ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are present for the j-th sub-layer. sub_layer_dpb_info_present_flag[ i ][ j ] equal to 0 specifies that the values of max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] are equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j − 1 ] for k in the range of 0 to NumSubDpbs[ LayerSetIdxForOutputLayerSet[ i ] ] − 1, inclusive, and that the values max_vps_num_reorder_pics[ i ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are set equal to max_vps_num_reorder_pics[ i ][ j - 1 ] and max_vps_latency_increase_plus1[ i ][ j - 1 ], respectively. The value of sub_layer_dpb_info_present_flag[ i ][ 0 ] for any possible value of i is inferred to be equal to 1.

- When not present, the value of sub_layer_dpb_info_present_flag[ i ][ j ] for j greater than 0 and any possible value of i, is inferred to be equal to be equal to 0.

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] plus 1 specifies the maximum required size of the k-th sub-DPB for the CVS in the i-th output layer set in units of picture storage buffers when HighestTid is equal to j.

When j is greater than 0, max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] shall be greater than or equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j - 1 ].

When ptl_dpb_info_present_flag[ i ] is equal to 1 and max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive, it is inferred to be equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j - 1].

When ptl_dpb_info_present_flag[ i ] is equal to 0 and max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive, max_vps_dec_pic_buffering_minus1[ i][ k][ j ] is inferred to be equal to the value of max_vps_dec_pic_buffering_minus1[ OlsIdx[ i ] ][ k][ j ].

- max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] plus 1 specifies the maximum number of decoded pictures, of the k-th layer for the CVS in the i-th output layer set, that need to be stored in the DPB when HighestTid is equal to j.

- When j is greater than 0, max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] shall be greater than or equal to max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j - 1 ].

- When ptl_dpb_info_present_flag[ i ] is equal to 1 and max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] is not present for j in the range of 1 to vps_max_sub_layers_minus1 − 1, inclusive, it is inferred to be equal to max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j - 1].

- When ptl_dpb_info_present_flag[ i ] is equal to 0 and max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] is not present for j in the range of 1 to vps_max_sub_layers_minus1 − 1, inclusive, max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] is inferred to be equal to the value of max_vps_layer_dec_pic_buff_minus1 [ OlsIdx[ i ] ][ k][ j ].

- max_vps_num_reorder_pics[ i ][ j ] specifies, when HighestTid is equal to j, the maximum allowed number of access units containing a picture with PicOutputFlag equal to 1 that can precede any access unit auA that contains a picture with PicOutputFlag equal to 1 in the i-th output layer set in the CVS in decoding order and follow the access unit auA that contains a picture with PicOutputFlag equal to 1 in output order.

- When ptl_dpb_info_present_flag[ i ] is equal to 1 and max_vps_num_reorder_pics[ i ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive, due to sub_layer_dpb_info_present_flag[ i ][ j ] being equal to 0, it is inferred to be equal to max_vps_num_reorder_pics[ i ][ j - 1].

- When ptl_dpb_info_present_flag[ i ] is equal to 0 and max_vps_num_reorder_pics[ i ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive, max_vps_num_reorder_pics[ i][ j ] is inferred to be equal to the value of max_vps_num_reorder_pics [ OlsIdx[ i ] ] [ j ].

- max_vps_latency_increase_plus1[ i ][ j ] not equal to 0 is used to compute the value of VpsMaxLatencyPictures[ i ][ j ], which, when HighestTid is equal to j, specifies the maximum number of access units containing a picture with PicOutputFlag equal to 1 in the i-th output layer set that can precede any access unit auA that contains a picture with PicOutputFlag equal to 1 in the CVS in output order and follow the access unit auA that contains a picture with PicOutputFlag equal to 1 in decoding order.

- When ptl_dpb_info_present_flag[ i ] is equal to 1 and max_vps_latency_increase_plus1[ i ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive, due to sub_layer_dpb_info_present_flag[ i ][ j ] being equal to 0, it is inferred to be equal to max_vps_latency_increase_plus1[ i ][ j - 1 ].

- When ptl_dpb_info_present_flag[ i ] is equal to 0 and max_vps_latency_increase_plus1[ i ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive, max_vps_latency_increase_plus1 [ i][ j ] is inferred to be equal to the value of max_vps_latency_increase_plus1 [ OlsIdx[ i ] ] [ j ].

- When max_vps_latency_increase_plus1[ i ][ j ] is not equal to 0, the value of VpsMaxLatencyPictures[ i ][ j ] is specified as follows:

VpsMaxLatencyPictures[ i ][ j ] = max_vps_num_reorder_pics[ i ][ j ] +
	max_vps_latency_increase_plus1[ i ][ j ] - 1	(F-1)

When max_vps_latency_increase_plus1[ i ][ j ] is equal to 0, no corresponding limit is expressed. The value of max_vps_latency_increase_plus1[ i ][ j ] shall be in the range of 0 to 232 − 2, inclusive.

# Bit amount analysis on Proposal 3

Analysis was carried out to evaluate bit-saving and bit-overhead introduced by the proposed change under the following use cases:

Use-case#1: Two output layer sets (OLS#1, OLS#2) associated with the same layer set (LS#1), which have the same output layers (OptLayer) and different AltOptLayerFlag (Alt) values:

- LS#1 = { Layer0, Layer1},

- OLS#1 = { LS#1, OptLayer={Layer1}, Alt=1},

- OLS#2 = { LS#1, OptLayer={Layer1}, Alt=0}

-OLS#2 is an additional output layer set against OLS#1.

Use-case#2: Three output layer sets (OLS#1, OLS#2, OLS#3) associated with same layer set (LS#1), which have different output layers and the same AltOptLayerFlag(Alt) values:

- LS#1 = { L#0, L#1, L#2},

- OLS#1 = { LS#1, OptLayer={Layer0, Layer1,.Layer2}, Alt=0} ,

- OLS#2 = { LS#1, OptLayer={Layer0, Layer1}, Alt=0} ,
		- OLS#3 = { LS#1, OptLayer={Layer0, Layer2}, Alt=0}

- OLS#2 and OLS#3 are additional output layer sets against OLS#1.

Bit-saving is estimated on the condition that profile and DPB info can be inferred and omitted (ptl_dpb_info_present_flag[i]=0; inferring profile_level_tier_idx/DPB size for additional output layer sets). Bit-overhead is estimated on the condition that all profile and DPB info are signalled (ptl_dpb_info_present_flag[i]=1; explicitly signaling profile_level_tier_idx/DPB size for additional output layer sets).

Table 1 and Table 2 shows the required bits on signaling profile_level_tier_idx, and DPB size syntax and ptl_dpb_info_present_flag for Use-case#1 and Use-case#2, in which bit-saving case and bit-overhead case are included, respectively.

Details are included in the attached excel files.

According to the bit-estimation, proposal 3 reduces bit-amounts by about 50-66% in the bit-saving scenario for two use-cases. In the bit-overhead case, proposal 3 needs about 0.5-0.7% extra bits.

Table 1: Results of bit-estimation for Use-case#1

| Use-case#1 (bit-saving case) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required bit-size for OLS#1 to OLS#2 |  | Ref. (SHVC/MV-HEVC) |  | Proposal |  | bit-saving |
|  | bits/OLS | Counts | Total size (bits) | Counts | Total size (bits) |  |
| Required bit-size on ptl_dpb_info_present_flag | 1 | 0 | 0 | 1 | 1 | N/A |
| Required bit-size on profile_level_tier_idx | 1 | 2 | 2 | 1 | 1 | 50.00% |
| Required bit-size on DPB Size syntax element | 64 | 2 | 128 | 1 | 64 | 50.00% |
|  |  | Total size | 130 |  | 66 | 49.23% |
| Use-case#1 (bit-overhead case) |  |  |  |  |  |  |
| Required bit-size for OLS#1 to OLS#2 |  | Ref. (SHVC/MV-HEVC) |  | Proposal |  | bit-saving |
|  | bits/OLS | Counts | Total size (bits) | Counts | Total size (bits) |  |
| Required bit-size on ptl_dpb_info_present_flag | 1 | 0 | 0 | 1 | 1 | N/A |
| Required bit-size on profile_level_tier_idx | 1 | 2 | 2 | 2 | 2 | 0.00% |
| Required bit-size on DPB Size syntax element | 64 | 2 | 128 | 2 | 128 | 0.00% |
|  |  | Total size | 130 |  | 131 | -0.77% |

Please note that NumSubDpb[] is equal to NumLayersInIdList[] since we assume all layer’s representation format are different for the Use-case#1.

Table 2: Results of bit-estimation for Use-case#1

| Use-case#2 (bit-saving case) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required bit-size for OLS#1 to OLS#2 |  | Ref. (SHVC/MV-HEVC) |  | Proposal |  | bit-saving |
|  | bits/OLS | Counts | Total size (bits) | Counts | Total size (bits) |  |
| Required bit-size on ptl_dpb_info_present_flag | 1 | 0 | 0 | 2 | 2 | N/A |
| Required bit-size on profile_level_tier_idx | 1 | 3 | 3 | 1 | 1 | 66.77% |
| Required bit-size on DPB Size syntax element | 124 | 3 | 372 | 1 | 124 | 66.77% |
|  |  | Total size | 375 |  | 127 | 66.13% |
| Use-case#2 (bit-overhead case) |  |  |  |  |  |  |
| Required bit-size for OLS#1 to OLS#2 |  | Ref. (SHVC/MV-HEVC) |  | Proposal |  | bit-saving |
|  | bits/OLS | Counts | Total size (bits) | Counts | Total size (bits) |  |
| Required bit-size on ptl_dpb_info_present_flag | 1 | 0 | 0 | 2 | 2 | N/A |
| Required bit-size on profile_level_tier_idx | 1 | 3 | 3 | 3 | 3 | 0.00% |
| Required bit-size on DPB Size syntax element | 124 | 3 | 372 | 3 | 372 | 0.00% |
|  |  | Total size | 375 |  | 377 | -0.53% |

Please note that NumSubDpb[] is equal to 1 since we assume all layer’s representation format are the same for the Use-case#2.

# Reference

- Y.-K. Wang, “MV-HEVC/SHVC HLS: On default target output layer set”, JCTVC-P0295/JCT3V-G0236.

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, “SHVC Draft 5”, JCTVC-P1008.

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 7”, JCT3V-G1004.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).