| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0166 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0054 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On scaled reference layer offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes change on scaled reference offset information signalling. It is proposed to signal the offset information in both VPS extension and slice header extension.

# Proposal : scaled reference layer offset signalling

It is proposed to re-consider moving scaled reference layer offset information from SPS extension to VPS extension and adding slice-level (picture-level) scaled reference layer offset information.

JCTVC-N0089 [1] proposes the same concept. At that time, it was not agreed and the note reflects the discussion as "Regarding allowing picture-level adaptation of the offset values, this might add decoder complexity and it did not seem very likely that encoders would want to use that feature." Although we agree that it might add some decoder complexity, we think that it is an important functionality for ROI application. We anticipate that encoders will likely to use this functionality to extract different sub-region for different pictures.

VPS extension

Scaled reference layer offset information is moved from SPS extension to VPS extension. The current syntax structure in SPS extension is preserved.

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... | u(1) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) { |  |
| num_scaled_ref_layer_offsets[ i ] | ue(v) |
| for( j = 0; j < num_scaled_ref_layer_offsets[ i ]; j++ ) { |  |
| scaled_ref_layer_id[ i ][ j ] | u(6) |
| vps_scaled_ref_layer_left_offset[ i ][ scaled_ref_layer_id[ i ] [ j ] ] | se(v) |
| vps_scaled_ref_layer_top_offset[ i ][ scaled_ref_layer_id[ i ] [ j ] ] | se(v) |
| vps_scaled_ref_layer_right_offset[ i ][ scaled_ref_layer_id[ i ] [ j ] ] | se(v) |
| vps_scaled_ref_layer_bottom_offset[ i ][ scaled_ref_layer_id[ i ] [ j ] ] | se(v) |
| vert_phase_position_enable_flag[ i ][ scaled_ref_layer_id[ i ] [ j ] ] | u(1) |
| } |  |
| } |  |
| vps_non_vui_extension_length | ue(v) |
| ... |  |
| } |  |

SPS extension

Original scaled reference layer offset information is removed. To signal whether the offsets exist in slice headers of the current layer, scaled_ref_layer_offsets_in_slice_header_flag is added.

| sps_multilayer_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| scaled_ref_layer_offsets_in_slice_header_flag | u(1) |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_id[ i ] | u(6) |
| scaled_ref_layer_left_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_top_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_right_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_bottom_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| vert_phase_position_enable_flag[ scaled_ref_layer_id[ i ] ] | u(1) |
| } |  |
| } |  |

Slice segment header (extension)

Scaled reference layer offsets are signalled if it is indicated in SPS.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| ... |  |
| if ( scaled_ref_layer_offsets_in_slice_header_flag ) { |  |
| for ( i = 0; i < NumActiveRefLayerPics; i++ ) { |  |
| slice_scaled_ref_layer_left_offset[ i ] | se(v) |
| slice_scaled_ref_layer_top_offset[ i ] | se(v) |
| slice_scaled_ref_layer_right_offset[ i] | se(v) |
| slice_scaled_ref_layer_bottom_offset[ i ] | se(v) |
| } |  |
| } |  |
| while( more_data_in_slice_segment_header_extension( ) ) |  |
| slice_segment_header_extension_data_bit | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

# Patent rights declaration

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Y.-K. Wang, Y. Chen, X. Li, " MV-HEVC/SHVC HLS: On signalling of scaled reference offset," JCTVC-N0089, JCT-VC 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013