| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0167 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0055 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On layers not present SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to remove semantics restriction of Layers not present SEI message for clean up. Additionally, it is proposed to introduce a new syntax to support maximum sub-layers information signaling for a specific period.

# Proposal

Remove a semantics

It is proposed to remove the following semantics restriction

When layer_not_present_flag[ i ] is equal to 0 and i is greater than 0, layer_not_present_flag[ LayerIdxInVps[ RefLayerId[ layer_id_in_nuh[ i ] ][ j ] ] ] shall be equal to 0 for all values of j in the range of 0 to NumDirectRefLayers[ layer_id_in_nuh[ i ] ] − 1, inclusive.

Because the SEI can apply to a particular set of access units rather than the whole CVS, the restriction, “if a layer is signalled as 'may present', the direct dependency layer of the layer also shall be signalled as 'may present' ”,  should be not valid. So we suggest removal.

Add a syntax, named lp_max_sub_layers_minus1

It is also proposed to add maximum temporal sub layer information to signal the case that the number of temporal sub layer is reduced in specific period in the CVS. The use case seems obvious.

Use-case:

- In the case, one layer provides higher frame rate video in some period, the information whether higher frame rate video may be provided at the specific period is useful.

Also if we remove higher-frame-rate layers for some periods, the higher temporal sub-layers are also eliminated by this. The proposed syntax helps us understand the case that temporal sub-layers are reduced due to the layer removal.

Note: Similar concept was proposed in JCTVC-N0173 and JCTVC-O0117.  Those contributions were remarked as “the actual number of temporal sub-layers present may be useful information, but would be difficult to derive and specify.", which was noted in HLS-BoG of JVTVC-O0349.

This time, we take care that the influence be minimum to address the above concerns.

- F.14.1.1 Layers not present SEI message syntax

| layers_not_present( payloadSize ) { | Descriptor |
| --- | --- |
| lp_sei_active_vps_id | u(4) |
| for( i = 0; i <= MaxLayersMinus1; i++ ) |  |
| layer_not_present_flag[ i ] | u(1) |
| lp_max_sub_layers_minus1 | u(3) |
| } |  |

- F.14.2.1. Layers not present SEI message semantics

The layers not present SEI message provides a mechanism for signalling that VCL NAL units of particular layers indicated by the VPS are not present in a particular set of access units.

The target access units are defined as the set of access units starting from the access unit containing the layers not present SEI message up to but not including the next access unit, in decoding order, that contains a layers not present change SEI message or the end of the CVS, whichever is earlier in decoding order.

When present, the layers not present SEI message applies to the target access units.

A layers not present SEI message shall not be included in a scalable nesting SEI message.

A layers not present SEI message shall not be included in an SEI NAL unit with TemporalId greater than 0.

lp_sei_active_vps_id identifies the active VPS of the CVS containing the layers not present SEI message. The value of lp_sei_active_vps_id shall be equal to the value of vps_video_parameter_set_id of the active VPS for the VCL NAL units of the access unit containing the SEI message.

layer_not_present_flag[ i ] equal to 1 indicates that there are no VCL NAL units with nuh_layer_id equal to layer_id_in_nuh[ i ] present in the target access units. layer_not_present_flag[ i ] equal to 0 indicates that there may or may not be VCL NAL units with nuh_layer_id equal to layer_id_in_nuh[ i ] present in the target access units.

When layer_not_present_flag[ i ] is equal to 0 and i is greater than 0, layer_not_present_flag[ LayerIdxInVps[ RefLayerId[ layer_id_in_nuh[ i ] ][ j ] ] ] shall be equal to 0 for all values of j in the range of 0 to NumDirectRefLayers[ layer_id_in_nuh[ i ] ] − 1, inclusive.

lp_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the target access units. The value of lp_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.

# Patent rights declaration

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. W. Kang, J. Lee, H. Lee, J. S. Choi, T. C. Thang, “MV-HEVC/SHVC HLS: On Layers Present SEI message”, JCTVC-N0173, JCT-VC 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013

[2] T. C. Thang, J. W. Kang, J. Lee, H. Lee, J. S. Choi, “MV-HEVC/SHVC HLS: On Layers Not Present SEI message”, JCTVC-O0177, JCT-VC 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013

[3] J. Boyce, “BoG report on SHVC/MV-HEVC HLS topics”, JCTVC-O0349, JCT-VC 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013

[4] Y.-K. Wang, Y. Chen, X. Li, " MV-HEVC/SHVC HLS: On signalling of scaled reference offset," JCTVC-N0089, JCT-VC 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013