| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0177 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0056 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Miscellaneous HLS topics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Adarsh K. Ramasubramonian Ye-Kui Wang Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-651-3481 vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes several changes to the current MV-HEVC/SHVC design. The proposed changes include the following:

- Modifying of the inference rule for sps_temporal_id_nesting_flag when it is not present.

- Clarification of the description related to the end of sequence NAL unit and the end of bitstream NAL unit.

- Signalling of the presence of discardable pictures in the bitstream.

# On inference value of sps_temporal_id_nesting_flag

The semantics of sps_temporal_id_nesting_flag in the text specification of SHVC/MV-HEVC are specified as follow:

sps_temporal_id_nesting_flag, when sps_max_sub_layers_minus1 is greater than 0, specifies whether inter prediction is additionally restricted for CVSs referring to the SPS. When vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag shall be equal to 1. When sps_max_sub_layers_minus1 is equal to 0, sps_temporal_id_nesting_flag shall be equal to 1. When not present, sps_temporal_id_nesting_flag is inferred to be equal to vps_temporal_id_nesting_flag.

The above semantics has problem in the inference of sps_temporal_id_nesting_flag when it is not present. The problematic constraints are as follow:

- When not present, sps_temporal_id_nesting_flag is inferred to be equal to vps_temporal_id_nesting_flag.

- When sps_max_sub_layers_minus1 is equal to 0, sps_temporal_id_nesting_flag shall be equal to 1.

Conflict occurs when sps_temporal_id_nesting_flag is not present and the values of both vps_temporal_id_nesting_flag and sps_max_sub_layers_minus1 are equal to 0. In such cases, both the above mentioned conditions cannot be satisfied at the same time and it is not clear what the value of sps_temporal_id_nesting_flag should be inferred to.

To solve the above problem, it is proposed to modify the semantics of sps_temporal_id_nesting_flag as follows:

sps_temporal_id_nesting_flag, when sps_max_sub_layers_minus1 is greater than 0, specifies whether inter prediction is additionally restricted for CVSs referring to the SPS. When vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag shall be equal to 1. When sps_max_sub_layers_minus1 is equal to 0, sps_temporal_id_nesting_flag shall be equal to 1.

When not present, the value of sps_temporal_id_nesting_flag is inferred as follows: to be equal to vps_temporal_id_nesting_flag.

- If sps_max_sub_layers_minus1 is greater than 0, the value of sps_temporal_id_nesting_flag is inferred to be equal to vps_temporal_id_nesting_flag.

- Otherwise, the value of sps_temporal_id_nesting_flag is inferred to be equal to 1.

# On end of sequence (EOS) NAL unit

The specification related to the end of sequence NAL unit in the current SHVC/MV-HEVC specifications is mostly inherited from HEVC. Given that SHVC/MV-HEVC may contain pictures of one or more layers and that an access unit contains pictures that have the same picture order count, it is not clear whether an end of sequence NAL unit applies to a certain layer or applies to all layers.

It is proposed that an EOS NAL unit apply to all layers in the bitstream, and that it specifies the end of a coded video sequence. To apply this proposal, the following changes are required:

In subclause F.7.4.2.2, add the following:

- When nal_unit_type is equal to EOS_NUT, the value of nuh_layer_id shall be equal to 0. Decoders shall ignore (i.e. remove from the bitstream and discard) all NAL units with a value of nal_unit type equal to EOS_NUT and a value of nuh_layer_id not equal to 0.

In subclause F.7.4.2.4.3:

The specifications in subclause 7.4.2.4.3 apply with the following modification:

Replace

It is a requirement of bitstream conformance that, when present, the next access unit after an access unit that contains an end of sequence NAL unit or an end of bitstream NAL unit shall be an IRAP access unit, which may be an IDR access unit, a BLA access unit, or a CRA access unit.

with

It is a requirement of bitstream conformance that, when present, the next access unit after an access unit that contains an end of sequence NAL unit or an end of bitstream NAL unit shall be an IRAP access unit, which may be an IDR access unit, a BLA access unit, or a CRA access unit.

In subclause F.7.4.3.6:

The specifications in subclause 7.4.3.6 apply with the following addition:

There is at most one end of sequence NAL unit present in an access unit.

In subclause F.7.4.3.7:

The specifications in subclause 7.4.3.7 apply with the following addition:

There is at most one end of bitstream NAL unit present in an access unit.

# On discardable pictures and all_ref_layers_active_flag

## Asserted issue

The semantics of all_ref_layers_active_flag state that when the flag is equal to 1, for each picture referring to the VPS, the reference layer pictures that belong to all direct reference layers of the layer containing the picture and that might be used for inter-layer prediction as specified by the values of sub_layers_vps_max_minus1[ i ] and max_tid_il_ref_pics_plus1[ i ][ j ] are present in the same access unit as the picture and are included in the inter-layer reference picture set of the picture. Furthermore, in the last meeting, it was also agreed that there shall be no picture in the RPS that has discardable_flag equal to 1. As consequence of the semantics of all_ref_layers_active_flag and the recently adopted constraint, when all_ref_layers_active_flag is equal to 1, the use of discardable picture will be very restricted as it can only present in the highest layer or in the sub-layer that are indicated not needed for inter-layer prediction.

Figure 1 illustrates the problem that occurs when all_ref_layers_active_flag is equal to 1 and discardable picture present in layer that is needed for reference layer. Suppose in a bitstream with three layers - Layer 0, Layer 1, and Layer 2 - in which Layer 1 depends on Layer 0, and Layer 2 depends on Layer 1 and Layer 0 and the value of all_ref_layers_active_flag is equal to 1. When a discardable picture in Layer 1 is removed/discarded from the bitstream, the derivation of inter-layer reference picture set (IL-RPS) of the picture in the same access unit in Layer 2 will have an error. Since the value of all_ref_layers_active_flag is equal to 1, the derivation process of IL-RPS will include all direct reference layers of Layer 2 into RefPicLayerId[ i ] and try to find reference pictures in the DPB that has the same POC as the current picture and with nuh_layer_id equal to the entry in the RefPicLayerId[ i ]. As the discardable picture has already been removed, there will be "no reference picture" entry in the resulting inter-layer reference picture set (IL-RPS). This will violate the constraint such that there shall be "no reference picture" entry in IL-RPS.

Figure 1 - Problem of discardable picture and all_ref_layers_active_flag

It is asserted that discardable picture offers a good feature to enable extraction of individual pictures. On the other hand, the syntax element all_ref_layers_active_flag when equal to 1 would allow not signalling any IL-RPS information in the slice segment header for typical MV-HEVC use-cases.

It is desirable to enable the use of both features (i.e., discardable picture and all_ref_layers_active_flag) in bitstream without any constraints, and hence the following solution is proposed.

## Solution

The key idea of the solution to enable the use of both discardable picture and all_ref_layers_active_flag is that even when the value of all_ref_layers_active_flag is equal to 1, when at least one discardable picture present in direct reference layers of the current picture, IL-RPS information shall be signalled explicitly.

To apply the idea, the following are proposed:

- A flag vps_discardable_pic_not_present_flag is signalled in VPS extension to indicate whether or not ref_layer_discardable_pic_not_present_flag (i.e., described in below point) present in slice segment header.

- When all_ref_layers_active_flag is equal to 1 and vps_discardable_pic_not_present is equal to 0, a flag ref_layer_discardable_pic_not_present_flag is signalled in slice segment headers of non-independent enhancement layers to indicate whether at least one of the pictures in direct reference layers in the current access unit is a discardable picture.

- When it is indicated that at least one of the pictures in direct reference layers in the current access unit is a discardable picture, the list of pictures to be included in IL-RPS shall be signalled.

- The syntax and semantics of the above proposed flags are as follows:

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| discardable_pic_not_present_flag | u(1) |
| … |  |
| } |  |

vps_discardable_pic_not_present_flag equal to 1 specifies that ref_layer_discardable_pic_not_present_flag is not signalled in the slice segment header. vps_discardable_pic_not_present_flag equal to 0 specifies that ref_layer_discardable_pic_not_present_flag is signalled in the slice segment header of pictures that belong to any layer that has at least one direct reference layer.. When not present, the value of vps_discardable_pic_not_present_flag is inferred to be equal to 1

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 && !vps_discardable_pic_not_present_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| ref_layer_discardable_pic_not_present_flag | u(1) |
| if( nuh_layer_id > 0 && (!all_ref_layers_active_flag ¦ ¦ (all_ref_layers_active_flag && !ref_layer_discardable_pic_not_present_flag)) && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

ref_layer_discardable_pic_not_present_flag equal to 1 indicates any picture in the current access unit with nuh_layer_id equal to one of the entries in RefLayerId[ nuh_layer_id ][ i ] for i in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] – 1, inclusive, and that may be used for inter-layer prediction as specified by the values of sub_layers_vps_max_minus1[ nuh_layer_id ] and max_tid_il_ref_pics_plus1[ i ][ nuh_layer_id  ] for i in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] – 1, inclusive, has discardable_flag is equal to 0. ref_layer_discardable_pic_not_present_flag equal to 0 indicates that any picture in the current access unit with nuh_layer_id equal to one of the entries in RefLayerId[ nuh_layer_id ][ i ] for i in the range from 0 to NumDirectRefLayers[ nuh_layer_id ] – 1, inclusive, and that may be used for inter-layer prediction as specified by the values of sub_layers_vps_max_minus1[ nuh_layer_id ] and max_tid_il_ref_pics_plus1[ i ][ nuh_layer_id  ] for i in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] – 1, inclusive, may or may not have discardable_flag is equal to 0. When not present, the value of ref_layer_discardable_pic_not_present_flag is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).