| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0061.r2 M32961.r2 |
| --- | --- |

| Title: | AHG4: Texture and Depth View Packing SEI Message Integration in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto Masayuki Tanimoto | Tel: Email: | +81-42-327-7262 {senoh, k.wakunami, y-ichihashi, sasaki, k.yamamoto}@nict.go.jp +81-52-781-1883 tanimototentative3@yahoo.co.jp |
| Source: | NICT and NISRI |  |  |

_____________________________

# Abstract

This contribution reports on 3D-AVC software integration for revised depth view info SEI message into ATM10.0r1.

# Introduction

At the 7th JCT-3V meeting, a revised texture and depth view packing SEI message in 3D-AVC (JCT3V-G0107) were proposed and reviewed which includes camera parameters for view synthesis. According to the reviewer’s comments, a revised text (JCT3V-H0059) is contributed to the 8th JCT-3V meeting. According to the revised text, the software for this SEI message was integrated in the 3D-ATM 10.0r1. This contribution reports the integration results.

# Experimental results

The encoder encodes base view, residual view, global depth map and SEI message for texture and depth view packing information. The decoder outputs decoded SEI message in the form of GVDViewSynth.cfg, GVDCameraParam.txt and GVDCameraParam.CSV files together with the decoded base view, residual view and global depth map. Examples of encode/decode.bat files, log file and output files are attached to this contribution. GVDViewSynth.cfg file contains parameter values for depth_type, num_residual_texture_views_minus1, residual_depth_flag, and other parameter values used for the view synthesis. GVDCameraParam.txt file contains parameter values for intrinsic camera parameters (focal length, principal point co-ordinates) and extrinsic camera parameters (rotation matrix and x-translation) for view synthesis. GVDCameraParam.CSV file contains parameter values for Znear, Zfar and x-translation for all views. Multi-views are synthesized from these decoded files.

Table 1 shows the test results for 3-view case. The anchor is ATM v.10.0r1 HP. Since GVD data structure doesn’t use the 3rd view in the ATM’s 3-view case, bitrate calculation function in ATM was modified to change the bitrate conversion coefficient to meet the actual data amount. enc. time includes global depth generation, its encoding, residual view generation, and encoding of the global depth, base view and residual view. The dec. time includes all texture view and global depth decoding. The view synthesis time will be improved by parallelizing the process.  Figure 1 shows average PSNR vs. total bitrate of synthesized views. Figure 2 shows an example of synthesized view 1 by GVD software based on ATM10.0r1HP mode and decoded view 1 by ATM10.0r1 HP mode. The detail is described in the attached files.

Table 1 Summary of GVD performance comparing to ATMv.10.0r1 HP mode for 3-view case.

Fugure1. average PSNR vs. total bitrate of synthesized views

- GVD　with SEI message                       (b) ATM10.0r1HP

Fugure2. Synthesized view 1 frame 0 for Undo_Dancer at QP=26

3D-ATM 10.0r1 software integrating revised texture and depth view packing SEI message is attached to this contribution.

# Conclusions

Integration of texture and depth view packing SEI message including camera parameters in ATM 10.0r1 and its test results were reported. According to this report, it is proposed to issue a sub-division of 14496-10:2012/Amd6: Texture and depth view packing SEI message in 3D-AVC.

# References

[1] T. Senoh, etal., “Texture and depth view packing SEI message in 3D-AVC”, JCT3V-H0059, Valencia, ES, March. 2014.

# Patent rights declaration(s)

NICT (National Institute of Information and Communications Technology) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).