| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0062 |
| --- | --- |

| Title: | CE1: Simplification of 3D-HEVC merge candidate construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

# Abstract

This proposal reports a CE result for removing 3D-HEVC merge candidates in smallest PU of 8x4 / 4x8. The experiment result shows that the BD-rate gain is 0.0 %, 0.0 % and 0.0 % in video, total video and synthesis respectively.

# Introduction

At the last meeting, JCT3V-H0036 reported that 3D-HEVC increases the worst case in merge candidate construction and proposed a simplification. The proposal was tested in CE1 activity.

# Proposal

It is proposed to remove 3D-HEVC merge candidates for 8x4 / 4x8 PU

# Proposed text

- I.8.5.3.2.1 Derivation process for luma motion vectors for merge mode

  - The derivation process for the base merge candidate list as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nCbS, nPbW, nPbH, and the partition index partIdx as inputs, and the output being a modified luma location ( xPb, yPb ), the modified variables nPbW and nPbH, the modified variable partIdx, the luma location ( xOrigP, yOrigP ), the variables nOrigPbW and nOrigPbH, the merge candidate list baseMergeCandList, the luma motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N, and the prediction list utilization flags predFlagL0N and predFlagL1N, with N being replaced by all elements of baseMergeCandList.

  - For N being replaced by A1, B1, B0, A0 and B2, the following applies:

    - If N is an element in baseMergeCandList, availableFlagN is set equal to 1.

    - Otherwise (N is not an element in baseMergeCandList), availableFlagN is set equal to 0.

  - If nOrigPbW+ nOrigPbH > 12, the following applies:,

    - Depending on iv_mv_pred_flag[ nuh_layer_id ] and DispAvailabilityIdc[ xPb ][ yPb ], the following applies:

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0 or DispAvailabilityIdc[ xPb ][ yPb ] is not equal to DISP_NONE, the flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as inputs, and the output is assigned to the availability flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC, the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC, predFlagLXivMCShift and predFlagLXIvDC, and the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC (with X being 0 or 1, respectively).

    - Depending on view_synthesis_pred_flag[ nuh_layer_id ] and DispAvailabilityIdc[ xPb ][ yPb ], the following applies:

    - If view_synthesis_pred_flag[ nuh_layer_id ] is equal to 0 or DispAvailabilityIdc[ xPb ][ yPb ] is equal to DISP_NONE, the flag availableFlagVSP is set equal to 0.

    - Otherwise (view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for a view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma locations ( xPb, yPb ) and the variables nPbW and nPbH as inputs, and the outputs are the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

    - Depending on mpi_flag[ nuh_layer_id ], the following applies:

    - If mpi_flag[ nuh_layer_id ] is equal to 0, the variables availableFlagT and availableFlagD are set equal to 0.

    - Otherwise (mpi_flag[ nuh_layer_id ] is equal to 1), the derivation process for the texture and the disparity derived merging candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as inputs, and the outputs are the flags availableFlagT and  availableFlagD, the prediction utilization flags predFlagLXT, and predFlagLXD, the reference indices refIdxLXT and refIdxLXD, the motion vectors mvLXT and mvLXD (with X being 0 or 1, respectively), and the variable dispDerivedDepthVal.

    - The merging candidate list, extMergeCandList, is constructed as follows:

i = 0
			if( availableFlagT )1
				extMergeCandList[ i++ ] = T
			if( availableFlagD ) 2
				extMergeCandList[ i++ ] = D
			if( availableFlagIvMC3  &&  ( !availableFlagT4  | |  differentMotion1( T, IvMC ) ) )
				extMergeCandList[ i++ ] = IvMC
			N = DepthFlag5 ? T : IvMC
			if( availableFlagA16  &&  ( !availableFlagN7  | |  differentMotion2( N, A1 ) ) )
				extMergeCandList[ i++ ] = A1
			if( availableFlagB17  &&  ( !availableFlagN8  | |  differentMotion3( N, B1 ) ) )
				extMergeCandList[ i++ ] = B1
			if( availableFlagB09 )
				extMergeCandList[ i++ ] = B0		(Error: Reference source not found-1)
			if( availableFlagIvDC10  &&  ( !availableFlagA111  | |  differentMotion4( A1, IvDC ) )  &&  
				( !availableFlagB112  | |  differentMotion5( B1, IvDC ) )  &&  ( i13 < ( 5 + NumExtraMergeCand ) ) )
				extMergeCandList[ i++ ] = IvDC
			if( availableFlagVSP14  &&  !ic_flag15  &&  iv_res_pred_weight_idx16  = =  0  &&  
									i17 < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = VSP
			if( availableFlagA018  &&  i19 < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = A0
			if( availableFlagB220  &&  i21 < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = B2
			if( availableFlagIvMCShift22  &&  i23 < ( 5 + NumExtraMergeCand )  &&
								( !availableFlagIvMC24  | |  differentMotion6( IvMC, IvMCShift ) ) )
				extMergeCandList[ i++ ] = IvMCShift

    - The variable availableFlagIvDCShift is set equal to 0, and when availableFlagIvMCShift is equal to 0, DepthFlag is equal to 0, and i is less than ( 5 + NumExtraMergeCand ), the derivation process for the shifted disparity merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, and the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N, of every candidate N being in extMergeCandList, extMergeCandList, and i as inputs, and the outputs are the flag availableFlagIvDCShift, the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift, the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift, and the motion vectors mvL0IvDCShift and mvL1IvDCShift.

    - The merging candidate list, extMergeCandList, is constructed as follows:

if( availableFlagIvDCShift )
				extMergeCandList[ i++ ] = IvDCShift
			j = 0
			while( i < MaxNumMergeCand ) {	(Error: Reference source not found-2)
				N = baseMergeCandList[ j++ ]
				if( N  !=  A1  &&  N  !=  B1  &&  N  !=  B0  &&  N  !=  A0  &&  N  !=  B2 )
					extMergeCandList[ i++ ] = N
			}

  - Othewise (nOrigPbW+ nOrigPbH<=12), the following applies

for ( i = 0;  i < MaxNumMergeCand; i++ ) {	(Error: Reference source not found-xxx)

extMergeCandList[ i] = baseMergeCandList[ i ]
			}

  - The variable N is set equal to extMergeCandList[ MergeIdx[ xOrigP ][ yOrigP ] ].

  - The derivation process for a view synthesis prediction flag as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nPbW and nPbH, the merge candidate indicator N as the inputs, and the output is the mergeCandIsVspFlag.

  - The variable vspModeFlag is derived as specified in the following:

      - vspModeFlag = mergeCandIsVspFlag  	&&  !ic_flag  &&  
	( iv_res_pred_weight_idx  = = 0 )  &&  availableFlagVSP	(Error: Reference source not found-3)

  - The variable subPbMotionFlag is derived as specified in the following:

      - subPbMotionFlag = ( ( N  = =  IvMC )  &&  ( PartMode  = =  PART_2Nx2N ) )  | |  vspModeFlag	(Error: Reference source not found-4)

  - The following assignments are made with X being replaced by 0 or 1:

      - mvLX = subPbMotionFlag ? 0 : mvLXN		(Error: Reference source not found-5)

      - refIdxLX = subPbMotionFlag ? −1 : refIdxLXN	(Error: Reference source not found-6)

      - predFlagLX = subPbMotionFlag ? 0 : predFlagLXN	(Error: Reference source not found-7)

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, the following applies:

      - refIdxL1 = −1		(Error: Reference source not found-8)

      - predFlagL1 = 0		(Error: Reference source not found-9)

  - The disparity availability flag ivpMvFlag is derived as follows:

      - ivpMvFlag = !DepthFlag  &&  ( ( N  =  =  IvMC )  | |  ( N  = =  IvMCShift ) )	(Error: Reference source not found-10)

  - The variable dispDerivedDepthFlag is derived as follows:

      - dispDerivedDepthFlag = ( N  = =  D )		(Error: Reference source not found-11)

# Experimental results

Experimental result based on HTM10 is shown in Table 1.

Table 1 Experimental result (anchor: HTM10)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.0% | 0.01% | 0.00% | 0.03% | 99.9% | 97.2% | 99.9% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.00% | 0.01% | 0.00% | 99.5% | 99.7% | 100.0% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | -0.05% | 99.4% | 100.4% | 99.2% |
| GT_Fly | 0.0% | 0.1% | 0.0% | 0.01% | 0.01% | 0.02% | 100.2% | 100.6% | 100.0% |
| Poznan_Hall2 | 0.0% | 0.2% | -0.7% | -0.06% | -0.06% | -0.11% | 100.1% | 102.0% | 100.0% |
| Poznan_Street | 0.0% | -0.1% | 0.1% | 0.02% | 0.02% | 0.01% | 99.5% | 99.5% | 100.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.02% | 0.03% | 0.01% | 99.7% | 101.3% | 100.9% |
| Shark | 0.0% | 0.1% | 0.0% | 0.01% | 0.00% | -0.01% | 99.7% | 100.6% | 100.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.01% | 99.6% | 99.1% | 99.7% |
| 1920x1088 | 0.0% | 0.1% | -0.1% | 0.00% | 0.00% | -0.02% | 99.8% | 100.8% | 100.2% |
| average | 0.0% | 0.0% | -0.1% | 0.00% | 0.00% | -0.01% | 99.7% | 100.2% | 100.0% |

# Complexity reduction

With the proposal, number of candidates decreases from 13 or 11 to 8. 3D-HEVC related inter-view motion data access is removed from 8x4/4x8 candidate derivation.

|  | HEVC | 3D-HEVC Texture | 3D-HEVC Depth | Proposal (8x4/4x8 PU) |
| --- | --- | --- | --- | --- |
| merge cand |  |  | T |  |
|  |  |  | DDD |  |
|  |  | IvMC | IvMC |  |
|  | A1 | A1 | A1 | A1 |
|  | B1 | B1 | B1 | B1 |
|  | B0 | B0 | B0 | B0 |
|  |  | IvDC |  |  |
|  |  | VSP |  | (inheritance only) |
|  | A0 | A0 | A0 | A0 |
|  | B2 | B2 | B2 | B2 |
|  |  | IvMCShift |  |  |
|  |  | IvDCShift |  |  |
|  | Col | Col | Col | Col |
|  | combined-Bi | combined-Bi | combined-Bi | combined-Bi |
|  | Zero | Zero | Zero | Zero |
| #merge cand | 8 | 13 | 11 | 8 |

# Conclusion

Because the proposal addresses the worst case concerns of merge candidate construction process with negligible impact, it is recommended to adopt this method in 3D-HEVC.

# Patent rights declaration

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] T. Ikai, “CE3-related: Worst case complexity reduction for merge candidate construction”, JCT3V-H0034, JCT-3V 7th Meeting: San Jose, USA, 11 Jan. – 17 Jan. 2013