| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0066 |
| --- | --- |

| Title: | 3D-CE1 related: Sub-PU Size for MPI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the Sub-PU prediction for MPI allows the motion compensation prediction (MCP) block sizes which are not in the HEVC base specification, such as 12x8 and 8x12. These MCP block sizes need to be avoided to have a design consistency with HEVC version 1. In order to resolve this problem, it is proposed to allow the Sub-PU prediction for MPI only in PART_2Nx2N PU partition mode. With the proposed method, the MCP block size for Sub-PU prediction always becomes 8x8. The proposed method reportedly provides no coding loss (0.04% bit-saving for synthesized views).

# Introduction

In the current 3D-HEVC [1], the Sub-PU prediction for MPI (Motion Parameter Inheritance) allows the motion compensation prediction (MCP) block sizes which are not in the HEVC base specification, such as 12x8 and 8x12.

Figure 1. Current Sub-PU partition for AMP mode (CU size: 16x16)

Figure 1 shows examples of not aligned MCP block sizes. These MCP block sizes need to be avoided to have a design consistency with HEVC version 1.

# Proposed Methods

In order to align MCP block sizes with HEVC version 1, it is proposed to apply the Sub-PU prediction for MPI only to PART_2Nx2N PU partition mode as doing in the Sub-PU IVMV (Inter-view Motion Vector) mode [2]. If the Sub-PU prediction for MPI is allowed only to PART_2Nx2N PU partition mode the MCP block size for the Sub-PU prediction always becomes 8x8.

# Experimental Results

The simulation was carried out on the common test condition [3] and was based on HTM 10.0r1 reference software. The performance of the proposed method is shown in Table 1. The proposed method provides no coding loss (0.01% and 0.04% bit-saving for coded and synthesized views, respectively).

Table 1. Comparison results between CTC anchor and proposed method

# Conclusion

This contribution proposes to allow the Sub-PU prediction for MPI only in PART_2Nx2N PU partition mode in order to align with MCP block sizes with HEVC version 1. The proposed method provides no coding loss (0.04% bit-saving for synthesized views). Therefore, it is recommended to adopt proposed method into next 3D-HEVC Working Draft.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” JCT3V-G1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.

[2] M. W. Park, J. Y. Lee, C. Kim, “3D-CE1 related: MCP Size and DV for Sub-PU Prediction,” JCT3V-G0077, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.