| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0067 |
| --- | --- |

| Title: | 3D-CE1 related: CU-level VSP and DV Candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the VSP and DV candidates use NBDV and DoNBDV which are derived in CU-level; however these candidates are performed in every PU partition. These candidates in PU-level seem to be a little redundant since the same disparity vector is used for all PU in one CU. Therefore, it is proposed to allow VSP and DV candidates only in PART_2Nx2N PU partition mode, which is the same unit of the NBDV and DoNBDV derivation. Additionally, with the proposed method, it is unnecessary to consider AMP modes in VSP candidates any longer as the Sub-PU IVMV candidate and to access the PU-level depth block for the VSP Sub-PU partition mode decision. The proposed method reportedly provides no coding loss.

# Introduction

In the current 3D-HEVC [1], the view synthesis prediction (VSP) and the disparity vector (DV) candidates in texture merge candidate list use NBDV (disparity vector from neighboring blocks) and DoNBDV (depth oriented NBDV) which are derived in CU (coding unit) level. However, these candidates are performed in every PU (prediction unit) partition. These candidates in PU-level seem to be a little redundant since the same disparity vector is used for all PU in the same CU. For instance, if the current CU is partitioned into two PU (such as PART_2NxN, PART_Nx2N, etc.) and the both PUs are coded with VSP or DV candidates, then both PUs always have the same disparity vector because the disparity vector used for the VSP or DV candidates is derived in CU-level.

# Proposed Methods

In this contribution, it is proposed to allow VSP and DV candidates only in PART_2Nx2N PU partition mode, which is the same unit of the NBDV and DoNBDV derivation. With the proposed method, the unit of the DV derivation and, VSP and DV candidates can be aligned, and it is unnecessary to consider AMP (asymmetric motion partition) modes in VSP candidates any longer as the Sub-PU IVMV (inter-view motion vector) candidate and to access the PU-level depth block for the VSP Sub-PU partition mode decision.

# Experimental Results

The simulation was carried out on the common test condition [2] and was based on HTM 10.0r1 reference software. In this section, there are three test cases; allowing VSP candidates only in PART_2Nx2N PU partition mode (Test 1), allowing DV candidates only in PART_2Nx2N PU partition mode (Test 2), combining Test 1 and Test 2 (Test 3). Table 1, Table 2 and Table 3 show the performance of Test 1, Test 2 and Test 3 respectively. As shown in the results, the proposed methods provide no coding loss.

Table 1. Comparison results between CTC anchor and Test 1 (allowing VSP only in PART_2Nx2N)

Table 2. Comparison results between CTC anchor and Test 2 (allowing DV candidate only in 2Nx2N)

Table 3. Comparison results between CTC anchor and Test 3 (combining Test 1 and Test 2)

# Conclusion

In this contribution, it is proposed to allow the VSP and DV candidates only in PART_2Nx2N PU partition mode, which is the unit of the DV derivation. With the proposed methods, the unit of DV derivation and these candidates can be aligned and additionally complexity for VSP process can also be reduced without any coding loss. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC Working Draft.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” JCT3V-G1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.