| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0068 |
| --- | --- |

| Title: | Simplification of DBBP in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, DBBP is relatively high complexity because it needs to access all depth samples in the corresponding depth block to derive a PU partition mode and segmentation mask while other depth-based processes such as the Sub-PU partition size and 2nd DV derivation for VSP and DoNBDV derivation only access 4 corner depth pixels. Moreover, DBBP has currently six partition modes, but four AMP modes would be redundant because they can be covered by two symmetric partition modes. In order to reduce the complexity of DBBP, it is proposed to use only corner pixels, which are already used in other depth-based process, in the corresponding depth block for deriving the PU partition mode and segmentation mask, and to allow 2 symmetric partition modes. The proposed methods provide no coding loss (0.04% bit-saving for synthesized views) with about 97% both encoding and decoding time.

# Introduction

In the current 3D-HEVC [1], DBBP (depth based block partitioning) [2] has the derivation processes of a PU partition mode and a segmentation mask. However, these derivation processes need to access all depth samples in the corresponding depth block while the other depth-based processes such as the Sub-PU partition size and 2nd DVs derivation for VSP (view synthesis prediction), and DoNBDV (depth oriented neighboring blocks disparity vector) derivation only access four corner depth pixels. Moreover, DBBP has currently six partition modes including AMP (asymmetric motion partition) modes, but four AMP modes (PART_2NxnU, PART_2NxnD, PART_nLx2N and PART_nRx2N) would be redundant because they can be covered by two symmetric partition modes (PART_2NxN and PART_Nx2N). For instance, the best matching PU partition of an example in Figure 1 can be either PART_Nx2N or PART_nLx2N.

Figure 1. Example of possible PU partition modes in DBBP [2].

# Proposed Methods

## Simplification of Segmentation Threshold Derivation

In DBBP process, all depth samples in the corresponding depth block are needed to access to derive the segmentation threshold used for making a segmentation mask. In order to reduce the complexity, it is proposed to use 4 corner depth samples (a, b, c and d in Figure 2) in the corresponding depth block as shown in Figure 2.

Figure 2. Example of 4 corner depth samples used for proposed derivation.

The current and proposed threshold derivation methods can be illustrated as equations in Figure 3 (Here, N indicates the width and height of the current CU and d(x, y) indicates a depth pixel in the corresponding depth block). While the threshold (Thcurr) of the current method uses the average of all depth samples, the threshold (Thprop) of the proposed one uses the average of 4 corner depth samples.

Figure 3. Comparison between the current and proposed derivation methods.

## Simplification of Partition Derivation

In this contribution, it is also proposed to use two symmetric PU partition modes (PART_2NxN and PART_Nx2N) for the partition derivation process in DBBP, and to use 3 depth samples in the corresponding depth block to derive the PU partition mode of the current CU.

The partition derivation process of DBBP consists of two steps. Firstly, the current CU is partitioned into in unit of 4x4 pixel block and then the threshold is calculated by averaging of top-left pixels of all 4x4 blocks, which are marked in Figure 4. Secondly, 4x4 block-wise segmentation map is derived by using the calculated threshold in the first step, and then the best matching PU partition mode is finally derived.

Figure 4. Example of used depth samples to derive the PU partition mode in the current method
 (CU size: 16x16).

The proposed partition derivation simply uses 3 corner depth samples (a, b and c in Figure 5) in the corresponding depth block as shown in Figure 5. It compares the gradients between ‘a’ and ‘b’ and between ‘a’ and ‘c’, and then directly decides the PU partition mode between PART_2NxN and PART_Nx2N.

Figure 5. 3 corner depth samples to derive the PU partition mode in the proposed method

The proposed partition derivation method can be illustrated as shown in Figure 6.

Figure 6. Proposed partition derivation method

# Experimental Results

The simulation was carried out on the common test condition [3] and was based on HTM 10.0r1 reference software. In this section, there are three test cases; simplification of the derivation of the segmentation threshold used for making segmentation mask (Test 1), simplification of the partition decision (Test 1), and combining Test 1 and Test 2 (Test 3).

Table 1 shows the results of simplification of the segmentation threshold derivation. The proposed method provides no coding loss (0.02% bit-saving for synthesized views) with 97.2% encoding time.

Table 1. Comparison results between CTC anchor and Test 1

Table 2 shows the results of simplification of the partition derivation. The proposed method provides no coding loss (-0.04% bit-saving for synthesized views) with 96.1% decoding time.

Table 2. Comparison results between CTC anchor and Test 2

Table 3 shows the result of combining two proposed methods. The proposed methods provide no coding loss (0.04% bit-saving for synthesized views) with about 97% both encoding and decoding time.

Table 3. Comparison results between CTC

# Conclusion

In this contribution, it is proposed to simply the derivation processes of the segmentation threshold and the PU partition. With the proposed method, only 4 corner depth samples, which are already used in other depth based processes, are used so that the memory and computational complexity can be reduced. The proposed methods provide no coding loss (0.04% bit-saving for synthesized views) with about 97% both encoding and decoding time. Therefore, it is recommended to adopt the proposed methods into next 3D-HEV Working Draft.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” JCT3V-G1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.

[2] F. Jäger, J. Konieczny, G. Cordara, “CE3: Results on Depth-based Block Partitioning (DBBP),” JCT3V-G0106, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.

[3] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.