| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0069 |
| --- | --- |

| Title: | Disparity Vector for DBBP in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, DBBP uses DoNBDV to find the corresponding depth block, which is used for deriving a segmentation mask and a PU partition mode. However, this can cause of increasing the memory access bandwidth requirement because it needs to fetch the depth block twice; one is for the DoNBDV derivation and the other is for the derivation of the segmentation mask and the PU partition mode in DBBP. Therefore, it is proposed to use NBDV instead of DoNBDV for the DBBP process in order to reduce the number of depth blocks to be fetched. The proposed method reportedly provides no coding loss.

# Introduction

In the current 3D-HEVC [1], DBBP (depth based block partitioning) uses DoNBDV (depth oriented neighboring blocks disparity vector) to find the corresponding depth block, which is used for deriving a segmentation mask and a PU (prediction unit) partition mode. However, this can cause of increasing the memory access bandwidth requirement because it needs to fetch the depth block twice as shown in Figure 1; the first corresponding depth block pointed by NBDV (neighboring blocks disparity vector) is for the DoNBDV derivation and the second corresponding depth block pointed by DoNBDV is for the derivation of the segmentation mask and PU partition mode in DBBP.

Figure 1. Example of two depth block fetches

# Proposed Methods

In this contribution, in order to reduce the memory access complexity, it is proposed to use NBDV instead of DoNBDV for the DBBP process. Through the proposed method which can align the corresponding depth block for the DoNBDV derivation with the one for the DBBP process, the number of depth blocks to be fetched can be reduced from 2 to 1.

# Experimental Results

The simulation was carried out on the common test condition [2] and was based on HTM 10.0r1 reference software. The performance of the proposed method is shown in Table 1. The proposed method provides no coding loss.

Table 1. Comparison results between CTC anchor and proposed method

# Conclusion

In this contribution, it is proposed to use NBDV instead of DoNBDV for DBBP process in order to reduce the memory access bandwidth requirement. With the proposed method, the number of depth blocks to be fetched is reduced from 2 to 1 without any coding loss. Therefore, it is recommended to adopt proposed method into next 3D-HEVC Working Draft.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” JCT3V-G1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.