| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0070 |
| --- | --- |

| Title: | 3D-CE1: Results on Adaptive Disabling Inter-view Motion Vector Candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Byeongdoo Choi Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com b.d.choi@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, IC is only applied to CU which performs the inter-view prediction. If the IVMV candidates which always perform the temporal prediction are used, IC is never performed for the current CU (i.e. ic_flag of the current CU is always set to ‘0’ and sometimes signaled). Therefore, when IC is enabled the IVMV candidates can be excluded from the merge candidate list. Additionally, even though the IVMV candidates will be disabled, the motion information of the IVMV candidates can be used for the pruning process. When the motion information is used for the pruning process, the proposed method reportedly provides 0.04% and 0.01% bit-saving for coded and synthesized views, respectively. However, if the motion information is not used for pruning process, it can simplify the merge candidate list construction process since the IVMV derivation process, which needs to access the motion parameters of the corresponding block in the reference picture, can be omitted. This simplified method reportedly provides no coding loss (0.02% and 0.03% bit-saving for coded and synthesized views, respectively).

# Introduction

In the current 3D-HEVC [1], IC (illumination compensation) is only applied to CU (coding unit) which performs the inter-view prediction. If IVMV (inter-view motion vector) candidates which always perform the temporal prediction are used, IC is never performed for the current CU (i.e. ic_flag of the current CU is always set to ‘0’ and sometimes signaled in case of the shifted IVMV candidate). Therefore, when IC is enabled the IVMV candidates need to be disabled.

# Proposed Methods

In this contribution, it is basically proposed to disable the IVMV candidates from the merge candidate list when IC is enabled. The current and proposed merge candidate list construction can be illustrated as shown in Figure 1. According to the syntax element ic_flag, the IVMV candidates (“Temporal Inter-view Candidate” and “Temporal Inter-view Candidate (BR)” in Figure 1) can be adaptively disabled.

Figure 1. Comparison between the current and proposed merge candidate list construction

Even if the IVMV candidates will be disabled, the motion information of the IVMV candidates can be used for the pruning process. In this case, it never reduces the complexity of merge candidate list construction.

However, if the motion information of the IVMV candidates is not used for the pruning process, it can simplify the merge candidate list construction process since the IVMV derivation process, which needs to access the motion parameters of the corresponding block in the reference picture, can be omitted. In the worst case, the number of motion parameters to be fetched can be reduced by 64 since, for example, if the size of current CU with IVMV is 64x64 and the Sub-PU size is 8x8, then 64 Sub-PUs need to obtain each motion parameter for each Sub-PU.

# Experimental Results

The simulation was carried out on the common test condition [2] and was based on HTM 10.0r1 reference software. In this contribution, there are two test cases; the proposed method with the pruning process (Test 1: CE 1 proposal) and without the pruning process (Test 2: CE 1 proposal with simplification).

Table 1 shows the results of the proposed method with pruning process. The proposed method provides no coding loss (0.04% and 0.01% bit-saving for coded and synthesized views, respectively).

Table 2 shows the results of the proposed method with simplification (i.e. without pruning process). The proposed method provides no coding loss (0.02% and 0.03% bit-saving for coded and synthesized views, respectively).

Table 1. Comparison results between CTC anchor and Test 1 (CE 1 proposal)

Table 2. Comparison results between CTC anchor and Test 2 (CE 1 proposal with simplification)

# Conclusion

In this contribution, when IC is enabled, it is proposed to disable the IVMV candidates. The motion information of the IVMV candidates can be used for the pruning process. But, it never reduces any complexity, so it is proposed to omit the IVMV derivation in order to simplify the merge candidate list construction (i.e. the motion information of the IVMV candidates is not used for the pruning process). The proposed method provides no coding loss (0.02% and 0.03% bit-saving for coded and synthesized views, respectively). Therefore, it is recommended to adopt the simplified method into next 3D-HEVC Working Draft.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” JCT3V-G1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.