| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCT3V-H0072 |
| --- | --- |

| Title: | Bug-fix of depth-based block partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1MediaTek Beijing: Building 1-B, No. 6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the depth-based block partitioning (DBBP) needs to determine a block partition mode among 2NxN, Nx2N, 2NxnU, 2NxnD, nLx2N, and nRx2N when the CU size is larger than 8x8, and among 2NxN, Nx2N in 8x8 CU. That is because the AMP partition modes are always disabled in 8x8 CU. However, the case when the AMP is disabled by amp_enable_flag=0 for entirely picture has not been considered in the block partitioning stage of DBBP in current software and WD. In this contribution, two methods are proposed to fix the bug. Method 1 proposes to add one condition check to see whether the AMP block partitions are allowed in the block partitioning stage of DBBP. Method 2 proposes to totally disable the AMP partitions in the block partitioning of DBBP. The experimental results reportedly show an averaged 0.04% bit savings on synthesis views for method 2, while the number of block partitioning candidates of DBBP could be reduced from six to two (2NxN and Nx2N).

# Introduction

- In the current working draft (WD) 3 for 3D-HEVC [1] and HTM10.0r1 [2], the DBBP determines a block partition mode among 2NxN, Nx2N, 2NxnU, 2NxnD, nLx2N, and nRx2N when the CU size is larger than 8x8, and among 2NxN, Nx2N in 8x8 CU. However, in current software and WD, the block partitioning of DBBP decides whether the AMP partitions are included in the block partitioning only according to the CU size, i.e., bAMPAvail = uiSize > 8. The case when the AMP is disabled by amp_enable_flag=0 for entirely picture has not been considered.

When AMP is disabled, the commonly-used encoding and decoding implementations will forbid the encoding and decoding processes of AMP. However according to the current WD and software, the partition mode may be modified to one of AMP partitions by DBBP, and it demands the encoder and decoders to invoke the motion prediction process for AMP. Such design will make the amp_enable_flag non-reliable and not consistent with its meaning in HEVC.

# Proposed method

To solve the problem, this contribution proposes two methods.

Method 1 proposes to add one condition check to see whether the AMP block partitions are allowed in the block partitioning stage of DBBP by modifying the definition and range of variable p in the WD as follows.

For p in the range of 0 to 1 + ( (amp_enabled_flag == 0 || log2CbSize == MinCbLog2SizeY)?0 : 4).

Method 2 proposes to totally disallow the AMP partition in the block partitioning of DBBP.

- Experimental results

Because the CTC of 3D-HEVC always enables the AMP partitions, Method 1 does not have impact on the coding efficiency in CTC. As the results of Method 2 shown it Table 1, the simplification achieves an averaged 0.04% bit savings on synthesis views.

Table 1.  The BD-rate results of the proposed method 2 over HTM10.0r1

- WD change

Method 1

I.8.5.7	Derivation process for a modified partitioning mode

For p in the range of 0 to 1 + ( (amp_enabled_flag == 0 || log2CbSize == MinCbLog2SizeY)?0 : 4), inclusive and i in the range of 0 to 1, inclusive, partSum[ p ][ i ] is set equal to 0.

Method 2

I.8.5.7	Derivation process for a modified partitioning mode

For p in the range of 0 to 1, inclusive and i in the range of 0 to 1, inclusive, partSum[ p ][ i ] is set equal to 0.

- Conclusion

In this document, a bug fix is proposed for DBBP to make it not conflict with conditions when AMP is disabled.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, "3D-HEVC Draft Text 3," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-G1001, Jan., 2014

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1

- Fabian Jäger, Jacek Konieczny and Giovanni Cordara, "CE3: Results on Depth-based Block Partitioning (DBBP)," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-G0106, Jan, 2014.