| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCT3V-H0073 |
| --- | --- |

| Title: | Simplification on depth-based block partitioning (DBBP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the depth-based block partitioning (DBBP) comprises steps of partition mode modification, segment mask generation and bi-segment motion compensation. In the step of partition mode modification, with derived virtual depth as input, sub-sample mean value calculation, pixel-by-pixel level CU contour pattern calculation and contour value counting are utilized to modify the partition mode. Such design consumes a certain memory access, hardware resources and computation complexity. In this contribution, two simplification methods are proposed respectively to simplify the partition mode modification and limit the suitable CU sizes for DBBP. Experimental results reportedly show the proposed simplifications bring no coding efficiency loss.

# Introduction

In DBBP coding, during decoding a 8x8 CU, the decoder requires four steps: virtual depth derivation, block partitioning, block segmentation, and bi-segment compensation. Fig. 1 shows the steps. The virtual depth derivation step utilizes the disparity vector from neighboring blocks (NBDV) to address a virtual block of depth from the depth picture in the dependent depth view. One partition mode among 2NxN, Nx2N, 2NxnU, 2NxnD, nLx2N, and nRx2N is selected in the block partitioning step to derive two predictive motion vectors (PMVs), which are utilized for the compensation procedure for the to-be-divided two segments. The block segmentation step divides the current texture block into two segments in forms of segment mask according to the virtual depth block. The bi-segment compensation step utilizes the PMVs and MVDs from the block partitioning step and the derived segment mask from the block segmentation step to fetch the predicted samples and then completes the compensation by two-time 2Nx2N compensations with their own decoded motion vectors. In the final bi-segment compensation step, the compensation is done under the control of the calculated segment mask. The 1st 2Nx2N predicted block samples corresponding to “1” in the segment mask and the 2nd 2Nx2N predicted block samples corresponding to “0” in the segment mask are selected as the predicted samples of the decoded CU. In addition, for border positions where 1 and 0 are neighboring in the segment mask, a blending operation is utilized to average the two predicted values at the same position in the two predicted 2Nx2N blocks.

Figure 1.  The steps which DBBP comprises

There are maximum 6 partition candidates for DBBP [3], including 2NxN, Nx2N, 2NxnU, 2NxnD, nLx2N, and nRx2N. In the process of partition mode modification (I.8.5.7), DBBP firstly calculates the mean value of the left-up corner samples of all the 4x4 sub-blocks of the derived virtual depth block. Secondly, a so-called inter-layer predicted contour pattern is produced using the calculated mean value. The contour pattern is represented by a binary matrix, in which 0 represents the corresponding pixel in virtual depth block is not larger than the mean value and 1 represents the opposite. Afterwards, for each partition candidate in the current 3D-HEVC, DBBP counts two results for each partition candidate. For example, the two results of Nx2N are: (1) the number of 0-value components in the left plus the number of 1-value components in the right and (2) the number of 1-value components in the left plus the number of 0-value components in the right. Finally, by comparing the counted two results for each partition candidate, the one with the largest counted result is selected as the modified partition for the current CU. The above description is summarized in Fig. 2.

Figure 2.  The steps how DBBP modifies the partitioning mode

# Proposed methods

Method 1: Simplifying the derivation process for a modified partitioning mode

This contribution proposes to utilize the comparison among corner points of the derived depth block of the current CU to implement the partition mode decision. The selected corner points for each candidate partition are shown in Fig. 3, which follows the same point selection in VSP. Let a1, a2, a3 and a4 denote the selected corner samples of the first partition of a derived depth block, b1, b2, b3 and b4 denote the ones in the second partition. The comparison rule is as follows: the modified partition is the one which owns the largest abs (a1+a2+a3+a4-b1-b2-b3-b4).

Figure 3.  The corner points utilized for comparison in the proposed method

Method 2: Disabling DBBP for 8x8 CU to reduce the worst-case complexity

Before DBBP is adopted, the worst case in decoding for a CTU is all CUs are 8x8 ones with NxN PU mode. Compared to the other sizes, 8x8 mode requires more syntax decoding (e.g. 4 mvds in NxN modes), address calculation (4mvp+4mvd in NxN modes), more memory access (each CU requires 4 predicted blocks at different positions and different interpolation-precision). However, from such above analysis, we can see DBBP owns more data access and data compensations than NxN mode, so DBBP for 8x8 CU becomes the worst case of decoding one CTU. In summary, DBBP mode is more complex than NxN mode in the following directions. 1) Two-time 2Nx2N compensations, which means two-time the data access of NxN mode; 2) Sample-level mask generation, which produces additional operations. 3) Sample-level compensation using conditional judgments from mask, which produces additional 8x8 comparing zero operations. 4) For the neighboring pixels between two segments, DBBP averages the collocated sample of two predicted blocks, which additionally takes 8x8 adding and shifting operations in the worst case. Furthermore, DBBP probably be suitable for larger CU sizes, because smaller CUs seldom have segments with different motion vectors. Therefore DBBP is proposed to be forbidden for CU sizes of 8x8 with the following syntax table change.

| if( depth_based_blk_part_flag[ nuh_layer_id ] && PartMode = = PART_2NxN && log2CbSize > MinCbLog2SizeY ) |  |
| --- | --- |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |

- Experimental results

Tests are done on HTM10.0r1. Table 3 shows Method 1 has 0.03% BD-rate savings on synthesis views.

Table 3.  The BD-rate and runtime results of the proposed method over HTM10.0r1

Table 4 shows Method 2 only leads to 0.01% BD-rate increase on synthesis views.

Table 4.  The BD-rate and runtime results of the proposed method over HTM10.0r1

- Working Draft change

Method 1

- I.8.5.7 Derivation process for a modified partitioning mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block.

- The derivation process for a depth predicted contour pattern as specified in subclause Error: Reference source not found is invoked with the sample location ( xTb, yTb ) equal to ( xCb, yCb ), and the block size nTbS equal to nCbS as inputs, and the output is a binary partition pattern wedgePattern[ x ][ y ].

The derivation process for a depth predicted contour pattern as specified in subclause Error: Reference source not found is invoked with the luma locations ( xP, yP ) equal to ( xTb, yTb ), the disparity vector mvDisp equal to MvRefinedDisp[ xTb ][ yTb ], the view identifier refViewIdx equal to RefViewIdx[ xTb ][ yTb ], the variables nPbW and nPbH, equal to nTbS, and the variable partIdc equal to 1 as inputs, and the output is the array refSamples of size (nTbS)x(nTbS) and the flag horSplitFlag..

For p in the range of 0 to 5, inclusive and i in the range of 0 to 1, inclusive, partSum[ p ][ i ] is set equal to 0.

The variable partIdc is derived as specified in the following:

partIdc = 0;

maxPartSum = 0;

Corner1 = refSamples[0][0] - refSamples[nTbS-1][ nTbS-1 ];

Corner2 = refSamples[nTbS-1][0] - refSamples[0][nTbS-1];

for(j=0; j<p; j++)  {

if(j==0 || j==4 || j==5){

u= (j==5? nTbS*3: nTbS);

d = (j==0?1:2);

pos=( u>>d);

gap = refSamples[0][pos-1] - refSamples[0][ pos]+refSamples[nTbS-1][ pos] - refSamples[nTbS-1][ pos];

partSum[j] = abs(Corner1 + Corner2 + gap);

}

else{

u= (j==3? nTbS*3: nTbS);

d = (j==1?1:2);

pos=( u>>d);

gap = refSamples[pos -1][0] - refSamples[pos][0]+refSamples[pos -1][nTbS-1] - refSamples[pos][nTbS-1];

partSum[j] = abs(Corner1 - Corner2 + gap);

}

if(matchedPartSum[j] > maxPartSum){

maxPartSum = matchedPartSum[j];

partIdc = virtualPartSizes[j];

}

}

Method 2

| if( depth_based_blk_part_flag[ nuh_layer_id ] && PartMode = = PART_2NxN && log2CbSize > MinCbLog2SizeY ) |  |
| --- | --- |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |

- Conclusion

In this proposal, two methods are proposed to reduce the complexity of DBBP. Method 1 proposes to simplify mode modification process in DBBP and Method 2 proposes to disable DBBP mode for 8x8 CUs. The experiments results show no coding efficiency loss.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, "3D-HEVC Draft Text 3," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG, JCT3V-G1001, Jan., 2014

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1

- Fabian Jäger, Jacek Konieczny and Giovanni Cordara, "CE3: Results on Depth-based Block Partitioning (DBBP)," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG, JCT3V-G0106, Jan, 2014.