| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0076 |
| --- | --- |

| Title: | Removal of chroma intra mode and cbf in depth map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Kai Zhang1, Jian-Liang Lin2, Shawmin Lei2 1MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, kai.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to remove the redundant syntax elements for depth coding, including chroma intra mode and chroma cbf flags.

# Introduction

In current 3D-HEVC, for depth map coding, the chroma sample values are meaningless and always set to 128. Therefore, the chroma intra mode and chroma cbf (coded block flag) are redundancies in depth map coding. However, in current 3D-HEVC software HTM10.0r1 [1], the chroma intra mode and cbf are still coded for depth map except for the SDC mode, which is weird. Firstly, coding the chroma intra mode (syntax element intra_chroma_pred_mode) and cbf (syntax elements cbf_cb and cbf_cr) are actually not necessary for depth map. Secondly, why is SDC mode the only mode that does not have chroma intra mode? The chroma intra mode should not be coded in any mode in depth map coding.

# Proposed method

- In light of the previously described problems, it is proposed to remove the chroma intra mode and chroma cbf coding for depth map in order to remove the syntax redundancies.

- The proposed syntax change is listed as follows (the added parts are highlighted):

| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| --- | --- |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( dim_not_present_flag[ x0 + i ][ y0 + j ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| if (!VpsDepthFlag[ layerId ] ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |

| If(( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) && !VpsDepthFlag[ layerId ] ) |  |
| --- | --- |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( (trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) && !VpsDepthFlag[ layerId ] ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |

- Experimental results

The proposed method has been implemented into HTM10.0r1. As the experimental results shown in Table 1, there is averagely 0.03% BD-rate savings on synthesis views.

Table 1.  The experimental results of the proposed method on HTM10.0r1

- Conclusion

This contribution proposes to remove the chroma intra mode and chroma cbf flags to remove the redundant syntaxes in depth map coding.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1