| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0077 |
| --- | --- |

| Title: | Motion hole filling for sub-PU level MPI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Kai Zhang1, Jian-Liang Lin2, Shawmin Lei2 1MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, kai.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to use a default motion parameter to fill the sub-PU MPI motion hole, where the default motion parameter is derived from the center sub-PU. If that motion parameter is unavailable, the MPI candidate is considered as unavailable. With the proposed method, the motion prediction for each sub-PU can be done in parallel. Experimental results reportedly show that the proposed simplification brings no coding loss.

# Introduction

In the last JCT-3V meeting, the JCT3V-G0119 [1] proposed to apply the MPI at a sub-PU level and got adopted. The sub-PU MPI derives motion parameters of each sub-PU from the motion parameters of the collocated texture block (reference block) in the same access unit. One set of temporary motion parameters are stored and updated during sub-PU MPI process. For each sub-PU, if the reference block of the current sub-PU has available motion parameters, temporary motion parameters are updated with the motion parameters of the reference block. Otherwise, if the reference block does not have available motion parameters, the temporary motion parameters are copied to the current sub-PU. That process is called the motion parameter hole filling process.

The sub-PU MPI motion hole filling process mentioned above is the same as that in sub-PU level inter-view motion prediction (IVMP) proposed in JCT3V-E0184 [2] and JCT3V-F0110 [3], and may not be friendly to parallel processing as pointed out in JCT3V-G0147 [4], because it needs to find where the last sub-PU position which has available motion parameter is.

In last meeting, the JCT3V-G0147 [4] proposed a solution to solve the problem in sub-PU IVMP motion hole filling and got adopted. However, the same problem still exists in the current sub-PU MPI.

# Proposed method

In light of the previously described problems, it is proposed to use the solution similar to JCT3V-G0147 [4] to solve the motion hole filling problem in sub-PU MPI.

To make sub-PU MPI friendly to parallel processing, one default motion parameter is generated before processing any sub-PU. For each sub-PU within the current PU, if its corresponding reference block does not contain any available motion parameter, the default motion parameter is used for the sub-PU.

The default motion parameter is set as the motion parameter of the center sub-PU. If that center sub-PU doesn’t have an available motion parameter, the MPI candidate is considered as unavailable. The default motion parameter is used for motion hole filling and pruning. In this way, the motion parameter hole filling is not dependent on the sub-PU processing order, and the motion prediction for each sub-PU can be done in parallel.

- Experimental results

The proposed method has been implemented on HTM10.0r1, experimental results in Table 1 show there is averagely 0.02% BD-rate savings on synthesis views.

Table 1.  The experimental results of the proposed method on HTM10.0r1

- Conclusion

This contribution proposed to use a default motion parameter to fill the sub-PU MPI motion hole, where the default motion parameter is derived from the center sub-PU. If that motion parameter is unavailable, the MPI candidate is considered as unavailable. The experimental results show no coding loss while the motion parameter inheritance of each sub-PU can be done in parallel.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y. Chen, H. Liu, L. Zhang, “CE2: Sub-PU based MPI,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G0119, Jan. 2014.

- J. An, K. Zhang, J.-L. Lin, S. Lei, “CE3.h related: Sub-PU level inter-view motion prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E0184, August, 2013.

- J. An, K. Zhang, J.-L. Lin, S. Lei, “3D-CE3: Sub-PU level inter-view motion prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F0110, Oct., 2013.

- Y.S. Heo, M.S. Lee, W.W. Gwun, Y.J. Lee, G.H. Park, “CE3 3D-CE1.h related: Simplification of sub-PU level temporal interview motion prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G0147, Jan., 2014.