- I.7.3.8.5.1	Intra mode extension syntax

| intra_mode_ext( x0 , y0 , log2PbSize ) { | Descriptor |
| --- | --- |
| if( log2PbSize < 6 ) |  |
| dim_not_present_flag[ x0 ][ y0 ] | ae(v) |
| if ( !dim_not_present_flag[ x0 ][ y0 ] ) |  |
| depth_intra_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ) |  |
| wedge_dir_idx[ x0 ][ y0 ] | ae(v) |
| wedge_refine_idx[ x0 ][ y0 ] | ae(v) |
| } |  |

- I.7.4.9.5.1	Intra mode extension semantics

The variable Log2MaxDmmCbSize is set equal to 5.

dim_not_present_flag[ x0 ][ y0 ] equal to 1 specifies that the depth_intra_mode_flag syntax element is not present and that intra modes with intraPredMode in the range of 0 to 34 is used for the current prediction unit. dim_not_present_flag[ x0 ][ y0 ] equal to 0 specifies that that the depth_intra_mode_flag syntax element might be present. When not present, the value of dim_not_present_flag[ x0 ][ y0 ] is inferred to be equal to 1.

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

  - DmmFlag[ x0 ][ y0 ] = !dim_not_present_flag[ x0 ][ y0 ]	(Error: Reference source not found-1)

depth_intra_mode_flag[ x0 ][ y0 ] is used to specify the depth intra mode of the current prediction unit.

The variable DepthIntraMode[ x0 ][ y0 ] is derived as specified in the following:

  - DepthIntraMode[ x0 ][ y0 ] = dim_not_present_flag[ x0 ][ y0 ] ? −1 : depth_intra_mode_flag[ x0 ][ y0 ]	(Error: Reference source not found-2)

Table Error: Reference source not found-1 specifies the value for the depth intra mode and the associated names.

Table Error: Reference source not found-1 – Specification of DepthIntraMode and associated name

| DepthIntraMode | Associated name |
| --- | --- |
| −1 | INTRA_DEP_NONE |
| 0 | INTRA_DEP_DMM_WFULL |
| 1 | INTRA_DEP_DMM_CPREDTEX |

wedge_dir_idx[ x0 ][ y0 ] specifies the wedgelet pattern list index when DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL.

wedge_refine_idx[ x0 ][ y0 ] specifies the index of the wedgelet pattern in the corresponding pattern list when DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL.

- I.8.4.4.2.7	Specification of intra prediction mode INTRA_DMM_WFULL

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = − 1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size.

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable wedgePattern[ x ][ y ] with x, y =0..nTbS − 1, specifying a binary partition pattern is derived as.

      - wedgePattern = WedgePatternTable
[ Log2( nTbS ) ][ wedge_dir_idx[ xTb ][ yTb ] ][ wedge_refine_idx[ xTb ][ yTb ] ]	(Error: Reference source not found-3)

  - The depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ xTb ][ yTb ], the sample location ( xTb, yTb ), and the transform size nTbS as inputs, and the output is assigned to predSamples[ x ][ y ].

- I.8.4.4.2.10	Specification of tables WedgePatternTable

NOTE − Tables and values resulting from the processes specified in the following are independent of any information contained in the bitstream. Therefore the derivation process described in this subclause can be carried out once as part of the initialization of the decoding process. Alternatively, the tables and values can be stored within the decoder (read-only) memory as fixed lookup tables, such that the derivation process described in this subclause does not need to be implemented in the decoder at all.

The array WedgePatternTable[ log2BlkSize ][ dirIdx ] of binary partition patterns of size (1  <<  log2BlkSize )x(1  <<  log2BlkSize), the variable NumWedgePattern[ log2BlkSize ][ dirIdx ] specifying the number of binary partition patterns in list WedgePatternTable[ log2BlkSize ][ dirIdx ] are derived as specified in the following:

- For log2BlkSize ranging from 2 to Log2MaxDmmCbSize, inclusive, the following applies:

  - Depending on log2BlkSize, the variable resShift is derived as specified in Table Error: Reference source not found−2.

Table Error: Reference source not found−2 – Specification of resShift

| log2BlkSize | resShift |
| --- | --- |
| 2,3 | 1 |
| 4 | 0 |
| Otherwise (5… Log2MaxDmmCbSize) | −1 |

  - The variable wBlkSize is set equal to ( 1  <<  ( log2BlkSize + resShift ) )

  - For wedgeOri in the range of 0 to 5, inclusive, the following ordered steps apply.

    - Depending on wedgeOri the variables xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE are derived as specified in Table Error: Reference source not found−3.

Table Error: Reference source not found−3 − Specification of xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE

| wedgeOri | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| xPosS | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 | wBlkSize − 1 |
| yPosS | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 |
| xPosE | 0 | wBlkSize − 1 | wBlkSize− 1 | 0 | 0 | 0 |
| yPosE | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | wBlkSize − 1 | 0 |
| xIncS | 1 | 0 | −1 | 0 | 1 | 0 |
| yIncS | 0 | 1 | 0 | −1 | 0 | 1 |
| xIncE | 0 | −1 | 0 | 1 | 1 | 0 |
| yIncE | 1 | 0 | −1 | 0 | 0 | 1 |

    - For m in the range of 0 to wBlkSize − 1, inclusive, the following applies:

      - For n in the range of 0 to wBlkSize − 1, inclusive,  the following applies:

        - The Wedgelet pattern generation process as specified in subclause Error: Reference source not found is invoked with patternSize being equal to ( 1  <<  log2BlkSize ), the variable resShift, variable wedgeOri, xS being equal to ( xPosS + m * xIncS ), yS being equal to ( yPosS + m * yIncS ), xE being equal to ( xPosE + n * xIncE ) and yE being equal to ( yPosE + n * yIncE ) as inputs, and the output is the binary array curWedgePattern.

        - The variable wDirIdx specifying the direction index of curWedgePattern is derived as specified in the following:

          - A variable deltaX is set equal to (( xPosE + n * xIncE ) − ( xPosS + m * xIncS )) and a variable deltaY is set equal to (( yPosE + n * yIncE ) − ( yPosS + m * yIncS )).

          - If deltaX is equal to 0 and deltaY is equal to 0, the following applies:

- wDirIdx = (( xPosS + m * xIncS )  = =  ( yPosS + m * yIncS )) ? 31 : 15

          - Otherwise (deltaX is not equal to 0 or deltaY is nor equal to 0), the following applies:

- verFlag = ( abs( deltaY ) > abs( deltaX ) ) ? 1 : 0

- if( verFlag  = =  1 ) {

- ( deltaX, deltaY ) = Swap( deltaX, deltaY )

- }

- if( deltaY  = =  0 )  &&  ( wedgeOri < 4 ) {

- deltaY = 1

- deltaX = deltaX  <<  1

- }

- lS = abs(( deltaY  <<  7 ) / deltaX)

- signFlag = (verFlag  = =  ( lS < 0 ) ? 1 : 0 ) ? 1 : 0

- angOff = ( lS < 4 ) ? 0 : ( ( lS < 14 ) ? 1 : ( ( lS < 28 ) ? 2 : ( ( lS < 44 ) ? 3 : ( ( lS < 60 ) ? 4 : ( ( lS < 76 ) ? 5 : ( ( lS < 94 ) ? 6 : ( ( lS < 115 ) ? 7 : 8 ) ) ) ) ) ) )

- wDirIdx = (( ( isVer ? 0 : 2 ) + signFlag )  <<  3 + angOff - signFlag) % 32

        - The wedgelet pattern list insertion process as specified in subclause  is invoked with log2BlkSize, the variable wDirIdx, and the binary partition pattern curWedgePattern as inputs.

- I.8.4.4.2.10.2	Wedgelet pattern list insertion process

Inputs to this process are:

- a variable log2BlkSize specifying the binary partition pattern size as ( 1  <<  log2BlkSize ),

- a variable wDirIdx specifying the direction index of the Wedgelet pattern,

- binary partition pattern wedgePattern[ x ][ y ], with x, y =0..( 1  <<  log2BlkSize ) − 1.

- The variable isValidFlag specifying whether the binary partition pattern wedgePattern is added to the list WedgePatternTable[ log2BlkSize ][ wDirIdx ] or not is set equal to 0.

- The value of isValidFlag is derived as specified by the following ordered steps.

  - For x, y = 0..( 1  <<  log2BlkSize ) − 1 the following applies:

    - When wedgePattern[ x ][ y ] is not equal to wedgePattern[ 0 ][ 0 ] the flag isValidFlag is set to 1.

  - For dirIdx in the range of 0 to 31, inclusive, the following applies:

- For k = 0..NumWedgePattern[ log2BlkSize ][ dirIdx ] − 1 the following applies:

    - The flag patIdenticalFlag is set equal to 1.

    - For x, y =0..( 1  <<  log2BlkSize )−1 the following applies:

      - When wedgePattern[ x ][ y ] is not equal to WedgePatternTable[ log2BlkSize ][ dirIdx ][ k ][ x ][ y ], patIdenticalFlag is set to 0.

    - When patIdenticalFlag is equal to 1, isValidFlag is set to 0.

  - For dirIdx in the range of 0 to 31, inclusive, the following applies:

- For k = 0..NumWedgePattern[ log2BlkSize ][ dirIdx ] − 1 the following applies:

    - The flag patInvIdenticalFlag is set to 1.

    - For x, y =0..( 1  <<  log2BlkSize ) − 1 the following applies:

      - When wedgePattern[ x ][ y ] is equal to WedgePatternTable[ log2BlkSize ][ dirIdx ][ k ][ x ][ y ], patInvIdenticalFlag is set to 0.

    - When patInvIdenticalFlag is equal to 1, isValidFlag is set to 0.

- When isValidFlag is equal to 1, the following applies:

  - The pattern WedgePatternTable[ log2BlkSize ][ wDirIdx ][ NumWedgePattern[ log2BlkSize ][ wDirIdx ] ] is set equal to wedgePattern.

  - The value of NumWedgePattern[ log2BlkSize ][ wDirIdx ] is increased by one.

- I.8.4.4.3	Segmental depth intra coding process

…

Depending on predModeIntra the array wedgePattern[ x ][ y ] with x, y =0..nTbS − 1 specifying the binary segmentation pattern is derived as follows:

- If predModeIntra is equal to INTRA_DMM_WFULL, the following applies:

    - wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ wedge_dir_idx[ xTb ][ yTb ] ][ wedge_refine_idx[ xTb ][ yTb ] ]

- Otherwise, if predModeIntra is equal to INTRA_DMM_CPREDTEX, the derivation process for a texture predicted contour pattern as specified in subclause Error: Reference source not found is invoked with the sample location ( xTb, yTb ), and the variable nTbS as inputs, and the output is the binary partition pattern wedgePattern[ x ][ y ], with x, y =0..nTbS − 1.

- Otherwise (predModeIntra is not equal to INTRA_DMM_WFULL or INTRA_DMM_CPREDTEX), the following applies:

…

- I.9.3.3.1	General

The specifications in subclause 9.3.3.1 apply with the following modifications.

- Table Error: Reference source not found-4 is appended to the end of Table 9-32.

Table Error: Reference source not found-4 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| cu_extension( ) | iv_res_pred_weight_idx | TR | cMax = 2, cRiceParam = 0 |
|  | ic_flag | FL | cMax = 1 |
|  | dbbp_flag | FL | cMax = 1 |
|  | sdc_flag | FL | cMax = 1 |
| intra_mode_ext( ) | dim_not_present_flag | FL | cMax = 1 |
|  | depth_intra_mode_flag | FL | cMax = 1 |
|  | wedge_dir_idx | FL | cMax = 5 |
|  | wedge_refine_idx | FL | cMax = wedgeDirTabIdxBits[ log2PbSize ] (defined in Table Error: Reference source not found-5 –Values ofeFulldxBiTabIdxBits[ log2PUSize ]) |
|  | depth_dc_flag | FL | cMax = 1 |
|  | depth_dc_abs | UEG0 | [Ed. (GT) To be specified] |
|  | depth_dc_sign_flag | FL | cMax = 1 |

Table Error: Reference source not found-5 –Values ofeFulldxBiTabIdxBits[ log2PUSize ]

| Initialization variable |  | wedgeDirTabIdxBits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| log2PbSize | wedge_dir_idx % 16 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 2 | Value | 0 | 0 | 1 | 3 | 0 | 3 | 2 | 0 | 0 | 1 | 3 | 0 | 2 | 2 | 1 | 3 |
| 3 | Value | 3 | 4 | 5 | 6 | 5 | 5 | 6 | 5 | 4 | 5 | 6 | 5 | 5 | 6 | 5 | 5 |
| > 3 | Value | 4 | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- I.9.3.4.2.1	General

The specifications in subclause 9.3.4.2.1 apply with the following modifications:

- Table Error: Reference source not found-6 is appended to the end of Table 9-37.

Table Error: Reference source not found-6 –Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| wedge_dir_idx | bypass | bypass | bypass | bypass | bypass | bypass |
| wedge_refine_idx | bypass | bypass | bypass | bypass | bypass | bypass |
| depth_dc_flag | 0 | na | na | na | na | na |
| depth_dc_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_sign_flag | bypass | 0 | 0 | 0 | 0 | 0 |
| iv_res_pred_weight_idx | 0, 1 Error: Reference source not found | 2 | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| dbbp_flag | 0 | na | na | na | na | na |
| depth_intra_mode_flag | 0 | na | na | na | na | na |
| sdc_flag | 0 | na | na | na | na | na |
| dim_not_present_flag | 0, 1,2 Error: Reference source not found | na | na | na | na | na |