| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0078 |
| --- | --- |

| Title: | CE3: Results on DMM1 signaling using intra direction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Merkle Karsten Müller | Email: | philipp.merkle@hhi.fraunhofer.de karsten.mueller@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the results on the tool under test in CE3 for studying the reduction of context coded bins for syntax element wedge_full_tab_idx. The proposed method modifies the signaling based based on the 32 directions of the angular intra mode, using a fixed length prefix and a variable length suffix, using bypass coding for all bins.

# Introduction

Test 2 of CE3 is about the tool test on reduction of context coded bins for syntax element wedge_full_tab_idx. Currently the syntax element wedge_full_tab_idx (representing an index in the wedgelet pattern list) is binarized by fixed length coding using 7 to 11 context coded bins depending on the block size. According to proposals JCT3V-G0062 and JCT3V-G0108, the purpose of this tool test is studying possible simplifications by reducing the number of context coded bins.

# Proposed method

The proposed method splits the information in wedge_full_tab_idx into a prefix and a suffix. The prefix wedge_dir_idx represents the direction of the wedgelet separation line by an index in the range [0..32], using 5 bypass coded bins. The suffix wedge_refine_idx represents refinement information for the position and slope of the wedgelet line by a list index with the range depending on the block size and the prefix value, using 0 to 6 bypass coded bins.

Table 1. Binarization comparison

| block size | 4×4 |  | 8×8 |  | 16×16 |  | 32×32 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bins | number | type | number | type | number | type | number | type |
| HTM-10.0r1 wedge_full_tab_idx | 7 | CABAC | 10 | CABAC | 11 | CABAC | 11 | CABAC |
| JCT3V-H0078 wedge_dir_idx wedge_refine_idx | 5 0~3 | bypass bypass | 5 3~6 | bypass bypass | 5 4~6 | bypass bypass | 5 4~6 | bypass bypass |

Table 1 and figure 1 highlight that the throughput is significantly improved by the proposed method: first, because all context coded bins are replaced by bypass coded bins and second, because a high degree of parallelization is supported, requiring only one or two instead of 7 to 11 cycles.

Figure 1. Throughput comparison

# Experimental results

According to the CE3 description in JCT3V-G1103 [1] and the common test conditions in JCT3V-G1100 [2], the proposed method is evaluated for random access (CTC) and all-intra configuration with HTM-10.0r1.

Table 2. Result for 3-view scenario under CTC

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.00% | 99.3% | 114.2% | 117.0% |
| Kendo | 0.0% | 0.0% | -0.06% | 99.8% | 114.8% | 101.7% |
| Newspapercc | 0.0% | 0.0% | -0.14% | 98.8% | 108.5% | 108.4% |
| GhostTownFly | 0.0% | 0.0% | -0.03% | 100.1% | 95.5% | 99.9% |
| PoznanHall2 | -0.1% | -0.1% | -0.18% | 100.9% | 108.0% | 101.7% |
| PoznanStreet | 0.0% | 0.0% | -0.03% | 99.2% | 93.3% | 98.7% |
| UndoDancer | 0.0% | 0.0% | 0.02% | 99.9% | 92.1% | 98.8% |
| Shark | 0.0% | 0.0% | -0.03% | 100.0% | 99.9% | 100.4% |
| 1024x768 | 0.0% | 0.0% | -0.07% | 99.3% | 112.5% | 109.0% |
| 1920x1088 | 0.0% | 0.0% | -0.05% | 100.0% | 97.8% | 99.9% |
| average | 0.0% | 0.0% | -0.06% | 99.7% | 103.3% | 103.3% |

Table 3. Result for 3-view scenario under all-intra configuration

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.05% | 99.9% | 101.8% | 100.1% |
| Kendo | 0.0% | 0.0% | -0.08% | 99.7% | 96.5% | 98.9% |
| Newspapercc | 0.0% | 0.0% | -0.08% | 100.0% | 99.9% | 99.9% |
| GhostTownFly | 0.0% | 0.0% | -0.05% | 99.5% | 98.5% | 96.5% |
| PoznanHall2 | 0.0% | 0.0% | -0.12% | 100.5% | 103.1% | 104.2% |
| PoznanStreet | 0.0% | 0.0% | -0.03% | 99.7% | 116.7% | 116.1% |
| UndoDancer | 0.0% | 0.0% | -0.02% | 100.0% | 98.2% | 108.1% |
| Shark | 0.0% | 0.0% | 0.01% | 99.3% | 104.5% | 98.6% |
| 1024x768 | 0.0% | 0.0% | -0.07% | 99.9% | 99.4% | 99.7% |
| 1920x1088 | 0.0% | 0.0% | -0.04% | 99.8% | 104.2% | 104.7% |
| average | 0.0% | 0.0% | -0.05% | 99.8% | 102.4% | 102.8% |

# Draft specification

The proposed method requires changing the specification in Annex H of JCT3V-G1001 [3]. The specification text can be found in the attached document “JCT3V-H0078_spec.doc”.

# Cross check

The cross check of the proposed method was done by MediaTek.

# Conclusion

The results show that the proposed binarization scheme for DMM1 Wedgelet segmentation pattern information leads to a consistent, small coding gain. At the same time the proposed method reduces the number of CABAC coded bins to zero, thus improving the throughput and allowing for a high degree of parallelization. In summary, the proposal combines the benefits of improving throughput and coding efficiency and is therefore recommended for adoption to 3D-HEVC.

# References

- Y.-L. Chang, H. Liu, “Description of Core Experiment 3 (CE3) on Entropy Coding Simplifications,” JCT3V-G1103, San Jose, USA, January 2014.

- K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, San Jose, USA, January 2014.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” JCT3V-G1001, San Jose, USA, January 2014.

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).