| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0183 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0082 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: SEI message cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry Adarsh K. Ramasubramonian 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes some clean-ups on semantics of SEI messages specified in HEVC for their use in SHVC/MV-HEVC. These changes include 1) definitions of output operation point and picture unit, 2) some restrictions on order of SEI messages, 3) some clarifications on the applicable operation points or layers of SEI messages, 4) some restrictions on nesting of SEI messages as well as some updaates to the semantics of the scalable nesting SEI message, 5) some restrictions on the values of nuh_layer_id and TemporalId of SEI NAL units, 6) some udpates to the restrictions on the presence of SEI messages between two VCL NAL units, 7) some udpates to the restrictions on repetition of SEI messages, 8) some updates to the semantics of a list of specific SEI messages, such as the display orientation SEI message, the recovery point SEI message, and the region refresh information SEI message. Some of the changes are asserted also apply to HEVC version 1 and thus could be considered as corrigendum items.

# Introduction

The HEVC SEI messages are currently included in the SHVC/MV-HEVC specifications with some changes that are needed for the SEI messages to be applied to multi-layer bitstreams. Several issues are associated with the current HEVC and SHVC/MV-HEVC specifications on SEI messages such that the semantics of a list of SEI messages need to be updated. Following are a description of some of the issues.

## Issues related to order of SEI messages within an access unit

- An SEI NAL unit containing an SEI message that applies to a layer with nuh_layer_id equal to layerIdA can follow a VCL NAL unit of a picture with nuh_layer_id greater than layerIdA within the access unit (AU). Having an SEI NAL unit together in the bitstream with the pictures in the layers it applies is more beneficial, e.g. in minimizing storage or transmission overhead when storing and transmitting associated NAL units together.

- A prefix SEI message is currently required to be present preceding the first VCL NAL unit of an AU when there is a prefix SEI message of the same type between two VCL NAL units of the AU. However, this should be changed to be picture-based instead of AU-based.

- A suffix SEI message is currently required to be present succeeding the last VCL NAL unit of an AU when there is a suffix SEI message of the same type between two VCL NAL units of the AU. However, this should be changed to be picture-based instead of AU-based.

- Currently, the amount allowed for repeition of an SEI message is specified per AU. However, this should be changed to be picture-based instead of AU-based.

## Issues related to nesting of SEI messages and semantics of the scalable nesting SEI message

- If the scalable nesting SEI message has bitstream_subset_flag equal to 1 and a nested SEI message that has payloadType equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, 132, or 134 (i.e. one of the SEI messages that have payloadType not equal to any of 0, 1, 4, 5, 130, and 133), the semantics of the nested SEI message are not clear.

- When bitstream_subset_flag is equal to 1 and none of the layer sets specified by the VPS includes and only includes the layers having nuh_layer_id values in the range of 0 to nuh_layer_id of the current SEI NAL unit, inclusive, it does not make sense to have default_op_flag equal to 1, which is currently allowed.

- When nesting_op_flag is equal to 0 and all_layers_flag is equal 1, the value of maxTemporalId[ 0 ] is unspecified.

- When a nested SEI message has payloadType equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, 132, or 134 (i.e. one of the SEI messages that have payloadType not equal to any of 0, 1, 4, 5, 130, and 133) and applies to a set of layers, the set of layers may be associated with a value of maxTemporalId[ i ] that is less than the greatest value of TemporalId in the bitstream. However, the current semantics of these SEI messages are described wtihout considering sub-layers, and are consequently inconsistent with the semantics of the scalable nesting SEI message.

- When bitstream_subset_flag is equal to 1 and nesting_op_flag is equal to 0, it is allowed that nestingLayeridList[ 0 ] does not correspond to a layer set specified by the VPS, which does not make sense.

## Issues related to applicable operation points or layers and persistency scope of SEI messages

- When the buffering period SEI message, picture timing SEI message, and decoding unit information SEI messsage are nested, it is not clear what bitstream the SEI message apply to.

- When an SEI message that is not one of a buffering period SEI message, a picture timing SEI message, or a decoding unit information SEI message is nested, it is not clear what layers the SEI message applies to. For example, when such an SEI message is applied to more than one list of layers, does the SEI message apply to each list of layers independently or does the SEI message apply to all the layers among all the lists of layers specified in the scalable nesting SEI message?

- The persistence scope of the pan-scan rectangle SEI message, the progressive refinement segment start SEI message, the film grain characteristics SEI message, the tone mapping information SEI message, the frame packing arrangement SEI message, and display orientation SEI message are not clear.

- The set of applicable layers of a decoded picture hash SEI message is not clearly specified.

- The set of applicable layers of an active parameter sets SEI message is not clearly specified.

- An active parameter sets SEI message may be nested, but that does not make sense as it applies to all layers.

- When frame_field_info_present_flag is equal to 1 for a picture timing information SEI message, nested or non-nested, the set of applicable layers is not clearly specified for the frame-field information carried in the syntax elements pic_struct, source_scan_type, and duplicate_flag.

## Issues related to TemporalId and nuh_layer_id values of SEI messages

- It is unclear what the value of TemporalId should be for the SEI NAL unit containing a non-nested decoded picture hash SEI message.

- It is unclear what the value of nuh_layer_id should be for the SEI NAL unit containing a non-nested buffering period, picture timing, or decoding unit information SEI message should be.

- When a non-nested SEI message has payloadType equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, 132, or 134 (i.e. one of the SEI messages that have payloadType not equal to any of 0, 1, 129, 130, and 133), it is not clear what the value of nuh_layer_id should be for the SEI NAL unit containing the non-nested SEI message.

## Other issues

- The semantics of the buffering period SEI message do not consider the value of discardable_flag of pictures.

- The semantics of the temporal sub-layer zero index SEI message do not consider the value of discardable_flag of pictures.

- When a layer is not a target output layer in any existing output layer set, the presense of display orientation SEI messages is redundant.

- Currently, in random access event, when decoding starts from an access unit that contains a recovery point SEI message, the value of prevPicOrderCnt[ nuh_layer_id ] is simply set equal to 0. This creates a problem when there is a picture picA that is in the same layer as the picture associated with the recovery point SEI message and follows it in decoding order has a signalled poc_msb_val. In such case, the derivation of POC of picA would be unclear or incorrect.

- At the last meeting, the semantics of the recovery point SEI message have been updated to be applicable to SHVC/MV-HEVC, including a differentiation on when the recovery point SEI message indicates a random access capability or a layer up-switching capability. Similar changes should be made for the region refresh information SEI message, which is dependent on an associated recovery point SEI message.

# Proposal

A summary of the proposed changes is given below. The proposed changes themselves are provided in the attachment, with changes marked in relative to the latest SHVC draft text.

## Solutions for issues related to order of SEI messages within an access unit.

- An SEI NAL unit containing an SEI message that applies to a layer with nuh_layer_id equal to layerIdA is disallowed to follow any VCL NAL unit as well as its associated non-VCL NAL units of a picture with nuh_layer_id greater than layerIdA within the AU.

- A prefix SEI message that applies to a layer layerA containing a picture is required to be present preceding the first VCL NAL unit of the picture when there is a prefix SEI message that is of the same type and applies to the layer layerA present between two VCL NAL units of the picture.

- A suffix SEI message that applies to a layer layerA containing a picture is required to be present succeeding the last VCL NAL unit of the picture when there is a suffix SEI message that is of the same type and applies to the layer layerA present between two VCL NAL units of the picture.

- The amount allowed for repeition of an SEI message is specified per picture unit, which is defined to contain the VCL NAL units of a coded picture and their associated non-VCL NAL units.

## Solutions for issues related to syntax and semantics specific to scalable nesting SEI message:

- When a scalable nesting SEI message contains an SEI message that has payloadType equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, 132, or 134 (i.e. one of the SEI messages that have payloadType not equal to any of 0, 1, 4, 5, 130, and 133), the value of bitstream_subset_flag of the scalable nesting SEI message is required to be equal to 0.

- When bitstream_subset_flag is equal to 1 and none of the layer sets specified by the VPS includes and only includes the layers having nuh_layer_id values in the range of 0 to nuh_layer_id of the current SEI NAL unit, inclusive, the value of default_op_flag is required to be equal to 0.

- When nesting_op_flag is equal to 0 and all_layers_flag is equal 1, maxTemporalId[ 0 ] is set equal to 7.

- When a nested SEI message has payloadType equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, 132, or 134 (i.e. one of the SEI messages that have payloadType not equal to any of 0, 1, 4, 5, 130, and 133), the SEI NAL unit containing the scalable nesting SEI message is required to have TemporalId equal to 0 and maxTemporalId[ i ] for all i to be equal to 7.

- When bitstream_subset_flag is equal to 1 and nesting_op_flag is equal to 0, nestingLayeridList[ 0 ] is required to include and only include the nuh_layer_id values of one of the layer sets specified by the VPS.

## Solutions for issues related to applicable operation points or layers and persistency scope of SEI messages

- On the buffering period SEI message, the picture timing SEI message, and the decoding unit information SEI message, clarified that the semantics apply independently to each particular operation point of the set of operation points to which the SEI message applies, and the terms "access unit" and "CVS" in the semantics apply to the bitstream BitstreamToDecode that is the sub-bitstream of the particular operation point.

- On other SEI messages that are not buffering period SEI message, picture timing SEI message, or decoding unit information SEI message, clarified that the semantics apply independently to each particular layer of the layers to which the SEI message applies.

- On the pan-scan rectangle SEI message, the progressive refinement segment start SEI message, the film grain characteristics SEI message, the tone mapping information SEI message, the frame packing arrangement SEI message, and display orientation SEI message, further clarified the semantics of the syntax elements related to the persistence scope to be layer specific.

- The set of applicable layers of a decoded picture hash SEI message, which cannot be nested, is specified to the layer with nuh_layer_id equal to the nuh_layer_id of the SEI NAL unit containing the SEI message.

- An active parameter sets SEI message, which cannot be nested, applies to all layers in the bitstream.

- An active parameter sets SEI message is disallowed to be nested in a scalable nesting SEI message.

- When frame_field_info_present_flag is equal to 1 for a picture timing information SEI message, nested or non-nested, the frame-field information carried in the syntax elements pic_struct, source_scan_type, and duplicate_flag applies to all the layers in all the operation points that the picture timing SEI message applies to.

## Solutions for issues related TemporalId and nuh_layer_id values of SEI messages:

- The value of TemporalId for the SEI NAL unit containing a non-nested decoded picture hash SEI message is required to be equal to the TemporalId of the access unit containing the SEI NAL unit.

- The value of nuh_layer_id for the SEI NAL unit containing a non-nested buffering period, picture timing, or decoding unit information SEI message is required to be equal to 0.

- When a non-nested SEI message has payloadType equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, 132, or 134 (i.e. one of the SEI messages that have payloadType not equal to any of 0, 1, 129, 130, and 133), the value of nuh_layer_id for the SEI NAL unit containing the non-nested SEI message is required to be equal to the nuh_layer_id of the SEI NAL unit's associated VCL NAL unit.

## Solutions for other issues

- On the buffering period SEI message, further clarified the presence condition of buffering period SEI messages to further consider the value of discardable_flag.

- On the temporal sub-layer zero index SEI message, further clarified the semantics to consider the value of discardable_flag.

- On the display orientation SEI message, addition of a constraint such that when a layer is not indicated as output layer in any output layer set, there shall be no display orientation SEI message applicable to the current layer.

- On the recovery point SEI message, addition of constraints to the presence of recovery point SEI messages such that the POC value would be derived correctly for pictures when the decoder starts decoding a bitstream or a layer starting at an access unit that contains a recovery point SEI message even when the POC MSB value is signalled for some pictures.

- The following changes are proposed for the region refresh information SEI message:

  - Define a list of target layer, so called targetLayerIdList, to which a region refresh information SEI applies to.

  - Specifies the association / relationship between the region refresh information SEI message and the recovery point SEI message. The association can be defined as follows: the region refresh SEI message is associated with a recovery point SEI message that applies to targetLayerIdList.

  - Update the prediction constraints such that the refreshed regions also consider inter-layer prediction from reference layer pictures.

# Discussion

According to the current SHVC/MV-HEVC specifications, particularly the syntaxes and semantics of the picture timing SEI message and the scalable nesting SEI message, when there is more than one layer in a layer set, within an AU, the values of the syntax elements pic_struct, source_scan_type, and duplicate_flag for all pictures in the AU have to be the same, restrictively. This is due to that:

- The field-frame information and the picture timing information are both included in the picture timing SEI message.

- It is specified that, when a picture timing SEI message is nested, it shall apply to one or more operation point, and a non-nested timing SEI message applies to the operation point corresponding to the entire bitstream. In other words, the field-frame information cannot be nested or non-nested in a way that different information can apply to different layers.

That means, for each layer set, all pictures in an AU must be either all frame pictures or all field pictures. Is this intended? If not, what to do with it?

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).