| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0083 |
| --- | --- |

| Title: | CE1 related: Bug-fix and clean ups on merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gun Bang, *Young Su Heo, *Gwang Hoon Park, Gwang Soon Lee, Namho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-6791 {gbang, gslee, namho} @etri.re.kr +82-31-201-3680 {* medist, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This report presents a bug-fix and clean ups on merging candidate list construction. In the bug-fix, pruning process of disparity inter-view merging candidate (IvDC) was corrected. In addition, source code was cleaned up to aligned with WD.

# Description

## Bug-fix for IvDC pruning

The current WD specifies that IvDC is pruned and inserted to the merging candidate list as shown in Table 1.

Table 1. IvDC pruning on WD (JCT3V-G1001)

if( availableFlagIvDC && ( !availableFlagA1 ¦ ¦ differentMotion( A1, IvDC ) ) && ( !availableFlagB1 ¦ ¦ differentMotion( B1, IvDC ) ) && ( i < ( 5 + NumExtraMergeCand ) ) ) extMergeCandList[ i++ ] = IvDC (I-108)

However, the current HTM software could be interpreted as the following Table 2. So, the IvDC candidate is not compared to the left (A1) or above (B1) candidate in the current HTM software.

Table 2. IvDC pruning on current HTM 10.0 software

if( availableFlagIvDC && ( !availableFlagA1 ¦ ¦ differentMotion( A1, extMergeCandList[ i ] ) ) && ( !availableFlagB1 ¦ ¦ differentMotion( B1, extMergeCandList[ i ] ) ) && ( i < ( 5 + NumExtraMergeCand ) ) ) extMergeCandList[ i++ ] = IvDC (I-108)

## Clean-up HTM software for merge candidate list construction

The right-shift function was removed in the current HTM source code to improve the readability and reusability in the future development of 3D-HEVC. The cleaned-up merging candidate list construction part on the HTM software is confirmed to align to the WD. Table 5 shows the bug-fixed and clean-up result compared to the anchor.

## Revision 1: Analysis for the revised software

While this meeting, there was a comment on this proposal that there needs the independent simulations for bug-fix and clean-up. So, two additional simulation was performed.

Table 3 is a results for only cleaned up software. The results are same to the anchor.

Table 3. Results of cleaned up HTM 10.0r1 (vs. HTM 10.0r1)

Table 4 is a results for bug-fixed based on cleaned up software.

Table 4. Results of bug-fix + cleaned up HTM 10.0r1 (vs. HTM 10.0r1)

The results of Table 4 is slight different from results of Table 5 which is combined results for both bug-fix and clean up.

The reason for difference is there was some difference on implementation of searching spatial neighboring disparity vector while deriving IvDCShift candidate.

The current WD specifies Derivation process for the shifted disparity merging candidate as following:

          - Derivation process for the shifted disparity merging candidate

…

The variable availableFlagIvDCShift is set equal to 0 and for i in the range of 0 to numMergeCand - 1, inclusive, the following applies:

...

However, initial software of this proposal performs as following:

          - Derivation process for the shifted disparity merging candidate

…

The variable availableFlagIvDCShift is set equal to 0 and for i in the range of 0 to k, inclusive, the following applies:

…

The initial software of H0083 utilizes an irregular number k. Therefore, the spatial neighboring disparity vectors are generally derived from only A1(left) or B1(upper). It is similar to use smaller loop range for “The variable availableFlagIvDCShift is set equal to 0 and for i in the range of 0 to numMergeCand - 1, inclusive, the following applies” on I.8.5.3.2.15. For the anchor, the spatial neighboring disparity vector can be derived from all available spatial neighboring blocks which are in the merging candidate list. However, in spite of such missing implementation, the results of Table 5 is slight better than Table 4.

# Experimental result

In result, the minor gain is shown since IvDC pruning is fixed in the proposed HTM software.

Table 5. Results of bug-fixed HTM 10.0r1 (vs. HTM 10.0r1) (There was wrong implementation on derivation of shifted disparity vector merging candidate)

# Conclusion

In conclusion, there is needed to a further study on derivation process for the shifted disparity-merging candidate, because a possibility on simplification of the shifted disparity-merging candidate is detected.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).