| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0084 |
| --- | --- |

| Title: | 3D-CE3: Simplification on CABAC contexts for the syntax related delta DC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Tzu-Der Chuang, Yu-Lin Chang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, peter.chuang, yulin.chang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the CE 3 results of simplifications on CABAC for the syntax related to the signaling of delta DC. The proposed simplifications first simplify the parsing process of the delta DC flag (depth_dc_flag) by reducing the number of context models from two to one. As for the syntax of absolute DC value (depth_dc_abs), in current 3D-HEVC, the prefix codeword bins (13 bins for the worst cases) are context coded by sharing one context. In this contribution, we also propose to simplify the CABAC of depth_dc_abs by only context coding the first bin with single context and bypass coding the remaining bins. Experimental results reportedly show that the simplifications of depth_dc_flag and depth_dc_abs cause 0.0% coding and 0.1% coding performance drop, respectively.

# Introduction

Table 1 summarizes the parsing process and the number of context coded bins for the syntax related to the signaling of delta DC in current 3D-HEVC [1]. To further improve the throughput of CABAC processing in 3D-HEVC and reduce the number of context coded bins, in this contribution, we propose to the reduce the context coded bin as well as reduce the number of context models for the coding of the delta DC.

Table 1.  The number of contexts and contexts coded bins for the syntax related to delta DC in 3D-HEVC.

As shown in Table 1, the number of the bins for coding the absolute value of delta DC could be very long (larger than 25) and the prefix codeword bins are context coded (up to 13 bins for the worst case), thus it accounts for a high percentage of the overall context coded bins in this case. In context-based adaptive binary arithmetic coding (CABAC), it is difficult to parallelize context coded bins. To improve the throughput of the CABAC, the number of context coded bins should be reduced. Moreover, for the hardware design, it is also important to reduce the worst case of the number of the context coded bins.

# Proposed Method

In this proposal, to reduce the number of the context coded bins as well as the context models, we propose two simplifications.

Firstly, the flag syntax depth_dc_flag are coded with only one context instead of two contexts.

Secondly, only the first bin of the codeword for the syntax of depth_dc_abs is context coded and the remaining bins are bypass coded.

Table 2 summarizes the number of contexts and contexts coded bins for the syntax with these two proposed simplifications. As shown in Table 2, the context coded bins for the worst case has been reduced from 14 to 2 and the context models is also reduced from 3 to 2.

Table 2. The number of contexts and contexts coded bins for the syntax related to inter SDC, intra SDC and DMM in the simplified scheme.

# Experimental results

The proposed schemes are integrated into HTM-10.0r1 [2], and the tests are conducted under the common test conditions (CTC) [3]. The anchor is HTM-10.0r1. The results for the simplification on the flag depth_dc_flag are shown in Table 3. Experimental results show that the proposed simplification on the flag depth_dc_flag introduces no coding efficiency loss while the number of utilized context models is reduced from two to one.

The results for the simplification on the absolute delta DC syntax depth_dc_abs are shown in Table 4. Experimental results show that the proposed simplification on the depth_dc_abs introduces 0.1% coding efficiency loss under CTC conditions. Compared to the anchor, the simplifications could significantly reduce the context coded bins for coding delta DC from 13 context coded bins to only 1 context coded bin.

Table 3.  The results of context coding of “depth_dc_flag” with one context

Table 4.  The results of bypass coding of “depth_dc_abs”

# Conclusion

In this contribution, it is proposed to simplify the CABAC coding of the syntax related to delta DC. The experimental results show that limited coding loss is introduced by the proposed simplifications, while the number of contexts and contexts coded bins could be significantly reduced.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1001, Jan., 2014.

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan., 2014.