| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0086 |
| --- | --- |

| Title: | Low-latency illumination compensation (IC) encoding algorithm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The current HTM encoder decides the enabling of illumination compensation (IC) flag at the slice-level by examining original sample differences between the current picture and the associated inter-view reference picture which results in one-picture latency and is not acceptable for the low-delay applications. In this contribution, a modified IC encoding algorithm with low latency is proposed by utilizing only the information of previously decoded pictures without accessing samples of the current picture and the inter-view reference picture. The simulation results reportedly show that the modified IC encoding algorithm introduces no coding loss while low-latency encoding is achieved for IC encoding.

# Introduction

In the current HTM encoder, to decide whether the illumination compensation (IC) is enabled for coding current slice, it has to do a statistic over the samples of current picture and the inter-view reference picture which introduces a latency of one-picture processing.

To be specific, in the current HTM encoder, it will first check if there is any inter-view reference picture in the reference picture list of current slice. If there is no inter-view reference picture in the reference list, the IC will be turned off for current slice. If at least one inter-view reference picture exists in the reference list, as illustrated in Table 1, it will derive two histograms of samples over the current picture and over the inter-view reference picture. After the two histograms are derived, a summation of the absolute differences between each entry of the two histograms is calculated. If the summation of difference value is larger than a predefined threshold, IC is enabled for coding current slice. Otherwise, IC is disabled for coding current slice. As described above, the current IC encoding scheme introduces one-picture processing latency and is not acceptable for the low-delay applications.

Table 1.  The encoding scheme to determine the slice-level IC enabling in HTM-10.0 (THRESHOLD is set equal to 0.05* picture height * picture width)

for (each coding slices in coding order)

{

The variable sliceEnableIC is set equal to false;

Derive interViewRefPic as the first inter-view reference using the following steps:

for (i=0; i<Max_Ref_Idx_L0; i++)

{

if(ViewIdx of reference picture with index i is not equal to the ViewIdx of curren picture)

{

Set interViewRefPic as the reference picture with index i;

Break;

}

}

Derive pixel samples histogram for the current picture and the inter-view reference picture:

for( i=0; i<Pic_Height; i++)

{

for( j=0; j<Pic_Width; j++)

{

CurrHist[CurrPic[i* Pic_Width+j]]++;

InterViewHist[interViewRefPic [i* Pic_Width+j]]++;

}

}

Initial the sum of absolute difference value SAD_Hist to 0;

for( i=0; i<MaxPixelValue; i++)

{

SAD_Hist += abs(CurrHist[i]- InterViewHist[i]);

}

if (SAD_Hist > THRESHOLD)

{

sliceEnableIC = true;

}

else

{

sliceEnableIC = false;

}

Set slice_ic_enable_flag = sliceEnableIC;

Encoding CUs within current coding

}

# Proposed method

The basic concept of the proposed method is to decide whether IC is enabled or disabled for current slice according to the information of the previously decoded slice without accessing samples of the current picture and the inter-view reference picture. In the proposed scheme, the enabling of IC for coding current slice depends on CU-level IC enabling ratio in the previously decoded slice. When the IC enabling ratio is larger than a predefined threshold, IC is enabled for coding current slice; otherwise IC is disabled for coding current slice. It is noted that the previously decoded slice is selected as the one with lower temporal layer than current slice. Besides, IC is always enabled for coding the slices with temporal layer equal to 0 to ensure IC could be enabled periodically. The detailed encoding algorithm is illustrated in TABLE 2. The CU-level IC enabling ratio of previously decoded slice is already available when coding current slice. Therefore, the proposed IC encoding scheme could achieve very low latency compared to the encoding algorithm in the current HTM encoder.

Table 2.  The proposed encoding scheme to determine the slice-level IC enabling

(THRESHOLD is set equal to 0.1 in this proposal)

for (each coding slices in coding order)

{

The variable sliceEnableIC is set equal to false;

if( TemporalId of current coding slice is equal to 0)

{

sliceEnableIC = true;

}

else

{

If ( ICEnableRatio[(TemporalId of current coding slice )-1] > THRESHOLD)

{

sliceEnableIC = true;

}

else

{

sliceEnableIC = false;

}

}

Set slice_ic_enable_flag = sliceEnableIC;

Encoding CUs within current coding and do the statistics of IC enabling ratio

Update the ICEnableRatio[TemporalId of current coding slice] with the ratio of current slice

}

# Experimental results

The proposed scheme is implemented on HTM-10.0r1 [2]. The tests are conducted under the common test conditions [3], and the anchor is HTM-10.0r1. As shown in Table 3, the proposed modification brings no coding loss while low-latency encoding is achieved for IC encoding.

Table 3.  The results of the proposed modifications

# Conclusion

In this contribution, a low-latency encoding scheme is proposed for illumination compensation (IC). Current HTM encoder needs to access the samples of current picture and the inter-view reference picture to determine the slice-level IC flag, which results in latency of one-picture processing. In this contribution, it is proposed to decide the slice-level IC flag only depending on the information of previously decoded slice. The simulation results reportedly show that the proposed modification brings no coding loss while the slice-level IC enabling could be determined with low latency.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1001, Jan., 2014.

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan., 2014.