| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0087 |
| --- | --- |

| Title: | Single depth intra mode for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an intra coding mode termed as single depth mode is proposed to code the smooth area within a depth map. A CU-level flag is signaled to indicate whether a CU is coded as single depth mode. When a CU is coded as single depth mode, it is reconstructed by filling this CU with one single depth value. Additionally, an index is transmitted to select one out of several available depth sample candidates derived from the spatial neighboring pixels. Compared to the anchor HTM-10.0r1, the proposed method reportedly shows overall 0.3% and 0.2% BD-rate savings under common test conditions and all intra conditions, respectively.

# Introduction

It is found that depth map data show several signal characteristics different from natural video data. The most intuitive one is that depth map contains a lot of smooth area with similar sample values. For most of the cases, the samples within the smooth area share a same depth value.

In this contribution, a coding mode termed as “Single Depth Mode” is proposed to efficiently code the smooth area within a picture. The concept of single depth mode is to simply reconstruct the current coding unit (CU) as a smooth area with a single depth sample value. With the help of this new coding mode, the smooth area within a depth map could be coded more efficiently through more concise syntax.

# Proposed method

In the proposed scheme, a flag is signaled in the slice header to indicate the enabling of the single depth mode in current slice. When the slice-level flag is enabled, one CU-level flag is further signaled to indicate the enabling of the single depth mode for each coding CU.

To reconstruct a CU coded as single depth mode, a sample candidate list is first constructed by inserting the sample candidates derived from the neighboring samples of current CU in a predefined order into the candidate list. As shown in Figure 1 (each square represents a sample), the spatial neighboring samples are those five reconstructed samples around the current CU and are inserted into the candidate list in the order: (Am, Bn, B0, A0 and D). In the proposed scheme, the size of the sample candidate list is fixed to 5. If empty entry still exists in the candidate list after the derivation of candidates from the spatial neighboring samples, offset candidates which are derived by adding offsets to first available sample candidate will be used to fill in those empty entries. After the sample candidate list is constructed, a candidate index is signaled in the bitstream to indicate which sample candidate is selected to fill the current CU. It is noted that, in the proposed scheme, no residual signal is transmitted for the single depth mode.

Figure 1. The spatial sample candidates used to construct the sample candidate list.

# Simulation results

Experiments were conducted based on HTM-10.0r1 [1]. All the results were simulated according to the common test conditions (CTC) and all intra test conditions (All-intra) defined in [2]. The anchor is HTM-10.0r1.

Table 1 and Table 2 show the results under CTC and All-intra, respectively. The results show that the proposed single depth mode brings 0.3% and 0.2% overall coding gain under CTC and All-intra, respectively. Since the reconstruction process of the proposed single depth mode is simple, the decoder could quickly decode and reconstruct a CU coded as single depth mode. As can be seen in Table 1 and Table 2, the decoding time is reduced by 2% to 3% compared to the anchor.

Table 1.  The results of the proposed single depth mode under CTC

Table 2.  The results of the proposed single depth mode under All-intra

# Conclusions

In this contribution, an intra coding mode termed as single depth mode is proposed to efficiently code the smooth area within a picture. The simulation results reportedly show that the proposed method introduces 0.3% and 0.2% under CTC and All-intra conditions, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan., 2014.