| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0088 |
| --- | --- |

| Title: | MV-HEVC: A virtual collocated picture for temporal motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

A virtual collocated picture (VCP) for temporal motion vector prediction (TMVP) is proposed to improve the coding performance of MV-HEVC. By constructing a VCP based on the motion information from the inter-view reference picture and two auxiliary temporal reference picture, inter-view motion prediction can be introduced into MV-HEVC without changing any CU-level syntax or procedure. Experimental results reportedly show that the proposed method can achieve coding gain 2.5%, 2.2% and 1.3% for V1, V2 and all videos respectively.

# Introduction

MV-HEVC [1] and 3D-HEVC [2] are both developed by JCT-3V for multi-view coding. MV-HEVC tries to manipulating multi-view coding by a high-level-oriented architecture which reuses the CU level syntax and procedure of HEVC. Such a design makes MV-HEVC very simple and easy to be implemented. However, the exclusion of CU-level methods prohibits MV-HEVC from several powerful coding tools such as inter-view motion prediction (IVMP) adopted in 3D-HEVC. As a result, there is a significant coding performance gap between MV-HEVC and 3D-HEVC.

# Proposed method

A virtual collocated picture (VCP) for temporal motion vector prediction (TMVP) is proposed to improve the coding performance of MV-HEVC. By constructing a VCP based on the motion information from the inter-view reference picture and two auxiliary temporal reference pictures, inter-view motion prediction can be introduced into MV-HEVC without changing any CU-level syntax or procedure.

A VCP generation process is conducted before a picture or a slice is encoded/ decoded as depicted in Fig. 1. The VCP generation process takes the inter-view reference picture and two auxiliary temporal reference pictures as input and output the VCP. VCP is treated as the collocated picture in TMVP for CU-level procedures. It is transparent to CU-level procedures whether a VCP or a real collocated picture is used in TMVP. Thus the VCP approach is a picture-level coding tool without violating the CU-level constrain in MV-HEVC.

In the proposed design, a flag is signaled for a slice on dependent views to indicate whether VCP should be used. If it is true, the two auxiliary pictures are then signaled explicitly. In the proposed encoder, the two auxiliary pictures are set equal to the two collocated picture used by NBDV in 3D-HEVC. The encoder should choose the inter-view reference picture as the collocated picture if it decides to apply the VCP method.

The VCP generation process works as by filling the motion information for each block:

- If a DV can be obtained from the collocated block by one of the auxiliary picture and there is a MV in the DV-calibrated block in the inter-view reference picture, the MV information in the DV-calibrated block is copied to the current block in VCP as depicted in Fig. 2. A DV-calibrated block is got by displacing the current block with the DV.

- Otherwise, motion information in the auxiliary picture is copied to the current block in VCP. It should be noted that the MV need to be scaled before copying in this case. Because the VCP possesses the POC equal to the inter-view reference picture (also equal to the current picture) but the auxiliary possess different POCs.

Fig. 1 VCP for MV-HEVC.

Fig. 2 Copy motion information from the inter-view reference picture to VCP.

# Experimental results

The proposed method is integrated on HTM-10.0r1 [3]. Table 1 show the results under the common test condition [4].

Table 1.  Performance for the proposed VCP (CTC)

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 7,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1004, Jan. 2014.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1001, Jan. 2014.

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1/

- K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan. 2014.