| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0089 |
| --- | --- |

| Title: | MV sharing in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a MV sharing method between the depth and texture components is proposed to reduce the MV storage. MVs needed by temporal motion vector prediction or inter-view motion prediction for depth coding are fetched from the corresponding texture picture instead of a depth picture. MV storage can be halved in the proposed approach. The experimental results reportedly show that the proposed simplification brings no coding loss.

# Introduction

Temporal motion vector prediction (TMVP) and inter-view motion prediction (IVMP) are applied on both texture and depth component in 3D-HEVC [1]. When TMVP or IVMP is used, a MV is fetched from the MV buffer of a reference picture, which is the collocated picture or an inter-view reference picture, as depicted in Fig.1(a). TMVP or IVMP in depth and texture component are manipulated separately.

# Proposed method

It is proposed that the MV storage for TMVP or IVMP should be shared by the depth and texture components. When TMVP or IVMP is used by a depth picture, the MV should be fetched from the MV buffer of a texture picture corresponding to the reference depth picture, which is the collocated picture or an inter-view reference picture, as depicted in Fig.1(b). In the proposed design, MVs for a depth reference picture is not required to be stored any longer. Thus the MV storage can be halved in 3D-HEVC.

# Experimental results

The proposed method is integrated on HTM-10.0r1 [2]. Table 1 shows the results under the common test condition [3]. The experiments results show that the proposed simplification brings no coding loss while the MV picture buffer used to store the motion information of coded depth pictures could be totally removed.

| (a) | (b) |
| --- | --- |

Fig. 1 MV storage in the current 3D-HEVC (a), and in the proposed method (b).

Table 1.  Performance for the proposed MV sharing (CTC)

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1001, Jan. 2014.

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1/

- K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan. 2014.