| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0090 |
| --- | --- |

| Title: | Simplification on illumination compensation for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current 3D-HEVC, the training process of illumination compensation (IC) imposes quite a high complexity burden on both the encoder and decoder side. This contribution proposes to constrain the number of training samples not to exceed a fixed number to reduce the complexity of the IC training process in the worst case. Experimental results reportedly show that the proposed method achieve 0.02% BD-rate saving for the synthesized views in the common test condition.

# Introduction

In the current 3D-HEVC [1], illumination compensation (IC) is adopted to compensate the difference of illumination intensity between views. When IC is applied, the prediction value y is calculated as y = a∙x+b, where x is a sample value in the reference block in the reference view. The two parameters a and b are trained with the neighboring reconstructed samples of the current block and the reference block as depicted in Fig. 1.

In the training process, a neighboring sample xi of the reference block and a neighboring sample yi of the current block possessing the same relative position are treated as a training pair. To reduce the number of training pairs, only one of two adjacent samples are involved in the training set as depicted in Fig.2.

Fig.1 A general IC paradigm in the current 3D-HEVC.

Fig.2 Training samples from neighboring samples of the reference block (labeled as ‘xLi’ for left neighboring samples and ‘xAi’ for above neighboring samples) and from neighboring samples of the current block (labeled as ‘yLi’ for left neighboring samples and ‘yAi’ for above neighboring samples).

Summations are required in the derivation of the two parameters a and b as

, , , .

where N is the number of training pairs. There will be 2*N multiplications, 2*N shifts and 4*(N-1) additions to calculate the four variables totally. Since other calculation in the training process does not depend on N, the complexity of the training process will grow linear to the width/height of the block.

In addition, since sumRef ∙ sumRef and sumRef ∙ sumCur are performed in the later calculation, the maximum value for sumRef or sumCur will determine the register bit-width in the training process.

In the worst case, where N is equal to 64 when the block size is 64x64, there are 128 multiplications, 128 multiplications and 252 additions in the summation process. And the required register bit-width is 28.

# Proposed method

To further simplify the training process of IC, it is proposed to constrain the number of training samples not to exceed a fixed number. In particular, the maximum value of N is set to be 16 no matter how large the block size is. If the width/height of the block, noted as C is smaller than 32, the training process keeps the original manner. Otherwise, only one of M adjacent samples are involved in the training set, where M = C >> 3. Fig. 3 demonstrates an example of proposed selected samples when C = 32.

With this proposed constrain, there are at most 32 multiplications, 32 multiplications and 60 additions in the summation process in the worst case. And the required register bit-width is also reduced to 24. Table 1 compares the complexity in worst case between the proposed method and the original one.

Table 1 Complexity comparison

|  | Multiplication | Shift | Addition | bit-width |
| --- | --- | --- | --- | --- |
| Anchor | 128 | 128 | 252 | 28 |
| Proposed | 32 | 32 | 60 | 24 |

Fig. 3 Proposed selected one training samples (black) when the block size is 32x32.

# Experimental results

The proposed method is integrated on HTM-10.0r1 [2]. Table 2 shows the results under the common test condition [3]. There is no coding performance loss.

Table 2. Performance for the proposed solution (CTC)

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1001, Jan. 2014.

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1/

- K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan. 2014.