| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0091 |
| --- | --- |

| Title: | On DLT signaling in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes several solutions to address problems in depth look-up table (DLT) signaling in 3D-HEVC. Firstly, it is clarified that a DLT cannot be predicted by a non-existed DLT. Secondly, the bit-depth for depth samples signaled in SPS and PPS is made consistent. Finally, the redundancy of signaling DLT several times is removed. Experimental results show that there is no obvious coding performance change.

# Introduction

Depth lookup table (DLT) is adopted into 3D-HEVC [1]. Since there are often only several values appearing in the depth component, DLT signals those valid values from the encoder to the decoder. When a CU is coded in segment-wise DC coding (SDC) mode or depth map modeling (DMM) mode, DLT is used to map the valid depth value to a DLT index which is much easier to compress. DLT is signaled as an extension to picture parameter set (PPS). Some syntax elements related to DLT signaling are listed high-lightened below.

I.7.3.2.3 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) { |  |
| for ( i = 0; i < 8; i++ ) |  |
| pps_extension_type_flag[ i ] | u(1) |
| if( pps_extension_type_flag[ 0 ] ) |  |
| poc_reset_info_present_flag | u(1) |
| if( pps_extension_type_flag[ 2 ] ) |  |
| pps_dlt_parameters( ) | u(1) |
| if( pps_extension_type_flag[ 7 ] ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- I.7.3.2.3.1 Picture parameter set depth look up table syntax

| pps_dlt_parameters( ) { | Descriptor |
| --- | --- |
| dlt_present_flag | u(1) |
| if( dlt_present_flag ) { |  |
| pps_depth_layers_minus1 | u(6) |
| pps_bit_depth_for_depth_views_minus8 | u(4) |
| for( i=0; i <= pps_depth_layers_minus1; i++ ) { |  |
| dlt_flag[ i ] | u(1) |
| if( dlt_flag[ i ] ) { |  |
| inter_view_dlt_pred_enable_flag[ i ] | u(1) |
| if( !inter_view_dlt_pred_enable_flag[ i ] ) |  |
| dlt_bit_map_rep_flag[ i ] | u(1) |
| if( dlt_bit_map_rep_flag[ i ] ) |  |
| for( j = 0; j <= depthMaxValue; j++ ) |  |
| dlt_bit_map_flag[ i ][ j ] | u(1) |
| else |  |
| entry_table( i ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Proposed method

There are several problems in the current design. Solutions are proposed to address the problems.

## Issue 1

- Problem

Inter-view prediction can be applied in DLT signaling when inter_view_dlt_pred_enable_flag[ i ] is set equal to 1. In such a case, the i-th DLT is predicted from the 0-th DLT. On the other hand, dlt_flag[ i ] indicates whether the i-th DLT exists. Thus if dlt_flag[ 0 ] is 0 but inter_view_dlt_pred_enable_flag[ i ] with i > 0 is 1, the i-th DLT is predicted from a non-existed DLT.

- Solution

It should be constrained that a DLT cannot be predicted from a non-exited DLT. If the 0-th DLT does not exist, inter_view_dlt_pred_enable_flag[ i ] should be derived as 0 implicitly without signaling as below.

| if(dlt_flag[ 0 ]) |  |
| --- | --- |
| inter_view_dlt_pred_enable_flag[ i ] | u(1) |

## Issue 2

- Problem

In PPS, pps_bit_depth_for_depth_views_minus8 is signaled to indicate the bit-depth for samples in the depth component. However, bit_depth_luma_minus8 which is signaled in sequence parameter set (SPS) for the depth component also indicates the bit-depth for samples in the depth component. There will be contradiction if these two syntax elements are different.

- Solution

Bit-depth for samples signaled anywhere in a video sequence must be consistent. All bit-depths for samples signaled in a video sequence must be the same to the one signaled in SPS. It should be stated in the specification that pps_bit_depth_for_depth_views_minus8 in PPS must be the same to bit_depth_luma_minus8 in SPS.

## Issue 3

- Problem

In the current design, DLT is signaled for all PPSs in the bit-stream. However, DLT is only required by the depth component of several views. Currently there are 4 PPS signaled totally when there are 3 views. As depicted in Fig. 1(a), three PPSs are sent for the texture component on three views, named as T0, T1 and T2, individually. And one PPS is sent to be shared by the depth component on three views, named as D0, D1 and D2, together. Therefore, 4 copies of the same content of DLT will be signaled in the 4 PPSs repeatedly with an obvious redundancy.

- Solution

DLT is not signaled in a PPS used by texture components. DLT for depth on all views is signaled in a single PPS which is shared by depth components on all views as depicted in Fig. 1(b).

| (a) | (b) |
| --- | --- |

Fig. 1 signal DLT in PPS in the current design (a) and in the proposed solution (b).

# Experimental results

The proposed method is integrated on HTM-10.0r1 [2]. Table 1 and Table 2 show the results under the common test condition [3] for the solutions for issue 1 and issue 3, respectively.

Table 1.  Performance for the proposed solution for issue 1 (CTC)

Table 2.  Performance for the proposed solution for issue 3 (CTC)

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1001, Jan. 2014.

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1/

- K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan. 2014.