| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0093 |
| --- | --- |

| Title: | Simplification on SDC DC mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HTM-10.0, the segment-wise DC coding (SDC) is applied as an alternative intra coding mode. For the DC mode in SDC, the DC value needs to be calculated in two steps. It firstly calculates the mean of the depth values of neighbouring samples as the initial prediction samples of current depth block. As the second step, after a boundary post-filter is applied on the initial prediction samples, it has to re-calculate the mean of the prediction samples as the input of a depth lookup table to derive an index value. In this proposal, it is proposed to simplify the DC mode in SDC by directly using the DC value calculated in the first step. The experimental results reportedly show no coding loss under the common tests conditions and all-intra test conditions.

# Introduction

In 3D-HEVC, the segment-wise DC coding (SDC) is applied as an alternative intra coding mode [1]. For SDC, the prediction samples of the current depth block are generated by the conventional intra prediction or DMM intra prediction. Instead of transmitting quantized transform coefficients to represent the residual signal, the residual of each partition in the current depth block is coded by transmitting a constant residual value to the decoder by signalling only the index into a Depth Lookup Table (DLT).

For the DC prediction modes in intra SDC, current block could be divided into one segment (SDC DC mode) or two segments (SDC DMM mode) selectively. Each segment is then predicted by a DC value derived from the spatial neighboring pixels. The detail is illustrated in figure 1.

For the two-segments case (SDC DMM mode), the DC predictor value could be derived directly from the adjacent samples of the neighboring left and top block. The derived DC predictor value is then used as the input of the depth lookup table to generate the index value.

However, for the one-segment case (SDC DC mode), the DC predictor value needs to be calculated in two steps. In the first step, it calculates the mean of the depth values of neighboring samples and assigns this mean value to all pixels within current depth block as the initial prediction samples. In the second step, the boundary post-filter is then applied to smooth the boundary prediction samples using an (1, 3) or (1, 2, 1) filter. Then, it needs to re-calculate the mean of the prediction samples after boundary post-filtering as the updated DC value and uses it as the input of the depth lookup table to generate the index value.

Fig. 1.  The DC prediction modes in intra SDC

# Proposed methods

In this proposal, we propose to remove the second step of the DC mode in SDC. The mean of the depth values of neighbouring samples is directly used as the prediction samples of current depth block and also used as the input of the depth lookup table to generate the index value as shown in Fig. 2. With this simplification, the input used to generate the index value could both be directly derived from the neighboring samples for SDC DC mode and SDC DMM modes.

Fig. 2.  The proposed simplification

# Experimental results

The proposed removal of the second step of the DC mode in SDC is conducted based on HTM-10.0r1 [2] under the common test conditions (CTC) [3] and all-intra test conditions. The experimental results of the proposed method under the common test conditions and under the all-intra test conditions are shown in Table 1 and Table 2, respectively. The experiments results show that the proposed simplification brings no coding loss.

Table 1.  The results of proposed simplification under CTC

Table 2.  The results of proposed simplification under All-Intra

# Conclusion

In this proposal, it is proposed to simplify the DC mode in SDC. The mean of the depth values of neighbouring samples is directly used as the prediction samples of current depth block and also used as the input of the depth lookup table to generate the index value. The experimental results show no coding loss in the common test conditions and all-intra test condition.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1001, Jan., 2014.

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan., 2014.