| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0094 |
| --- | --- |

| Title: | Improvement on the signaling of DBBP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin1, Yi-Wen Chen1, Tzu-Der Chuang1, Xianguo Zhang2, Yu-Wen Huang1, Shawmin Lei1 1MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, peter.chuang, xianguo.zhang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In depth-based block partitioning (DBBP) mode, instead of signaling a partition mode to a decoder, a best partition mode is implicitly derived from a corresponding depth block. However, in current syntax design, in order to signal the DBBP mode, encoder firstly needs to signal a 2NxN partition mode to a decoder and then transmit a DBBP flag to indicate it is DBBP mode. The signaling of the 2NxN partition mode is actually unnecessary, because the real partition mode is derived from the corresponding depth block when DBBP is enabled. In order to clean up the syntax and remove the redundancy, a solution is proposed in this contribution to remove the dependence on 2NxN partition mode in DBBP flag signaling and remove the redundant partition mode signaling. The experiment results reportedly show no coding loss.

# Introduction

In 3D-HEVC [1], when the depth-based block partitioning (DBBP) mode is applied on a texture coding block, an arbitrary shaped segmentation mask is derived from a corresponding depth block. The motion compensation is then performed using two 2Nx2N motion compensated blocks and merged based on the depth-based segmentation mask. After the depth-based segmentation mask is derived, a best matched block partition mode is selected among 2NxN, Nx2N, 2NxnU, 2NxnD, nLx2N, and nRx2N based on the corresponding depth block for motion information coding and storing.

A DBBP flag is signaled at CU level to indicate whether current CU applies DBBP prediction or other conventional inter partition modes. Table 1 shows the syntax design for the DBBP flag. In current syntax design, the DBBP flag is signaled depended on partition mode equals to 2NxN which is quite unreasonable. To signal the DBBP mode, encoder firstly needs to signal a 2NxN partition mode to a decoder. Then a DBBP flag is transmitted to indicate whether this is a conventional 2NxN inter partition mode or a DBBP mode. If it is DBBP mode, the transmitted 2NxN partition mode is actually meaningless because the real partition mode will be implicitly derived from the corresponding depth block. Therefore the signaling of the partition mode is actually unnecessary when DBBP is enabled.

Table 1.  The syntax design for the DBBP flag

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && PartMode = = PART_2NxN ) |  |
| dbbp_flag[ x0 ][ y0 ] | u(1) |

# Proposed methods

In this proposal, two solutions are proposed to fix the problems described above and improve the syntax design for DBBP mode. In solution 1, the dependence on 2NxN partition mode in DBBP flag signaling is removed as shown in Table 3. The encoder doesn’t need to transmit a 2NxN partition mode before the signaling of DBBP flag since the partition mode derived by DBBP could be one of 2NxN, Nx2N, 2NxnU, 2NxnD, nLx2N, and nRx2N modes.

Table 2.  Removal of the dependence on 2NxN partition

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && PartMode = = PART_2NxN ) |  |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |

Furthermore, when DBBP is enabled, the signaling of partition mode is actually unnecessary because the partition mode is implicitly derived from the depth block. To further remove the redundancy, in solution 2, it is proposed to skip the signaling of partition mode syntax when the DBBP flag equals to 1 as shown in Table 3.

Table 3.  Proposed syntax design for DBBP signaling

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) |  |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag &&! dbbp_flag[ x0 ][ y0 ]) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && PartMode = = PART_2NxN ) |  |
| dbbp_flag[ x0 ][ y0 ] | u(1) |

# Experimental results

The proposed syntax design for DBBP flag signaling is conducted based on HTM-10.0r1 [2] under the common test conditions (CTC) [3]. The experimental results of the proposed solution 1 and solution 2 are shown in Table 4 and Table 5, respectively. The experiments results show that the both proposed solutions bring slight coding gains, while the syntax could be cleaned up and the redundancy is also removed.

Table 4.  The results of the proposed solution 1

Table 5.  The results of the proposed solution 2

# Conclusion

In this proposal, we propose to improve the syntax design for signaling of DBBP mode by cleaning up the syntax and removing the redundancy of signaling partition mode. The experimental results show slight coding gain could be achieved while the syntax design for signaling of DBBP mode is improved.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1001, Jan., 2014.

- HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan., 2014.