| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0096 |
| --- | --- |

| Title: | 3D-CE1: Simplification of a pruning process in 3D Merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

At Geneva meeting, the simple Merge candidate list construction was adopted (F0093). It presented the a simple and low-complexity way to implement the Merge list construction module for 3D-HEVC by re-using the existing HEVC Merge list construction module. As follow-up of G0099, this contribution proposes to remove some pruning processes of the HEVC Merge candidates for the further simplification. The proposed method only performs the pruning processes, between IVMV-A1/B1 in the texture coding and MPI-A1/B1 in the depth coding in option 1, between IVMV-A1 in the texture coding and MPI-A1 in the depth coding in option 2, and between IVMV-A1 in the texture coding in option 3, respectively. The result demonstrates that the proposed method has the almost no coding loss (less than 0.05%) in spite of the simplification. The number of the pruning processes can be reduced from 8 to 1 in the option 3.

# Proposed Method

In the 3D HEVC Merge list construction, five and three pruning processes are performed in the texture and depth coding, respectively, as shown in Table 1. It is observed that some pruning processes have no coding gain at all. Based on the evaluation, The proposed method only performs the pruning processes, between IVMV-A1/B1 in the texture coding and MPI-A1/B1 in the depth coding in option 1, between IVMV-A1 in the texture coding and MPI-A1 in the depth coding in option 2, and between IVMV-A1 in the texture coding in option 3, respectively.

Table 1. Proposed pruning process

| Texture | Depth | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| N/A | MPI-IVMV | Removed | Removed | Removed |
| IVMV-A1 | MPI-A1 | Allowed | Allowed | Allowed only for texture |
| IVMV-B1 | MPI-B1 | Allowed | Removed | Removed |
| A1-DV | N/A | Removed | Removed | Removed |
| B1-DV | N/A | Removed | Removed | Removed |
| IVMV-Shift IVMV | N/A | Removed | Removed | Removed |

# Proposed Text (for Option 3)

- H.8.5.3.2.1 Derivation process for luma motion vectors for merge mode

…

  - The merging candidate list, extMergeCandList, is constructed as follows:

i = 0
			if( availableFlagT )
				extMergeCandList[ i++ ] = T
			if( availableFlagD )
				extMergeCandList[ i++ ] = D
			if( availableFlagIvMC)
				extMergeCandList[ i++ ] = IvMC
			N = DepthFlag ? T : IvMC
			if( availableFlagA1  && ( !availableFlagN  | |  differentMotion( N, A1 ) || N!=IvMC ) )
				extMergeCandList[ i++ ] = A1
			if( availableFlagB1)
				extMergeCandList[ i++ ] = B1
			if( availableFlagB0 )
				extMergeCandList[ i++ ] = B0	
			if( availableFlagIvDC  
				&&  ( i < ( 5 + NumExtraMergeCand ) ) )
				extMergeCandList[ i++ ] = IvDC
			if( availableFlagVSP  &&  !ic_flag  &&  iv_res_pred_weight_idx  = =  0  &&  
									i < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = VSP
			if( availableFlagA0  &&  i < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = A0
			if( availableFlagB2  &&  i < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = B2
			if( availableFlagIvMCShift  &&  i < ( 5 + NumExtraMergeCand )
								)
				extMergeCandList[ i++ ] = IvMCShift

...

# Test Results

Test results of the options 1 to 3 in the proposed method are summarized in Tables 2 to 4. As displayed in Tables, the almost no coding loss is observed (less than -0.05%).

Table 5. Performance when the pruning processes between A1-DV, B1-DV, IVMV-Shift IVMV, and MPI-IVMV are removed (Option 1)

Table 4. Performance when the pruning processes are only allowed between IVMV-A1 in the texture coding and MPI-A1 in the depth coding (Option 2)

Table 4. Performance when the pruning processes are only allowed between IVMV-A1 in the texture coding only (Option 3)

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can simplify the pruning process in the 3D Merge mode list construction. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).