| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0097 |
| --- | --- |

| Title: | 3D-CE1: Simplification of 3D Merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim Yichen Zhang and Lu Yu | Email: | jinyoung79.lee@samsung.com felixzyc@gmail.com yul@zju.edu.cn |
| Source: | Samsung Electronics Co. Ltd. and Zhejiang University |  |  |

_____________________________

# Abstract

At Geneva meeting, the simple Merge candidate list construction was adopted (F0093). It introduced the a simple and low-complexity way to implement the Merge list construction module for 3D-HEVC by re-using the existing HEVC Merge list construction. As follow-up of G0099 and G0104, this contribution proposes to remove the availability checking process of the HEVC candidates such as A0, A1, B0, B1, and B2 in the 3D Merge list construction. As a result, it is not required to know what candidates are in the HEVC list, which means that it can minimize the dependency between the already constructed HEVC list and 3D-HEVC candidates. The result demonstrates that the proposed method has negligible coding loss which is less than 0.1%. In terms of the simplification, the availability of the A0, A1, B0, B1, and B2 is not needed to be checked in the Merge list construction, the number of the pruning processes is reduced from 8 to 2, and the VSP flag storage is not required. In addition, the corresponding WD can be reduced.

# Proposed Method

In the current 3D-HEVC Merge list construction, it is required to know what candidates are in the HEVC list and where the candidates are placed in the list. However, in the proposed method, it is not required to know the information of the already constructed HEVC list at all, which means that it can minimize the dependency between the HEVC list and 3D-HEVC candidates.

First of all, the proposed method always performs the pruning processes between IVMV (or MPI) and the first candidates in the HEVC Merge list, as shown in Table 2. Hence, the availability of A1 does not need to be checked in the pruning process.

Table 1. Proposed pruning process

Texture(or Depth)

IVMV(or MPI)- First candidate in the HEVC list

Second, the proposed method does not require checking the availability of A0, A1, B0, B1, and B2 in the 3D Merge list construction. For example, the HEVC Merge list is already constructed and five candidates are decided in the list. In the texture coding, firstly, IVMV is added in the 3D Merge list. In the depth coding, MPI is placed ahead of DDD and IVMV. After the addition of IVMV, the candidates firstly placed in the HEVC list are added. Whether A1 and B1 exist in the list or not is not necessary to be known here. Next, DV, the third candidate in the HEVC list, Shift IVMV, and Shift DV are added in order. Finally, the remaining candidates in the HEVC list are added. Tables 2 and 3 illustrate the proposed 3D Merge list construction in the texture and depth coding, respectively.

Table 2. Proposed 3D Merge list construction (Texture)

| Current | Proposed |
| --- | --- |
| IVMV | IVMV |
| A1 | First candidate in the HEVC list |
| B1 | Second candidate in the HEVC list |
| B0 | DV |
| DV | Third candidate in the HEVC list |
| VSP | Shift IVMV |
| A0 | Shift DV |
| B2 | Forth candidate in the HEVC list |
| Shift IVMV | Fifth candidate in the HEVC list |
| Shift DV |  |
| Temporal |  |

Table 3. Proposed 3D Merge list construction (Depth)

| Current | Proposed |
| --- | --- |
| MPI | MPI |
| DDD | DDD |
| IVMV | IVMV |
| A1 | First candidate in the HEVC list |
| B1 | Second candidate in the HEVC list |
| B0 | Third candidate in the HEVC list |
| A0 | Forth candidate in the HEVC list |
| B2 | Fifth candidate in the HEVC list |
| Temporal |  |

Finally, the proposed method removes all the VSP flags which indicate whether the neighboring blocks are coded with VSP or not. Instead, the candidate corresponded to a VSP coded neighboring block is treated as a DCP candidate. As a result, the VSP flag storage can be totally removed. Also, the VSP candidate is always inserted to the fourth position in the Merge list, i.e., with merge index equaling to 3, the coding mode is always VSP (when IC and ARP is not applied to current PU).

Fig. 1. Proposed VSP position

# Proposed Text

- I.8.5.3.2.1 Derivation process for luma motion vectors for merge mode

- …

…

- 6. The merging candidate list, extMergeCandList, is constructed as follows:

i = 0
			if( availableFlagT )
				extMergeCandList[ i++ ] = T
			if( availableFlagD )
				extMergeCandList[ i++ ] = D
			if( availableFlagIvMC)
				extMergeCandList[ i++ ] = IvMC
			N = DepthFlag ? T : IvMC
			if(( !availableFlagN  | |  differentMotion( N, baseMergeCandList[0] ) ) )
				extMergeCandList[ i++ ] = baseMergeCandList[0]
			)
				extMergeCandList[ i++ ] = baseMergeCandList[1]
			
				extMergeCandList[ i++ ] = baseMergeCandList[2]	
			if( availableFlagIvDC  
				&&  ( i < ( 5 + NumExtraMergeCand ) ) )
				extMergeCandList[ i++ ] = IvDC
			
									
				
			
				
			
				
			if( availableFlagIvMCShift  &&  i < ( 5 + NumExtraMergeCand )
								)
				extMergeCandList[ i++ ] = IvMCShift

...

- 8. The merging candidate list, extMergeCandList, is constructed as follows:

if( availableFlagIvDCShift )
				extMergeCandList[ i++ ] = IvDCShift
			j = 0
			while( i < MaxNumMergeCand ) {	(Error: Reference source not found-1)
				
				)
					extMergeCandList[ i++ ] = baseMergeCandList[3+(j++)N
			}

if( availableFlagVSP && !ic_flag && iv_res_pred_weight_idx = = 0){
			j = 4
			while ( j < MaxNumMergeCand ){
				extMergeCandList[j] = extMergeCandList[j-1]
			}
			extMergeCandList[3] = VSP
		}

- 9. The variable N is set equal to extMergeCandList[ MergeIdx[ xOrigP ][ yOrigP ] ].

- 11. The variable vspModeFlag is derived as specified in the following:

      - vspModeFlag = ( view_synthesis_pred_flag[ layerId ] ) && (merge_idx[xPb][yPb] = = 3) && (!ic_flag && iv_res_pred_weight_idx  = =  0)

- I.8.5.3.2.15 Derivation process for the shifted disparity merging candidate

- …

- The variable availableFlagIvDCShift is set equal to 0 and for i in the range of 0 to numMergeCand - 1, inclusive, the following applies:

  - The variable N is set equal to mergeCandList[ i ].

  - When availableFlagIvDCShift is equal to 0 and availableFlagN is equal to 1, the candidate N is not equal to IvMC or IvDC, predFlagL0N is equal to 1 and ViewIdx( RefPicList0[ refIdxL0N ] ) is not equal to ViewIdx, the following applies:

- …

          - -

          - -

          - -

          - -

# Test Results

Test results of the proposed method are summarized in Table 4. Although the proposed method simplifies the Merge list construction, almost no coding loss is observed (less than 0.1%). Table 5 shows the benefit of the proposed method in terms of simplification.

Table 4. Performance of the proposed method

Table 5. Analysis in terms of simplification

|  | Availability Check | Pruning | VSP flag |
| --- | --- | --- | --- |
| Current | A0, A1, B0, B1, and B2 | 8 | Needed |
| Proposed | N/A | 2 | N/A |

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can simplify the process of the 3D Merge mode list construction. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).