| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0098 |
| --- | --- |

| Title: | 3D-CE1: Simplification of shift DV candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

In 3D-HEVC, there are two shift DV candidates. one is the shifted first available DV and the other is the shifted DoNBDV. In order to find the shifted first available DV, all the neighboring blocks are searched whether they have DV or not. These candidates are generated by adding four to the horizontal component of the existing DV. However, the shift DVs reportedly provide the small coding gain. Therefore, this contribution proposes to remove the shift DVs as the first option. Based on WD, it reduces 30 condition checks by removing subclause H.8.5.3.2.15 in the worst case. The second option is to use the depth-based minimum DV (G0100), which only requires the operation finding the minimum depth value, instead of the shift DVs, when the depth map is available. When the depth image is not available, the shift DV is employed without the searching process. The experiment shows that both the options provide the coding gain loss of less than 0.1% in both CTC and VSP/DBBP off conditions, respectively, which is negligible.

# Proposed Method

Currently, there are three additional shift candidates. One is the shift IVMV and the others are the shift DVs. The shift IVMV has some gain as displayed in Table 1, but the two shift DVs have very minor gain. This contribution proposes to remove the shift DVs (Option1) or replace the shift DVs with the minimum depth-based DV when the depth map is available (Option2).

Table 1. Performance of the shift IVMV candidate

Option 1 is just to remove the shift DVs. So, only shift IVMV is used as the shift candidates.

Option 2 is to use the depth-based minimum DV (G0100), when the depth image is available. When the depth image is not available, the proposed method uses the shift DV without the searching process. The current depth-based DV (DoNBDV) is derived by choosing the maximum depth value among four corner pixels in the corresponding depth block. The proposed depth-based DV (DoNBDVmin) is derived by choosing the minimum depth value among four corner pixels.

Figure 1. Depth-based DV derivation

Since DoNBDV is derived in CU, sometimes DoNBDV can be different from the real disparity if CU has more than two PUs. For instance, when the prediction mode is PART_Nx2N as shown in Figure 2 and the first PU (PU 0) has a background and the second PU (PU 1) has an object, DoNBDV is most likely to become a disparity of the second PU and cannot reflect the disparity of the first PU. However, if the proposed disparity vector (DoNBDVmin) is used, DoNBDVmin might be able to reflect the disparity of the first PU.

Figure 2. Example of the coding unit having 2 prediction units

# Proposed Text

- I.8.5.3.2.1 Derivation process for luma motion vectors for merge mode

- …

- 5. Depending on iv_mv_pred_flag[ nuh_layer_id ] and DispAvailabilityIdc[ xPb ][ yPb ], the following applies:

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0 or DispAvailabilityIdc[ xPb ][ yPb ] is not equal to DISP_NONE, the flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause  is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as inputs, and the output is assigned to the availability flags availableFlagIvMC, availableIvMCShift, availableFlagIvDC, and availableIvDCmin, the reference indices refIdxLXIvMC, refIdxLXIvMCShift, refIdxLXIvDC, and refIdxLXIvDCmin, the prediction list utilization flags predFlagLXIvMC, predFlagLXivMCShift, predFlagLXIvDC, and predFlagLXIvDCmin, and the motion vectors mvLXIvMC, mvLXIvMCShift, mvLXIvDC, and mvLXIvDCmin (with X being 0 or 1, respectively).

- ...

- 7. The variable availableFlagIvDCShift is set equal to 0, and when availableFlagIvMCShift is equal to 0, DepthFlag is equal to 0, depth_refinement_flag[ nuh_layer_id ] is equal to 0, and i is less than ( 5 + NumExtraMergeCand ), the derivation process for the shifted disparity merging candidate as specified in subclause  is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, and the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N, of every candidate N being in extMergeCandList, extMergeCandList, and i as inputs, and the outputs are the flag availableFlagIvDCShift, the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift, the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift, and the motion vectors mvL0IvDCShift and mvL1IvDCShift.

- 8. The merging candidate list, extMergeCandList, is constructed as follows:

if(availableIvDCmin)
		           extMergeCandList[ i++ ] = IvDCmin

else if( availableFlagIvDCShift )
				extMergeCandList[ i++ ] = IvDCShift
			j = 0
			while( i < MaxNumMergeCand ) {	
				N = baseMergeCandList[ j++ ]
				if( N  !=  A1  &&  N  !=  B1  &&  N  !=  B0  &&  N  !=  A0  &&  N  !=  B2 )
					extMergeCandList[ i++ ] = N
			}

- I.8.5.3.2.10 Derivation process for inter-view merge candidates

- …

The availability flags availableFlagIvMC, availableFlagIvMCShift, availableFlagIvDC, and availableFlagIvDCmin are set equal to 0, and for X in the range of 0 to 1, inclusive, the variables predFlagLXIvMC, predFlagLXIvMCShift, predFlagLXIvDC, and predFlagLXIvDCmin are set equal to 0, the variables refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC, and refIdxLXIvDCmin are set equal to −1,  and both components of mvLXIvMC, mvLXIvMCShift, mvLXIvDC, and mvLXIvDCmin are set equal to 0.

- …

- 2. The availability flag availableFlagIvDC, and the prediction utilization flags predFlagL0IvDC and predFlagL1IvDC are derived by

      - availableFlagIvDC = availableFlagL0IvDC | | availableFlagL1IvDC

      - predFlagL0IvDC = availableFlagL0IvDC

      - predFlagL1IvDC = availableFlagL1IvDC

When DepthFlag is equal to 0 and depth_refinement_flag[ nuh_layer_id ] is equal to 1, the minimum disparity inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for a disparity inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ], the disparity vector MvMinRefinedDisp[ xPb ][ yPb ], and the prediction list indication X, as the inputs and the outputs are the flag availableFlagLXIvDCmin, the motion vector mvLXIvDCmin, and the reference index refIdxLXIvDCmin.

  - The availability flag availableFlagIvDCmin, and the prediction utilization flags predFlagL0IvDCmin and predFlagL1IvDCmin are derived by

      - availableFlagIvDCmin = availableFlagL0IvDCmin | | availableFlagL1IvDCmin

      - predFlagL0IvDCmin = availableFlagL0IvDCmin

      - predFlagL1IvDCmin = availableFlagL1IvDCmin

- I.8.5.3.2.15 Derivation process for the shifted disparity merging candidate

- …

This process is not invoked when DepthFlag is equal to 1.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

- the availability flags availableFlagN,

- the reference indices refIdxL0N and refIdxL1N,

- the prediction list utilization flags predFlagL0N and predFlagL1N,

- the motion vectors mvL0N and mvL1N,

- a merging candidate list mergeCandList,

- the variable numMergeCand specifying the number of merge candidates in list mergeCandList

Outputs of this process are:

- the flag availableFlagIvDCShift, specifying whether shifted disparity merging candidate is available

- the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift,

- the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift,

- the motion vectors mvL0IvDCShift and mvL1IvDCShift.

- The variable availableFlagIvDCShift is set equal to 0 -

- When availableFlagIvDCShift is equal to 0 and availableFlagIvDC is equal to 1, availableFlagIvDCShift is set to 1 and the following applies for X being 0 to 1, inclusive:

- predFlagLXIvDCShift is set equal to predFlagLXIvDC,

- refIdxLXIvDCShift is set equal to refIdxLXIvDC,

- mvLXIvDCShift[ 0 ] is set equal to mvL0IvDC[ 0 ] + 4

- mvLXIvDCShift[ 1 ] is set equal to mvL0IvDC[ 1 ]

- I.8.5.3.2.13.1 Derivation process for luma motion vectors for view synthesis prediction merge candidate

- …

The derivation process for a disparity sample array as specified in subclause  is invoked with the luma locations xPb, yPb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPbW , the variable nPbH, variable minFlag equal to 0, and the variable partIdc equal to 2 as inputs, and the output is the array disparitySamples of size (nPbW)x(nPbH) and the flag horSplitFlag.

- …

- I.8.5.5 Derivation process for disparity vectors

…

Depending on depth_refinement_flag[ nuh_layer_id ], the following applies:

- If depth_refinement_flag[ nuh_layer_id ] is equal to 1, the following applies:

  - The derivation process for a disparity sample array as specified in subclause  is invoked with the luma locations xCb, yCb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS, variable minFlag equal to 0 and the variable partIdc equal to 0 as inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The disparity vector mvRefinedDisp is set equal to ( disparitySamples[ 0 ][ 0 ], mvDisp[ 1 ] ).

  - The derivation process for a disparity sample array as specified in subclause  is invoked with the luma locations xCb, yCb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS, variable minFlag equal to 1 and the variable partIdc equal to 0 as inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The disparity vector mvMinRefinedDisp is set equal to ( disparityMinSamples[ 0 ][ 0 ], mvDisp[ 1 ] ).

- Otherwise (depth_refinement_flag[ nuh_layer_id ] is equal to 0), the disparity vector mvRefinedDisp is set equal to mvDisp.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xCb.. ( xCb + nCbS − 1 ), y = yCb..( yCb + nCbS− 1 ):

  - MvDisp[ x ][ y ] = mvDisp		(Error: Reference source not found-1)

  - MvRefinedDisp[ x ][ y ] = mvRefinedDisp		(Error: Reference source not found-2)

  - MvMinRefinedDisp[ x ][ y ] = mvMinRefinedDisp

  - RefViewIdx[ x ][ y ] = refViewIdx		(Error: Reference source not found-3)

  - DispAvailabilityIdc[ x ][ y ] = dvAvailFlag ? DISP_AVAILABLE : 
	( DefaultRefViewIdxAvailableFlag ? DISP_DEFAULT : DISP_NONE )	(Error: Reference source not found-4)

- I.8.5.5.2 Derivation process for a disparity sample array

- …

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of disparities values,

- …

The array disparitySamples is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH ) − 1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nPSW / nSubBlkW ) − 1 ), inclusive, the following applies:

    - The variable repDep is set equal to −1 and modified as specified in the following:

xSubB = sBx * nSubBlkW
			ySubB = sBy * nSubBlkH
			xP0 = Clip3( 0, pic_width_in_luma_samples − 1, xTL + xSubB )
			yP0 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB ) 
			xP1 = Clip3( 0, pic_width_in_luma_samples − 1, xTL + xSubB + nSubBlkW − 1 )
			yP1 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB + nSubBlkH − 1 )

if(minFlag==0) {
			repDep = Max(refDepPels[ xP0 ][ yP0 ], refDepPels[ xP0 ][ yP1] )
			repDep = Max( maxDep, refDepPels[ xP1][ yP0 ] )
			repDep = Max( maxDep, refDepPels[ xP1][ yP1 ] )

} else {

repDep = Min(refDepPels[ xP0 ][ yP0 ], refDepPels[ xP0 ][ yP1] )
			repDep = Min( minDep, refDepPels[ xP1][ yP0 ] )
			repDep = Min( minDep, refDepPels[ xP1][ yP1 ] )

}

    - The values of the array depthSamples are modified as specified in the following:

for ( yOff = 0; yOff < nSubBlkH; yOff++ )
				for( xOff = 0; xOff < nSubBlkW; xOff++ ) {
					x = xSubB + xOff 
					y = ySubB + yOff 
					disparitySamples[ x ][ y ] = DepthToDisparityB[ refViewIdx ][ repDep ]

}

# Test Results

Test results of the proposed method are summarized in Tables 2-5. Tables 2 and 3 show the performance of the option1 in the CTC and VSP/DBBP disabled condition, respectively. Tables 4 and 5 illustrate the performance of the option2 in the CTC and VSP/DBBP disabled condition, respectively. As shown in the tables, the almost no coding loss is observed (less than 0.1%).

Table 6 shows the benefit of the proposed method in terms of the simplification.

Table 2. Performance of the option1 in CTC

Table 3. Performance of the option1 in VSP/DBBP disabled condition

Table 4. Performance of the option2 in CTC

Table 5. Performance of the option2 in VSP/DBBP disabled condition

Table 6. Analysis in terms of simplification

|  | Current | Option1 | Option2 |
| --- | --- | --- | --- |
| The number of condition checks | 30 | 0 | Minimum function |

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can simplify the process of the 3D Merge mode list construction. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).