| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0098 |
| --- | --- |

| Title: | 3D-CE1: Simplification of shift DV candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

In 3D-HEVC, there are two shift DV candidates. one is the shifted first available DV and the other is the shifted DoNBDV. In order to find the shifted first available DV, all the neighboring blocks are searched whether they have DV or not. These candidates are generated by adding four to the horizontal component of the existing DV. However, the shift DVs reportedly provide the small coding gain. Therefore, this contribution proposes to remove the shift DVs as the first option. Based on WD, it reduces 30 condition checks by removing subclause I.8.5.3.2.15 in the worst case. The second option is to use the depth-based minimum DV (G0100), which only requires the operation finding the minimum depth value, instead of the shift DVs, when the depth map is available. When the depth image is not available, the shift DV is employed without the searching process. The experiment shows that both the options provide the coding gain loss of less than 0.1% in both CTC and VSP/DBBP off conditions, respectively, which is negligible.

# Proposed Method

Currently, there are three additional shift candidates. One is the shift IVMV and the others are the shift DVs. The shift IVMV has some gain as displayed in Table 1, but the two shift DVs have very minor gain. This contribution proposes to remove the shift DVs (Option1) or replace the shift DVs with the minimum depth-based DV when the depth map is available (Option2).

Table 1. Performance of the shift IVMV candidate

Option 1 is just to remove the shift DVs. So, only shift IVMV is used as the shift candidates.

Option 2 is to use the depth-based minimum DV (G0100), when the depth image is available. When the depth image is not available, the proposed method uses the shift DV without the searching process. The current depth-based DV (DoNBDV) is derived by choosing the maximum depth value among four corner pixels in the corresponding depth block. The proposed depth-based DV (DoNBDVmin) is derived by choosing the minimum depth value among four corner pixels.

Figure 1. Depth-based DV derivation

Since DoNBDV is derived in CU, sometimes DoNBDV can be different from the real disparity if CU has more than two PUs. For instance, when the prediction mode is PART_Nx2N as shown in Figure 2 and the first PU (PU 0) has a background and the second PU (PU 1) has an object, DoNBDV is most likely to become a disparity of the second PU and cannot reflect the disparity of the first PU. However, if the proposed disparity vector (DoNBDVmin) is used, DoNBDVmin might be able to reflect the disparity of the first PU.

Figure 2. Example of the coding unit having 2 prediction units

# Proposed Text

- Please refer the attached WD.

# Test Results

Test results of the proposed method are summarized in Tables 2-5. Tables 2 and 3 show the performance of the option1 in the CTC and VSP/DBBP disabled condition, respectively. Tables 4 and 5 illustrate the performance of the option2 in the CTC and VSP/DBBP disabled condition, respectively. As shown in the tables, the almost no coding loss is observed (less than 0.1%).

Table 6 shows the benefit of the proposed method in terms of the simplification.

Table 2. Performance of the option1 in CTC

Table 3. Performance of the option1 in VSP/DBBP disabled condition

Table 4. Performance of the option2 in CTC

Table 5. Performance of the option2 in VSP/DBBP disabled condition

Table 6. Analysis in terms of simplification

|  | Shift DV | Option1 | Option2 |
| --- | --- | --- | --- |
| The number of condition checks | 30 (I.8.5.3.2.15) | 0 | Minimum function (4) |

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can simplify the process of the 3D Merge mode list construction. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).