| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0099 |
| --- | --- |

| Title: | 3D-CE1 related: Default motion parameter for sub-PU MPI |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

In order to improve parallel processing capability, default motion parameters for the sub-PU IVMP were adopted at the last meeting (G0147). As the same concept, this contribution proposes the default motion parameters for sub-PU MPI. They are derived from corresponding texture block which is located at center of the current PU. The default motion parameters are utilized on motion hole filling as well as the pruning process of merge candidates. The result shows that the proposed method has no coding loss.

# Proposed Method

The default motion parameters for the sub-PU IVMV are derived from the corresponding reference block which is located at center of the current PU. If the motion information is not available at the center, the sub-PU IVMV is not performed. If available, it is defined as the default motion parameter and utilized on the motion hole filling as well as the pruning process of merge candidates for the sub-PU IVMV.

Most of the operations for the current sub-PU MPI are very similar to the sub-PU IVMV, except for the default motion information derivation. Therefore, this contribution unifies the default motion parameter derivation parts of sub-PU IVMV and sub-PU MPI.

The default motion parameters for the sub-PU MPI are derived from the collocated texture block which is located at center of the current PU. If the motion information is not available at the center, the sub-PU MPI is not performed. If available, it is defined as the default motion parameter and then used in the same manner as the sub-PU IVMV.

# Proposed Text

- H.8.5.3.2.16 Derivation process for inter layer predicted sub prediction block motion vector candidates

- …

Depending on DepthFlag, the following applies:

      - If DepthFlag is equal to 0, for X in the range of 0 to 1, inclusive, the derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location (xPb, yPb), the variables nPbW and nPbH, the prediction list indication X, the view order index refViewIdx, and the disparity vector mvDisp as inputs, and the outputs are the flag availableFlagLXN, the motion vector mvLXN and the reference index refIdxLXN.

      - If DepthFlag is equal to 1, For X in the range of 0 to 1, inclusive, the derivation process for a texture merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb,  yPb), the variables nPbW and nPbH, and the variable mvAccFlag being equal to 0 as inputs, and the outputs are the flag curAvailableFlag, the flag availableFlagLXN, the motion vector mvLXN and the reference index refIdxLXN.

- When availableFlagL0N or availableFlagL1N is equal to 1, the following applies:

- For yBlk in the range of 0 to ( nPbH / nSbH − 1 ), inclusive, the following applies:

  - For xBlk in the range of 0 to ( nPbW / nSbW − 1 ), inclusive, the following applies:

    - If DepthFlag is equal to 0, the following applies:

      - For X in the range of 0 to 1, inclusive, the derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb + xBlk * nSbW, yPb + yBlk * nSbH ), the variables nSbW and nSbH, the prediction list indication X, the view order index refViewIdx, and the disparity vector mvDisp as inputs, and the outputs are the flag spPredFlagLX[ xBlk ][ yBlk ], the motion vector spMvLX[ xBlk ][ yBlk ] and the reference index spRefIdxLX[ xBlk ][ yBlk ].

      - When spRefIdxL0[ xBlk ][ yBlk ] and spRefIdxL1[ xBlk ][ yBlk ] are both equal to −1, the following applies for X in the range of 0 to 1, inclusive:

          - spMvLX[ xBlk ][ yBlk ] = mvLXN		(Error: Reference source not found-1)

          - spRefIdxLX[ xBlk ][ yBlk ] = refIdxLXN	(Error: Reference source not found-2)

          - spPredFlagLX[ xBlk ][ yBlk ] = availableFlagLXN	(Error: Reference source not found-3)

    - Otherwise (DepthFlag is equal to 1), the following applies:

      - For X in the range of 0 to 1, inclusive, the derivation process for a texture merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb + xBlk * nSbW, yPb + yBlk * nSbH ), the variables nSbW and nSbH, and the variable mvAccFlag being equal to 0 as inputs, and the outputs are the flag curAvailableFlag, spPredFlagLX[ xBlk ][ yBlk ], the reference index spRefIdxLX[ xBlk ][ yBlk ] and the motion vector spMvLX[ xBlk ][ yBlk ].

      - When spRefIdxL0[ xBlk ][ yBlk ] and spRefIdxL1[ xBlk ][ yBlk ] are both equal to −1, the following applies for X in the range of 0 to 1, inclusive:

          - spMvLX[ xBlk ][ yBlk ] = mvLXN		(Error: Reference source not found-4)

          - spRefIdxLX[ xBlk ][ yBlk ] = refIdxLXN	(Error: Reference source not found-5)

          - spPredFlagLX[ xBlk ][ yBlk ] = availableFlagLXN	(Error: Reference source not found-6)

                - -

                - -

                - -

                - -

                - -

                - -

                - -

                - -

            - -

            - -

            - -

…

# Test Results

Test results of the proposed method are summarized in Table 1. The result demonstrates that the proposed method does not affect the coding performance.

Table 2. Performance when all the vertical DVs are restricted to 0

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can improve the parallel processing capability for the sub-PU MPI. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).