| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0100 |
| --- | --- |

| Title: | 3D-CE3 related: Simplification on CABAC contexts for a DMM flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

At the previous meeting, CABAC context reduction of ic_flag and iv_res_pred_weight_idx were adopted to avoid storing the coded information of upper CTU row (G0061). As the same concept, this contribution proposes to simplify the CABAC contexts of dim_not_present_flag. Experimental results show that the proposed simplifications do not affect the coding performance in CTC. In addition, based on WD, the SW bug-fix for depth_dc_flag is reported.

# Proposed Method

It is proposed that only one context is used for the CABAC of dim_not_present_flag without referring to any information from neighboring blocks, so the number of its contexts can be reduced from 3 to 1.

# Proposed Text

…

Table Error: Reference source not found-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
|  | sdc_flag |  | 0 | 1 | 2 |
|  | dim_not_present_flag |  | 0 | 1 | 2 |

- Table Error: Reference source not found-21 – Values of initValue for dim_not_present_flag ctxIdx

| Initialization variable | ctxIdx of dim_not_present_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 |  |  |  |  |  |  |

- Table Error: Reference source not found-24 –Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| sdc_flag | 0 | na | na | na | na | na |
| dim_not_present_flag | Table Error: Reference source not found-20 | na | na | na | na | na |

Table Error: Reference source not found-2 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| iv_res_pred_weight_idx | iv_res_pred_weight_idx [ xL ][ yL ] |  | ( condL && availableL ) |

# Test Results

Test results of the proposed method are summarized in Tables 1 (CTC) and 2 (All intra). The results show that the proposed method does not affect the coding performance. In addition, the result of SW bug-fix is displayed in Table 3.

Table 2. Performance of the proposed method in CTC

Table 3. Performance of the proposed method in All intra condition

Table 2. Performance of the SW bug-fix in CTC

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can reduce the number of contexts and fix the bug in SW. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).