| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0101 |
| --- | --- |

| Title: | Allowing zero DC candidate in inter SDC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

At the last meeting, multiple DC candidates were adopted to improve the performance of the inter SDC (G0101). However, the zero DC candidate was not adopted due to the additional branch in merge mode in WD (G0058). To figure out the problem and for the design generality (that covers the zero DC candidate as well), the proposed method allows the zero candidate. It can be represented by deleting one condition in WD. Results show that the proposed method does not affect the coding performance a lot.

# Proposed Method

In the current 3D-HTM, to find more the accurate DC for all the residual, inter SDC checks five different candidates, which consist of DC, DC-1, DC+1, DC-2, and DC+2, and decides the best one through RDO. However, when the DC candidate is equal to 0, it is not performed. For the design generality (that covers the zero DC candidate as well), the proposed method allows the zero DC candidate in AMVP only.

Fig. 1. Multiple DC candidates

# Proposed Text

…

if( cuDepthDcPresentFlag ) {

pbOffset = ( PartMode = = PART_NxN && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) ? ( nCbS / 2 ) : nCbS

for( j = 0; j < nCbS; j = j + pbOffset )

for( k = 0; k < nCbS; k = k + pbOffset )

if( DmmFlag[ x0 + k ][ y0 + j ] ¦ ¦ sdc_flag[ x0 ][ y0 ] ) {

depth_dc_flag[ x0 + k ][ y0 + j ]

dcNumSeg = DmmFlag[ x0 + k ][ y0 + j ] ? 2 : 1

if( depth_dc_flag[ x0 + k ][ y0 + j ] )

for( i = 0; i < dcNumSeg; i ++ ) {

depth_dc_abs[ x0 + k ][ y0 + j ][ i ]

if ( depth_dc_abs[ x0 + k ][ y0 + j ][ i ] > 0 )

depth_dc_sign_flag[ x0 + k ][ y0 + j ][ i ]

}

}

}

The variable cuDepthDcPresentFlag is derived as specified in the following:

  - cuDepthDcPresentFlag = 	( sdc_flag[ x0 ][ y0 ]  | |  ( CuPredMode[ x0 ][ y0 ]  = =  MODE_INTRA ) )	(Error: Reference source not found-1)

depth_dc_flag[ x0 ][ y0 ] equal to 1 specifies that depth_dc_abs[ x0 ][ y0 ][ i ] and depth_dc_sign_flag[ x0 ][ y0 ][ i ] are present. depth_dc_flag[ x0 ][ y0 ] equal to 0 specifies that depth_dc_abs[ x0 ][ y0 ][ i ] and  depth_dc_sign_flag[ x0 ][ y0 ][ i ] are not present.

# Test Results

A test result of the proposed method is summarized in Tables. The result demonstrates that the proposed method does not affect the coding performance a lot.

Table 1. Performance of the proposed method

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can cover the zero DC candidate in inter SDC. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).